Cam Machine Virtual Experiment

For the book Design of Machinery

Prof. R. L. Norton  -  WPI

Purpose

Experimentally measure the displacement, velocity, and acceleration of a cam machine and compare them with the theoretically predicted parameters for the theoretical cam-follower system.

Equipment

Cam Machine demonstrator, a Dytran piezoelectric 10g/v accelerometer,  LVT velocimeter (600 mv/in/s), LVDT displacement transducer (20v/in), torque (48.3 mv/N-m) and a Hewlett Packard 35670A dynamic signal analyzer.   A 1.4 MB floppy disk, formatted for DOS. Cam has a 20- deg dwell, followed by a mod trap rise over 50 deg,  40 deg dwell,  modsine fall over 50 deg,  40 deg dwell, 4567 poly rise over 50 deg, 40 deg dwell,  345 poly fall over 50 deg,  20 deg dwell.  Rise is 0.5 in at roller.

Software

Program DYNACAM Student Edition with the book Design of Machinery for theoretical analysis and a spreadsheet package such as Excel to compare the measured and theoretical data.

Procedure

1	View the video of the cam machine to understand how it works and what was measured

2	All the measured and theoretical data are provided in the Excel file Cam Machine Virtual Lab.xls.  One worksheet in this file has the measured data and one the theoretical data.

3	The file Cam Machine Virtual Lab.cam can be opened in Dynacam to see the plotted functions.

4	See the drawing provided which shows the locations of the transducers on the links of the follower linkage.  The accelerometer is on the top arm at 9 inches from the right pin.  The displacement transducer is attached at a 9-in radius and the velocity transducer is attached at a 10-in radius, both to the lower arm.  You must convert the measured data into equivalent motions at the roller follower where Dynacam calculates the motion.

5	After scaling the measured data appropriately, plot the theoretical and measured data of each function together using an XY scatter plot in Excel.  Note that the theoretical and measured data have a different number of data points, but span the same total time.  Only a scatter plot allows superposition of differently divided data.

6	Compare the theoretical and measured data and draw your best engineering conclusions as to the reasons for the differences seen.

7	Discuss your findings in a report as defined by your instructor.