Design of Machinery Virtual Laboratory 11-1

Prof. R. L. Norton, WPI

Dynamic Accelerations, Forces, and noise in a Fourbar Linkage

Purpose

Analyze measured dynamic acceleration at a coupler point and reaction forces at the fixed pivots of a running fourbar linkage and compare them with the theoretically predicted data for the same linkage.

Equipment

Fourbar linkage demonstrator machine, four piezoelectric force transducers (10 lb/v), two piezoelectric accelerometers (10g/v), current source, and an Agilent 35670A dynamic signal analyzer.

Software

Program Fourbar for theoretical linkage analysis and a spreadsheet package such as Excel.

Procedure

1	View the Fourbar Machine video to see how the experiment was run.  Three variables (or factors) were switched between each of 2 states sequentially to test all eight factor combinations:  motor mounts soft and hard, flywheel on and off, and balance weights on and off.  The coupling was not changed for these tests.  A zero-backlash bellows-type coupling was used for all tests.  This coupling is different than either of those shown in the video. It is closer to the rigid coupling in behavior but does have some lateral compliance.  The bellows coupling can be seen in the still photos with this lab.  For this laboratory, analyze only the unbalanced linkage data, i.e., four factor combinations (tests 3, 4, 7 and 8 in the files noted below in step 3).

2	Eight measurements were taken for each of the 8 machine setups: Timing mark, Accx, Accy, F12x, F12y, F14x, F14y, and a microphone signal of the machine noise.  Two complete experiments were recorded to allow statistical analysis of the data such as analysis of variance (ANOVA).

3	The Excel files “4bar Machine Virtual Laboratory Test Data Exp 1.xls” and “4bar Machine Virtual Laboratory Test Data Exp 2.xls” contain a matrix of the eight tests run and all the test data taken.    The Excel file also contains theoretical acceleration and force data exported from program Fourbar.  A “4br Machine.4br” file of the linkage for program Fourbar is also provided so you can view the linkage and its theoretical data within that program.

4	Study the photographs of the machine, particularly those showing the rulers, to obtain dimensions of the shaft supports.  Calculate the reaction forces at the supports and determine the factors needed to relate the theoretical linkage forces to those measured at the transducers.

5	Apply the reaction factors and the transducer calibration factors to the data in Excel and plot the measured curves on top of the theoretical ones at the same scale to see the differences.

6	Create error functions by subtracting the theoretical data, point by point, from the experimental data for each test.  Calculate the RMS averages of these error functions.

7	Plot and compare the RMS averages of the error functions for each of the eight tests and draw conclusions about the best mode to run the linkage in to minimize the error.  Optional: If your instructor so directs, do a statistical analysis of variance (ANOVA) of the data to determine which factors interact and which are most significant in reducing noise and vibration.

8	Discuss the engineering tradeoffs of the two modes of motor mounting, and the addition/deletion of a flywheel.  What mode do you recommend the linkage be operated in?

9	Write your results in a report in the format required by your instructor.