CASE STUDY 7A

Preliminary Design of a Compressor Drive Train

Problem	Determine the force-time function within the compressor's cylinder and the torque-time function acting on the input shaft of the compressor during any one cycle.

Given	Compressor speed is 1500 rpm.  The compressor has a bore of 3.125 in, a stroke of 3.26 in, and a connecting rod to crank ratio of 3.5.  Inlet pressure is atmospheric (14.7 psia), peak cylinder pressure is 132 psig and the mean effective pressure (mep) is 26 psig.  Flow is 8.9 cfm at mep giving 1.6 hp.

Assumptions	The piston weight is 1 lb, the connecting rod weighs 2 lb with its center of mass at the 1/3 point from the big end.  The crankshaft weighs 5.4 lb including a counterbalance that optimally overbalances it to minimize the shaking force.  The exponent for the gas law equation is k = 1.13.

Solution	See Figures 8-1 through 8-3, and M-Book CASE7A.doc. (Note: This Case Study is descriptive. No calculations are performed so there is no M-file associated with this Case Study.)

1	The force-time function in the cylinder depends on the compressed-gas pressure, which in turn depends on the slider-crank mechanism's geometry and the gas-law:

p1⋅v1k=p2⋅v2k	(a)

where p1 is atmospheric pressure (in psia), v1 is the expanded cylinder volume at bottom dead center (BDC), and p2, v2 are the pressure and volume of the compressed gas at top dead center (TDC) or at any other position.  The gas-law exponent k is assumed to be 1.13 since the process is neither isothermal (k=1) nor adiabatic (k=1.4).  A compression ratio of 10.9:1 is also assumed as typical.  An expression for piston displacement y referenced to BDC (assuming constant crankshaft ω) is

y=r⋅cos(θ)+l⋅1−rl⋅sin(θ)2−l+r	(b)

where r = crank radius, l = connecting rod length, and θ = crank angle.  See reference 1 for a derivation of the expression in parentheses.

2	Combining these functions with the assumed pressure ranges and gas law constant gives an approximate function for the cylinder pressure p as a function of crank angle for this particular problem's data:

If π≤θ≤2⋅π then  p=924⋅θπ6−792⋅θπ7 else  p=0	(c)

This function is shown in Figure 8-2.  The force Fg on the piston and cylinder head due to the gas pressure is then

Fg=p⋅Ap=π4⋅p⋅dp2	(d)

where Ap is piston area and dp is piston diameter.  This is the same function as shown in Figure 8-2 multiplied by a constant.  A second scale is shown on the ordinate giving gas force Fg in addition to the gas pressure p for this problem.

3	The torque required to drive the compressor crankshaft has two components, one from the gas force Fg and another from the inertial forces Fi due to the accelerations.[1]

T=Tg+Ti

where

Tg=Fg⋅r⋅sin(θ)⋅1+rl⋅cos(θ)	(e)

and

Ti=12⋅m⋅r2⋅ω2⋅r2⋅l⋅sin(θ)−sin(2⋅θ)−3⋅r2⋅l⋅sin(3⋅θ)

The mass m is taken as that of the piston and wrist pin plus the portion of the connecting rod (about 1/3) considered to be acting at the piston.[1]  When the data for this problem are substituted into (e), the torque time function is as shown in Figure 8-3.