CASE STUDY 9C

Design of a Return Spring for a Cam-Follower Arm

Problem	Design an extension spring for the cam-follower arm in Figure 8-8 based on the loadings defined in Case Study 9A in Chapter 8.

Given	The spring rate is 25 lb/in with a preload of 25 lb.  The spring's dynamic deflection is 1.5 in.

Spring rate

k = 25;         % lbf / in

Minimum force

Fmin = 25;      % lbf

Working deflection

DELy = 1.50;    % in

Forcing frequency

ff = 180;       % cpm

Assumptions	The spring operates in an oil bath whose temperature is below 250 deg F.  Infinite life is required.  Use ASTM A228 music wire and standard loops on each end.

Shear modulus

G = 11.5e6;     % psi

Solution	See Figure 13-36, M-Book CASE9C.doc and M-File Case9c.m.

1	Assume a trial wire diameter from the available sizes in Table 13-2.  Assume a spring index of 8 and calculate the mean coil diameter D from equation 13.5.

Wire diameter

d = 0.177;      % in

Spring index

C = 8;

Mean coil diameter

D = C * d       % in

D =

1.4160

2	Use the assumed value of C to find an appropriate value of initial coil stress τi from equations 13.21as the average value of the functions that bracket the acceptable range of spring preloads in Figure 13-22:

TAUi1 = -4.231*C^3 + 181.5*C^2 - 3387*C + 28640     % psi

TAUi2 = -2.987*C^3 + 139.7*C^2 - 3427*C + 38404     % psi

TAUi = (TAUi1 + TAUi2) / 2                     % psi (eqn d)

TAUi1 =

1.0994e+004

TAUi2 =

1.8399e+004

TAUi =

1.4697e+004

3	Find the direct shear factor Ks:

Direct shear factor

Ks = 1 + 0.5/C

Ks =

1.0625

4	Use the value of τi from step 2 in equation 13.8 to find the corresponding initial coil-tension force Fi:

Fi = (pi * d^3 * TAUi) / (8 * Ks * D)   % lbf

Fi =

21.2718

Check that this force is less than the required minimum applied force Fmin, which in this case, it is.  Any applied force smaller than Fi will not deflect the spring.

5	Find the maximum force from the given rate and deflection and use them to find the mean and alternating forces from equation 13.15a:

Maximum force

Fmax = Fmin + k*DELy    % lbf

Fmax =

62.5000

Alternating force

Fa = (Fmax - Fmin) / 2  % lbf

Fa =

18.7500

Mean force

Fm = (Fmax + Fmin) / 2  % lbf

Fm =

43.7500

6	Use the direct shear factor Ks and previously assumed values to find the minimum stress τmin and the mean stress τm:

Stress at Fmin

TAUmin = Ks * (8*Fmin*D) / (pi*d^3)     % psi

TAUmin =

1.7272e+004

Stress at Fm

TAUm = Ks * (8*Fm*D) / (pi*d^3)     % psi

TAUm =

3.0227e+004

7	Find the Wahl factor Kw and use it to calculate the alternating shear stress τa in the coil.

Wahl factor

Kw = (4*C-1) / (4*C-4) + 0.615 / C

Kw =

1.1840

Alternating stress

TAUa = Kw * (8*Fa*D) / (pi*d^3)     % psi

TAUa =

1.4436e+004

8	Find the ultimate tensile strength of this music-wire material from equation 13.3 and Table 13-4 and use it to find the ultimate shear strength from equation 13.4 and the torsional yield strength for the coil body from Table 13-10, assuming no set removal.

From Table 13-4, for A228 music wire

A = 184.649 * 1000      % psi

b = -0.1625

A =

184649

b =

-0.1625

Ultimate tensile strength

Sut = A * d^b           % psi

Sut =

2.4465e+005

Shear yield strength

Sys = 0.45 * Sut        % psi

Sys =

1.1009e+005

Ultimate shear strength

Sus = 0.667 * Sut       % psi

Sus =

1.6318e+005

9	Find the wire endurance limit for unpeened springs from equation 13.12 and convert it to fully reversed endurance strength with equation 13.16c.

Wire endurance limit

Sew = 45.0 * 1000;      %psi

Fully reversed endurance limit

Ses = 0.5 * (Sew*Sus) / (Sus - 0.5*Sew)     % psi

Ses =

2.6098e+004

10	The fatigue safety factor for the coils in torsion is calculated from equation 13.16b.

Fatigue factor of safety

Nfs = (Ses * (Sus - TAUmin)) / ...

(Ses * (TAUm - TAUmin) + (Sus * TAUa))

Nfs =

1.4136

Note that the minimum stress due to force Fmin is used in this calculation, not the coil-winding stress from (d).

11	The stresses in the end hooks also need to be determined.  The bending stresses in the hook are found from equation 13.23:

C1=2⋅R1d=2⋅D2⋅d=C

C1 = C

Kb = (4*C1^2 - C1 - 1) / (4*C1*(C1-1))

SIGa = Kb * ((16*D*Fa) / (pi*d^3)) + ((4*Fa) / (pi*d^2)) % psi

SIGm = Kb * ((16*D*Fm) / (pi*d^3)) + ((4*Fm) / (pi*d^2)) % psi

SIGmin = Kb * ((16*D*Fmin) / (pi*d^3)) + ...

((4*Fmin) / (pi*d^2))    % psi

C1 =

8

Kb =

1.1027

SIGa =

2.7650e+004

SIGm =

6.4517e+004

SIGmin =

3.6867e+004

12	Convert the torsional endurance strength to a tensile endurance strength with equation 13.4 and use it and the ultimate tensile strength in equation 13.16 to find a fatigue safety factor for the hook in bending:

Se = Ses / 0.67     % psi

Nfb = (Se * (Sut - SIGmin)) / ...

(Se * (SIGm - SIGmin) + (Sut*SIGa))

Se =

3.8953e+004

Nfb =

1.0321

13	Find the torsional stresses in the hook at point B in Figure 13-23 using equation 13.24 and an assumed value of

C2 = 5;

R2 = (C2 * d) / 2       % in

Kw2 = (4*C2 - 1) / (4*C2 - 4)

TAUBa = Kw2 * (8*Fa*D) / (pi*d^3)       % psi

TAUBm = Kw2 * (8*Fm*D) / (pi*d^3)       % psi

TAUBmin = Kw2 * (8*Fmin*D) / (pi*d^3)   % psi

R2 =

0.4425

Kw2 =

1.1875

TAUBa =

1.4478e+004

TAUBm =

3.3783e+004

TAUBmin =

1.9304e+004

14	The fatigue safety factor for the hook in torsion is calculated from equation 13.16b.

Nfs = (Ses * (Sus - TAUBmin)) / ...

(Ses * (TAUBm - TAUBmin) + (Sus*TAUBa))

Nfs =

1.3702

Two of these safety factors are acceptable.  The safety of the hook in bending is low.

15	To get the specified spring rate, the number of active coils must satisfy equation 13.7, solving for Na yields:

Number of active coils

Na = (d^4 * G) / (8 * D^3 * k)

Na =

19.8779

Na = 20;

Note that we round it to the nearest 1/4 coil as the manufacturing tolerance cannot achieve better than that accuracy.  Having rounded the number of active coils, we must now calculate the spring rate using equation 13.7:

Corrected spring rate

k = (d^4 * G) / (8 * D^3 * Na)      % lbf / in

k =

24.8474

16	The total number of coils in the body and the body length are

Total coils

Nt = Na + 1

Nt =

21

Body length

Lb = Nt * d     % in

Lb =

3.7170

17	The free length can now be determined.  The length of a standard hook is equal to the coil inside diameter:

Hook length

Lhook = D - d       % in

Lhook =

1.2390

Free length

Lf = Lb + 2*Lhook   % in

Lf =

6.1950

18	The inside and outside coil diameters are

Inside coil dia

Di = D - d      % in

Di =

1.2390

Outside coil dia

Do = D + d      % in

Do =

1.5930

19	The weight of the spring's active coils is found from equation 13.11b and is

Weight density

RHO = 0.28;     % lbf / in3

Weight

Wa = (pi^2 * d^2 * D * Na * RHO) / 4    % lbf

Wa =

0.6130

20	The natural frequency of this spring is found from equation 13.25a and is:

Natural frequency

g = 32.174 * 12     % in / sec2

fn = (1/2) * sqrt((k*g)/Wa)         % Hz

fn = (1/2) * sqrt((k*g)/Wa) * 60    % cpm

g =

386.0880

fn =

62.5511

fn =

3.7531e+003

The ratio between the natural frequency and the forcing frequency is

Frequency ratio

freqRatio = fn / ff

freqRatio =

20.8504

which is sufficiently high.

21	We now have a complete design specification for this A228-wire spring:

Wire diameter

d   % in

d =

0.1770

Outside diameter

Do  % in

Do =

1.5930

Total coils

Nt  % coils

Nt =

21

Free length

Lf  % in

Lf =

6.1950

This design is marginal as the hook is predicted to be at failure in bending fatigue after about a million cycles of operation.  If that is too short a life, then the design should be iterated again to improve it.  Increasing the wire size to 0.192 in and the spring index to 8.5 will raise all the safety factors, with the hook in bending the lowest at 1.2.