CASE STUDY 4A

Bicycle Brake Arm Loading Analysis

Problem	Determine the forces acting in three dimensions on the bicycle brake arm in its actuated position as shown in Figure 3-9.  This brake arm has been failing in service and may need to be redesigned.

Given	The existing brake arm geometry is known and the arm is acted on by a cable force of 1047 N in the position shown.  (See also Case Study 1A).

Applied cable force

Fcable = 1047;   % N

Positon vector components

R12x = 5.2;      % mm

R12y = -27.2;    % mm

R12z = 23.1;     % mm

R32x = -75.4;    % mm

R32y = 38.7;     % mm

R32z = 0;        % mm

R52x = -13.0;    % mm

R52y = -69.7;    % mm

R52z = 0;        % mm

Angle of normal force

THETAdeg = 172;             % deg

THETA = 172 * (2*pi/360)    % rad

THETA =

3.0020

Coefficient of friction

MU = 0.4;

Assumptions	The accelerations are negligible.  A class 1 load model is appropriate and a static analysis is acceptable.  The coefficient of friction between the brake pad and wheel rim has been measured and is 0.45 at room temperature and 0.4 at 150F.

Solution	See Figures 3-9 and 3-10, M-Book CASE4A.doc and M-File Case4a.m.

1	Figure 3-9 shows a center-pull brake arm assembly commonly used on bicycles.  It consists of six elements or subassemblies, the frame and its pivot pins (1), the two brake arms (2 and 4), the cable spreader assembly (3), the brake pads (5), and the wheel rim (6).  This is clearly a three-dimensional device, and must be analyzed as such.

2	The cable is the same one that is attached to the brake lever in Figure 3-1.  The 267-N (60-lb) hand force is multiplied by the mechanical advantage of the hand lever and transmitted via this cable to the pair of brake arms as was calculated in Case Study 1A.  We will assume no loss of force in the cable guides, thus the full 1047-N (235-lb) cable force is available at this end.

3	The direction of the normal force between the brake pad and the wheel rim is shown in Figure 3-9 to be at θ = 172° with respect to the positive x axis, and the friction force is directed along the z axis.  (See Figures 3-9 and 3-10 for xyz axis orientations.)

4	Figure 3-10 shows free-body diagrams of the arm, frame, and cable spreader assembly.  We are principally interested in the forces acting on the brake arm.  However, we first need to analyze the effect of the cable spreader geometry on the force applied to the arm at A.  This analysis can be two dimensional since the cables are in a common z-plane.  Note also that the cable subassembly (3) is a concurrent force system.  Writing equations 3.3b for this subassembly, and noting the symmetry about point A, we can write from inspection of the FBD:

Fx=F23x+F43x=0Fy=F23y+F43y+Fcable=0	(a)

F23x=−F43xF23y=F43y=−Fcable2	(b)

THETA3deg = 56;             % deg

THETA3 = 56 * (2*pi/360)    % rad

THETA3 =

0.9774

F23y = -Fcable / 2          % N

F23x = F23y / tan(THETA3)   % N

F43x = F23x     % N

F43y = F23y     % N

F23y =

-523.5000

F23x =

-353.1052

F43x =

-353.1052

F43y =

-523.5000

Newton's third law relates these forces to their reactions on the brake arm at point A:

F32x = -F23x    % N

F32y = -F23y    % N

F32z = 0        % N

F32x =

353.1052

F32y =

523.5000

F32z =

0

5	We can now write equations 3.3a for the arm (link 2).

Fx=F12x+F32x+F52x=0Fy=F12y+F32y+F52y=0Fz=F12z+F32z+F52z=0	(d)

Mx=M12x+R12y⋅F12z−R12z⋅F12y ...+R32y⋅F32z−R32z⋅F32y ...+R52y⋅F52z−R52z⋅F52y=0

My=M12y+R12z⋅F12x−R12x⋅F12z ...+R32z⋅F32x−R32x⋅F32z ...+R52z⋅F52x−R52x⋅F52z=0	(e)

Mz=R12x⋅F12y−R12y⋅F12x ...+R32x⋅F32y−R32y⋅F32x ...+R52x⋅F52y−R52y⋅F52x=0

Note that all unknown forces and moments are initially assumed positive in the equations, regardless of their apparent directions on the FBDs.  The moments M12x and M12y arise because there is a moment joint between arm (2) and the pivot pin (1) about the x and y axes.  We assume negligible friction about the z axis, thus allowing M12z to be zero.

6	The joint between the brake pad (5) and the wheel rim (6) transmits a force normal to the plane of contact.  The friction force magnitude, Ff, in the contact plane is related to the normal force by the Coulomb friction equation,

Ff=μ⋅Fnorm	(f)

where µ is the coefficient of friction and Fnorm is the normal force.  The velocity of the point on the rim below the center of the brake pad is in the z direction.  The force components F52x and F52y are due entirely to the normal force being transmited through the pad to the arm and are therefore related by Newton's third law.

F52x=−Fnormx=−Fnorm⋅cos(θ)F52y=−Fnormy=−Fnorm⋅sin(θ)	(g)

The direction of the friction force must always oppose motion and thus it acts in the negative z direction on the wheel rim.  Its reaction force on the arm has the opposite sense.

F52z=−Ff	(h)

7	We now have 10 equations (in sets d through h) containing 10 unknowns: F12x, F12y, F12z, F52x, F52y, F52z, M12x, M12y, Fnorm, and Ff.  Forces F32x, F32y, and F32z are known from equations c.  These 10 equations can be solved simultaneously either by matrix reduction or by iterative root finding methods. Using matrix methods, we first create the coefficent matrix and right-hand-side vector.

Coeff = [1 0 0 1 0 0 0 0 0 0;...

0 1 0 0 1 0 0 0 0 0;...

0 0 1 0 0 1 0 0 0 0;...

0 -R12z R12y 0 -R52z R52y 1 0 0 0;...

R12z 0 -R12x R52z 0 -R52x 0 1 0 0;...

-R12y R12x 0 -R52y R52x 0 0 0 0 0;...

0 0 0 0 0 0 0 0 -MU 1;...

0 0 0 1 0 0 0 0 cos(THETA) 0;...

0 0 0 0 1 0 0 0 sin(THETA) 0;...

0 0 0 0 0 1 0 0 0 1];

rhs = [-F32x; -F32y; -F32z;...

-(R32y*F32z - R32z*F32y);...

-(R32z*F32x - R32x*F32z);...

-(R32x*F32y - R32y*F32x);...

0; 0; 0; 0];

Results (Forces [=] N, Moments [=] N m):

Soln = Coeff \ rhs;

F12x = Soln(1)  % N

F12y = Soln(2)  % N

F12z = Soln(3)  % N

F52x = Soln(4)  % N

F52y = Soln(5)  % N

F52z = Soln(6)  % N

M12x = Soln(7) / 1000  % N m

M12y = Soln(8) / 1000  % N m

Fnorm = Soln(9) % N

Ff   = Soln(10) % N

F12x =

-1.8060e+003

F12y =

-319.3107

F12z =

586.8642

F52x =

1.4529e+003

F52y =

-204.1893

F52z =

-586.8642

M12x =

-32.3178

M12y =

52.3992

Fnorm =

1.4672e+003

Ff =

586.8642