CASE STUDY 9B

Design of Hydrodynamic Bearings for a Cam Test Fixture

Problem	Determine the hydrodynamic conditions in the proposed bearings for the camshaft in the CDTF.

Given	The cam generates a peak dynamic force of 110 lb at a maximum speed of 180 rpm (3 rps), as defined in Case Study 9a (Chapter 8).  The flywheel weighs 220 lb and is located midway between the two bearings.  Bulk oil temperature is controlled to 200F.  The cam-shaft is 2-in dia and the preliminary design of the bearings allows up to 2 in of length each.

Peak cam force

Fcam = 110;      % lbf

Shaft speed

nprm = 3;        % rev / sec

Flywheel weight

W = 220;         % lbf

Shaft diameter

d = 2.000;       % in

Assumptions	Plain bearings must be used throughout since rolling-element bearings introduce too much noise.  Porous bronze bushings are proposed.  Use a clearance ratio of 0.001. Try a mineral oil of SAE30W (ISO VG 100).  Gravity oilers will be provided at each bearing.

Clearance ratio

CR = 0.001;

Solution	See Figures 8-8 and 10-31, M-Book CASE9B.doc and M-File Case9b.m.

1	Find the reaction forces acting on each bearing from the applied forces and dimen- sions defined in Figure 10-31.  Sum moments about R1 and assume upward forces positive.

Distances from R1 to:

Cam load

a = 4.50;   % in

Bearing reaction R2

c = 6.25;   % in

Flywheel centerline

b = 0.5*c   % in

b =

3.1250

ΣM:     a⋅Fcam−b⋅W+c⋅R2=0

R2 = (b*W - a*Fcam) / c     % lbf

R2 =

30.8000

ΣF:     −Fcam+R1−W+R2=0

R1 = W + Fcam - R2          % lbf

R1 =

299.2000

The bearing at R1 takes most of the load so we will design for that force with

P = R1                      % lbf

P =

299.2000

2	An ISO VG100 oil was suggested.  Figure 10-1 gives a viscosity for this oil at 200 deg F of about

ETA = 1.5 / 1e6;  % reyn

3	Find the average pressure in the bearing for an assumed length of

l = 2;              % in

pavg = P / (l*d)    % psi

pavg =

74.8000

4	Find the diametral and radial clearances in the bearing for the assumed clearance ratio.

cd = CR * d         % in

cr = 0.5 * cd       % in

cd =

0.0020

cr =

1.0000e-003

5	Find the surface velocity of the shaft in the bearing.

U = (d/2) * nprm * (2*pi)   % in / sec

U =

18.8496

6	Since the load and speed are known and the bearing dimensions are assumed, as is the viscosity, equation 10.8b can be solved for the dimensionless parameter Kε.

Keps = (P*cr^2) / (ETA*U*l^3)

Keps =

1.3228

7	The Ocvirk number can now be found from equation 10.12c.

ON = 4*pi*Keps

ON =

16.6222

The bearing design is viable based on this acceptable Ocvirk number.

8	The eccentricity ratio can now be found from equation 10.13, which fits actual experimental data better than the theoretical equation.

EPS = 0.21394 + 0.38517*log10(ON) - 0.0008*(ON-60)

EPS =

0.7188

9	The minimum film thickness is found from equation 10.4b.

hmin = cr * (1-EPS)     % in

hmin =

2.8118e-004

This is an ample film to protect even a poorly finished bearing, which this is not.

10	We conclude that the design is acceptable as proposed.