CASE STUDY 8A

Preliminary Design of a Winch Lift

Problem	Determine the force-time function in the lifting cable, the necessary drum diameter, and the torque-time function acting on the shaft of the winch drum during any 1 cycle.  Define the gear ratio and the power and torque requirements for the motor.

Given	A hay bale's weight varies depending on its moisture content but can be assumed to average about 60 lb.  The hay truck holds 100 bales and the farmer would like to unload it in 30 min.  The lift height is 24 ft.

Assumptions	Nylon rope of 3/4-in-dia has a minimum break-strength of about 8000 lb and a spring constant of about 50000 lb/in per ft of length in axial tension.

Solution	See Figures 8-4 through 8-6, M-Book CASE8A.doc and M-File Case8a.m.

1	The nominal load depends on the number of bales to be lifted at one time and the weight of any structure used to support the bales.  To unload 100 bales from the truck, one at a time in 30 min, requires that the average bale rate be 100/30 = 3.3 bales/min, or an 18-sec average period per bale.  Since some of this time must be used to return the empty lift to the ground, we cannot use the entire 18 sec to lift the load.  We must also allow some time for manual loading and unloading of the bales at top and bottom.  The portion of the total period during which the mechanism is working is called its duty cycle.  Lets assume that 1/3 of the period is used to load/unload, 1/3 to lift, and 1/3 to lower.  This allows 6 sec per bale if we lift only one bale at a time.  The average velocity of the lift would then have to be 24 ft/6sec = 4 ft/sec.  A better arrangement would seem to be to load two bales at a time on the lift.  This doubles the period to 36 sec, doubles the time available for lifting to 12 sec and halves the average velocity to 2 ft/sec, still keeping the same duty cycle.

2	The payload with two bales on the lift is 120 lb.  The dead-load will be the weight of the rope, hook, and any platform or structure used to support the bales.  Since this structure has yet to be designed, its weight is unknown.  We will assume that we can keep this deadweight under 50 lb.  The total nominal load will then be 170 lb for the lift phase, and 50 lb for the lowering phase.

3	At steady state, the load on the rope should be the above number.  However, at start-up, the load can be significantly higher due to the need to accelerate the load to its steady-state velocity and also due to the fact that there are both spring and mass in the system.  A combination of spring and mass in a dynamic system allows oscillations to occur as the kinetic energy of the moving mass is transferred to potential energy in the elastic spring and vice versa.  The rope is a spring.  When the slack in the rope is suddenly taken up against the mass of the load, the rope will stretch, storing potential energy.  When the force in the stretched rope becomes sufficient to move the load, it will accelerate the mass upward, increasing its velocity and transferring the spring's potential energy to kinetic energy in the mass.  If the mass accelerates sufficiently it will take the rope slack again.  When the mass falls to take up the slack, the cycle repeats.  Thus, as it starts up, the force in the rope can oscillate from zero to some value significantly greater than the steady-state nominal load.  To calculate the dynamic loading requires writing and solving the differential equations of motion for the system.

4	Figure 8-5a shows a simplified schematic of the portion of the dynamic system containing the lift mass and the rope spring.  Figure 8-5b shows the system modeled as a lumped mass supported by a spring and a damper.  Figure 8-5c shows a free-body diagram (FBD) of the mass acted upon by its weight W, the spring force Fs, and the damper force Fd.  Writing Newton's 2nd law for this FBD gives

F=m a Fs+Fd−W=Wg⋅y2''	(a)

where

Fs=k⋅y1−y2 if y1>y2 , else Fs=0Fd=c⋅y1'−y2' if y1>y2 , else Fd=0	(b)

and

y'=ddty y''=d2dt2y

The initial conditions are:

when t = 0 :        y1(0) = 0 ,   y'1(0) = 0 ,   y2(0) = 0 ,   y'2(0) = 0	(c)

From which      Fs(0) = 0    and   Fd(0) = 0	(d)

Solving equation a for y''

y''=1m⋅Fs+Fd−W	(e)

5	Letting v=y1', v⋅t=y1, and y=y2; and assigning values for v, k, m and W

Average velocity

v = 24;         % in / sec

Payload weight

W = 170;        % lbf

m = W / 386.09  % lbf sec2 / in

m =

0.4403

Rope stiffness

k = 50000/24    % lbf ft / in ft

k =

2.0833e+003

The critical damping cc is easily calculated from the known mass and spring constant values.  This system is only lightly damped by the ropes internal friction.  We will assume that its ratio of actual damping to critical damping, z, is about 10% (ζ = 0.10) and use this to calculate a damping value for equation (e).

ZETA = 0.1;

cc = 2*m*sqrt(k/m)  % lbf sec / in

c = ZETA * cc       % lbf sec / in

c/m                 % sec^-1

cc =

60.5745

c =

6.0574

ans =

13.7572

6	Equation (e) is a nonlinear, second-order differential equation that can only be solved by numerical methods.  First, we rewrite the second-order equation as two, first-order differential equations.

d2dt2y=1m⋅Fs+Fd−Wddtddty=1m⋅Fs+Fd−W

Let

D1=ddtyD2=ddtD1=ddtddty=1m⋅Fs+Fd−W

Or, in matrix form,

D=ddty1m⋅Fs+Fd−W

We will use one of Matlab's differential equation solving functions (ode45) to solve this problem.  Next, we will create a function that evaluates the D vector. This function, called dYdt.m, will be used by the ode45 function during the integration.

function D = dYdt(t,Y)

k = 2083;   % lbf / in

v = 24;     % in / sec

m = 0.440;  % lbf sec2 / in

W = 170;    % lbf

c = 6.1;    % lbf sec / in

D = zeros(2,1);  % establishes a column vector for D

D(1) = Y(2);

if v*t > Y(1)

FS = k*(v*t-Y(1));

FD = c*(v-Y(2));

else

FS = 0;

FD = 0;

end

D(2) = (1/m)*(FS + FD - W);

Next, we define the initial conditions, where the first element is the initial value of y and the second is the initial value of y'.

Y = [0 0];

We can now use the Runge-Kutta solver, called ode45 in Matlab, to integrate. The result is assigned to the matrix [t  Y].  This matrix will have Ntimesteps+1 rows ( one row for each timestep in the integration, plus one for the initial condition) and three columns [t Y1 Y2].  The first column contains values of time, the second will have the position of the load, y2, and the third will have the velocity y'2.

Let

[t Y] = ode45('dYdt',[0 1],Y);   % (eqn j)

The first argument in equation j tells ode45 where to find the function that defines the derivatives to be integrated (here called function dYdt.m). The second argument [0 1] defines the integration range (t = 0 to t = 1 second), and the final argument is the vector that contains the initial conditions.  When using the ode45 function, you do not set the number of integration steps; the ode45 function is an adaptive integration function. This means that it will adjust the size of the timesteps throughout the integration to try to achieve an accurate solution.

Extracting the time, position, and velocity of the load from [t  Y],

y2 = Y(:,1);

yprm2 = Y(:,2);

where t, y2, and y'2 (called yprm2) are each vectors of length Ntimesteps + 1.  The acceleration can be calculated from these results. A function M-file, accFUNC.m, was written to accomplish this task. This function is listed at the end of this case study.

yprmprm = accFUNC(t,y2,yprm2);

The acceleration of the load over the first second of operation is shown in Figure 8-6a.  Downward acceleration (gravity) is taken as negative.  Note the periods of negative acceleration (at a limiting value of g ) during which the load is in free-fall and the rope is slack with no tension.

The force in the rope can also be calculated from the integration results. Another function M-file, forceFUNC.m, was written for this calculation. This function is also listed at the end of this case study.

force = forceFUNC(t,y2,yprm2);

The acceleration and force results have been plotted in Figure 8-6, which can be created using the following Matlab commands.

subplot(1,2,1)

plot(t,yprmprm)

title('(a) Acceleration of load')

xlabel('Time (sec)')

ylabel('Acceleration of load (in/sec^2)')

subplot(1,2,2)

plot(t,force)

ylim([0 800])

title('(b) Dynamic force in cable')

xlabel('Time (sec)')

ylabel('Force in cable (lb)')

FIGURE 8-6. Acceleration and Cable Force at Startup of Load-Lift

The force in the rope over the first second of operation is shown in Figure 8-6b.  Note that the tension force rises to over four times the nominal load on the first oscillation and then drops to zero as the rope goes slack since it cannot support a compressive force.  This pattern repeats for 3 cycles, at which point the damping has reduced the oscillations to the point that the rope is always in tension.  After about 10 cycles, it has settled down to the value of the nominal load.

7	The torque required to drive the drum shaft will depend on the dynamic loads just calculated and on the diameter of drum selected.  Too small a diameter will cause high stresses and wear on the rope.  A large drum diameter will increase the required torque and increase the package size.  A 3/4-in-dia rope can wrap around a 20-in-dia sheave.  Since T=Fr, the torque required on the shaft will then be 10 times the tension in the rope (using in-lb) and will have the same time variations as shown in Figure 8-6b.

8	The average power required can be easily found from the change in potential energy over the time desired.  To raise a 170 lb load 24 ft in 12 sec requires

W = 170;        % lbf

hlift = 24;     % ft

timelift = 12;  % sec

P = (W * hlift) / timelift          % ft lbf / sec

P = (W * hlift) / timelift / 550    % hp

P =

340

P =

0.6182

Since there will be losses in the gear train and winch, we will need an input power larger than this, say 1 hp for a first trial.  It is desirable to keep it at or below this level since larger horsepower motors will require higher voltage than 110V.

This average power is based on the nominal load.  The peak load at startup requires more power.  Rather than size the motor to accommodate the transient start-up load, another approach is to provide sufficient flywheel in the system to supply the transient pulse of energy to get it past the start-up phase.  It is possible that the rotational inertia of the drum and worm gear will supply enough flywheel effect assuming that the winch is up to speed before the slack is first taken out of the rope.

9	The average drum angular velocity is determined from the required average linear velocity of the rope which is 2 ft/sec.  At a 10-in drum radius this gives

v = 2;          % ft / sec

r = 10 / 12;    % ft

OMEGA = v / r   % rad / sec

OMEGA =

2.4000

10	Electric motors for 60-Hz AC operation are made in only a few standard rotational speeds, the most common of which are 1725 rpm and 3450 rpm.  These speeds come about from the line-frequency-synchronous speeds of 1800 rpm and 3600 rpm minus some slippage in these nonsynchronous motors.  To minimize the gear ratio in the wormset, we should choose the slower of the two standard speeds, or 1725 rpm.  This gives a desired gear ratio of 23:1725 or 1:75.  This ratio is obtainable in one stage of a worm-wheel combination, and so is feasible.

11	To summarize the parameters determined from this preliminary design study, we are looking to design a system that has a 1 hp, 1725 rpm, 110-V AC electric motor, driving a 1:75 reduction worm-wheel set, that, in turn, drives a 20-in-dia winch drum at 23 rpm.  A 3/4-in rope is capstan-wound around the drum and its forged hook attached to a platform that weighs no more than 50 lb and stably supports two hay bales of up to 60 lb each.  These constitute a set of task specifications for our design.

Required Function M-Files

Function Listing: dYdt.m

function D = dYdt(t,Y)

k = 2083;   % lbf / in

v = 24;     % in / sec

m = 0.440;  % lbf sec2 / in

W = 170;    % lbf

c = 6.1;    % lbf sec / in

D = zeros(2,1);

D(1) = Y(2);

if v*t > Y(1)

FS = k*(v*t-Y(1));

FD = c*(v-Y(2));

else

FS = 0;

FD = 0;

end

D(2) = (1/m)*(FS + FD - W);

Function Listing: accFUNC.m

function Ypp = accFUNC(t,y2,yprm2)

k = 2083;   % lbf / in

v = 24;     % in / sec

m = 0.440;  % lbf sec2 / in

W = 170;    % lbf

c = 6.1;    % lbf sec / in

for i = 1:length(t)

if v*t(i) > y2(i)

FS = k*(v*t(i)-y2(i));

FD = c*(v-yprm2(i));

else

FS = 0;

FD = 0;

end

Ypp(i,1) = (1/m)*(FS + FD - W);

end

Function Listing: forceFUNC.m

function F = forceFUNC(t,y2,yprm2)

k = 2083;   % lbf / in

v = 24;     % in / sec

m = 0.440;  % lbf sec2 / in

W = 170;    % lbf

c = 6.1;    % lbf sec / in

for i = 1:length(t)

if v*t(i) > y2(i)

FS = k*(v*t(i)-y2(i));

FD = c*(v-yprm2(i));

else

FS = 0;

FD = 0;

end

F(i,1) = FS + FD;

end