CASE STUDY 7D

Design of the Head Bolts for an Air Compressor

Problem	Design a set of fasteners to attach the compressor head to the cylinder in Figure 8-1 based on the loadings defined in Case Study 7A.

Given	The compressor bore is 3.125-in diameter.  The dynamic force acting on the head fluctuates from 0 to 1000 lb each cycle from the 130 psi cylinder pressure.  A 0.06-in-thick, unconfined copper-asbestos gasket covers the entire head- cylinder interface.  The head thickness at the attachment points (exclusive of cooling fins) is 0.4 in.

Bore diameter

Di = 3.125;      % in

Maximum force on head

Pmax = 1000;     % lbf

Gasket thickness

tg = 0.06;       % in

Head thickness

th = 0.40;       % in

Material properties:

Bolt

Es = 30e6;       % psi

Gasket

Eg = 13.5e6;     % psi

Members

Em = 10.4e6;     % psi

Assumptions	Infinite life.  Use standard hex-head cap screws without washers.  The operating temperature is less than 350 deg F.  Use 99.9% reliability.

Solution	See Figures 8-1, 8-2, M-Book CASE7D.doc and M-File Case7d.m.

1	Choose a trial diameter for the screws of d = 0.250 in.  Use UNC threads to avoid stripping problems in the cast-aluminum cylinder.  The trial fastener is then a 1/4-20 UNC-2A cap screw with rolled threads for fatigue resistance.

d = 0.250;   % in

2	Choose a bolt circle and outside diameter based on the cylinder bore and the rule of thumb of at least 1.5d to 2d of distance between any bolt and an edge.  We will use 2d because of the need for sealing area against the cylinder pressure.

Dbc = Di + 2*(2*d)  % in

Do = Dbc + 2*(2*d)  % in

Dbc =

4.1250

Do =

5.1250

3	To get the recommended 6-bolt-diameters spacing between bolts we will need about 8 screws equispaced around the bolt circle.  Calculate the spacing between screws in units of bolt diameters with

nb = 8;

DELb = (pi * Dbc) / (nb * d)    % bolt diameters

DELb =

6.4795

This is slightly larger than the recommended 6 bolt diameters, but we will never-the-less proceed with it on the basis that the head is quite stiff in bending due to its cooling fins and the cylinder pressure of 130 psi is relatively low.  We will later calculate the gasket pressure to check for possible leakage.

4	Assume a trial bolt length of 1 in.  The head thickness of 0.40 in at the screw holes plus the 0.06-in gasket will leave 0.54 in of thread penetration into the cylinder's tapped hole.  This is 2x the screw diameter (10 threads), which is the minimum recommended length for a steel screw in aluminum threads.  A 1-in-long 1/4-in screw has threads over 0.75 in of its length[2], which allows the desired penetration.  The trial clamp length for the stiffness calculations is then,

l = 1;   % in

since the entire cap screw is engaged.

5	Try an SAE grade 7 bolt preloaded to 70% of its proof strength.  Table 14-6 shows the proof strength of this bolt to be

Sp = 105*1000;       % psi

The tensile stress area from Table 14-1 is

At = 0.0318;         % in2

The required preload is then

Fi = 0.70*Sp*At      % lbf

Fi =

2.3373e+003

6	Find the lengths of thread and shank of the bolt:

lthd = 2*d + 0.25   % in

ls = l - lthd       % in

lthd =

0.7500

ls =

0.2500

Because it is a cap screw, the entire thread is in the clamp zone.

lt = l - ls         % in

lt =

0.7500

7	Find the stiffness of the bolt from equation 14.11a.

kb = 1 / ((lt/(At*Es)) + ((4*ls)/(pi*d^2*Es)))  % lbf / in

kb =

1.0461e+006

8	The material and gasket stiffness combine according to equation 13.2b.  The length of a "barrel" of material shortened by the gasket thickness t is:

lg = l - tg     % in

lg =

0.9400

And, from Table 14-9a, the factors for aluminum are

A = 0.79670;

b = 0.63816;

9	The stiffness of the aluminum members by themselves is

km1 = d*Em*A*exp(b*(d/lg))  % lbf / in

km1 =

2.4546e+006

10	The portion of the unconfined gasket subjected to the clamp force can be assumed to be from the outside diameter of the head to the inside diameter of the bore.  The bolt hole should be subtracted from the gasket area.  The area of the clamped gasket around one bolt is

Ag = (pi/4) * ((Do^2 - Di^2)/nb - d^2)  % in2

Ag =

1.5708

11	The stiffness of this piece of gasket is found from equation 14.11c.

km2 = (Ag * Eg) / tg    % in2

km2 =

3.5343e+008

The modulus of elasticity of the gasket material is found in Table 14-9b.

12	The combined stiffness of the gasketed joint (from equation 13.2b) is then

km = 1 / ((1/km1) +(1/km2))     % lbf / in

km =

2.4377e+006

Note that the combined stiffness in this case is dominated by the aluminum mem- bers because the copper-asbestos gasket is stiffer.

13	The joint constant is

C = kb / (km + kb)

C =

0.3003

and

1 - C

ans =

0.6997

14	The 1000-lb load is assumed to be divided equally among the 8 bolts at 125 lb each.  The portions of the applied load felt by each bolt and material (equations 14.13) are:

P = Pmax / nb   % lbf

Pb = C * P      % lbf

Pm = (1-C) * P  % lbf

P =

125

Pb =

37.5350

Pm =

87.4650

15	The resulting peak loads in bolt and material are

Fb = Fi + Pb    % lbf

Fm = Fi - Pm    % lbf

Fb =

2.3748e+003

Fm =

2.2498e+003

16	The alternating and mean components of force on the bolt are

Fa = (Fb - Fi) / 2      % lbf

Fm = (Fb + Fi) / 2      % lbf

Fa =

18.7675

Fm =

2.3561e+003

17	The nominal mean, alternating, maximum, minimum stresses in the bolt are:

SIGanom = Fa / At       % psi

SIGmnom = Fm / At       % psi

SIGmaxnom = SIGanom + SIGmnom   % psi

SIGminnom = SIGmnom - SIGanom   % psi

SIGanom =

590.1730

SIGmnom =

7.4090e+004

SIGmaxnom =

7.4680e+004

SIGminnom =

7.3500e+004

18	The fatigue stress-concentration factor

Kf = 3.0

Kf =

3

for rolled threads is taken from Table 14-8.

19	The mean stress-concentration factor Kfm is found from equation 6.17 for

Sy = 115*1000   % psi

Kfm = KfmFUNC(Kf,Sy,SIGmaxnom,SIGminnom,SIGanom,SIGmnom)

Sy =

115000

Kfm =

1.5283

A function M-file called KfmFUNC.m has been used to select the appropriate value. The M-file is listed at the end of this example.

20	The local mean and alternating stresses in the bolt are

SIGa = Kf * Fa / At     % psi

SIGm = Kfm * Fm / At    % psi

SIGa =

1.7705e+003

SIGm =

1.1323e+005

21	The stresses at the initial preload and at the maximum bolt force are

SIGi = Kfm * Fi / At    % psi

SIGb = Kfm * Fb / At    % psi

SIGi =

1.1233e+005

SIGb =

1.1413e+005

22	The endurance strength for this material is found using the methods of Section 6.6:

Sut = 133*1000      % psi

Sprme = 0.5 * Sut   % psi

Sut =

133000

Sprme =

66500

Endurance limit modification factors:

Cload = 0.70;

Csize = 1;

Csurf = 0.74;

Ctemp = 1;

Creliab = 0.75;     % (R = 0.99)

Modified endurance limit

Se = Cload*Csize*Csurf*Ctemp*Creliab*Sprme  % psi

Se =

2.5835e+004

23	The corrected endurance strength and the ultimate tensile strength are used in equation 14.16 to find the safety factor from the Goodman line.

Nf = (Se * (Sut - SIGi)) /...

((Se * (SIGm - SIGi)) + (Sut * SIGa))

Nf =

2.0638

24	The safety factor against yielding during tightening is

Ns = Sy / SIGb

Ns =

1.0076

25	The load required to separate the joint and the safety factor against joint separation are found from equations 14.14c and 14.14d.

Nsep = Fi / (P * (1-C))

Nsep =

26.7227

26	The joint will leak unless the clamping forces are sufficient to create more pressure at the gasket than exists in the cylinder.  The minimum clamping pressure can be found from the total area of the gasketed joint and the minimum clamping force Fm.

Total gasket area

Aj = (pi/4) * (Do^2 - Di^2 - nb*d^2)    % in2

Aj =

12.5664

Average clamping pressure

pavg = Fm / Aj      % psi

pavg =

187.4899

Ratio clamp to cylinder pressure

Nleak = pavg / 130

Nleak =

1.4422

This ratio of clamp pressure over cylinder pressure makes the spacing of the screws acceptable.

27	The preload force needed was found in step 5 and is

Fi      % lbf

Fi =

2.3373e+003

28	The torque required to obtain that preload is

Ti = 0.21 * Fi * d     % lbf in

Ti =

122.7083

29	These safety factors are acceptable.  This design uses eight 1/4-20 UNC-2A, grade 7, hex-head cap screws, 1.0 in long, preloaded to 70% of proof strength and equispaced on a 4.125-in-diameter bolt circle.  This design has a safety factor against leakage to 1.4, a fatigue safety factor of 2.1, and can stand an overpressure of 27 times the operating pressure before joint separation will occur.  The low value of Ny reflects only localized yielding in the thread's stress concentrations and so is acceptable in this case.

Function M-File Listing: KfmFUNC.m

function ret = KfmFUNC(K,S,SIGmax,SIGmin,SIGanom,SIGmnom)

if K * abs(SIGmax) < S

ret = K;

end

if K * abs(SIGmax) > S

ret = (S - K*SIGanom) / abs(SIGmnom);

end

if K * abs(SIGmax - SIGmin) > (2 * S);

ret = 0

end