CASE STUDY 7C

Design of Spur Gears for a Compressor Drive Train

Problem	Determine a spur gearset for the compressor gearbox in Figure 8-1 based on the loadings defined in Case Study 7A and specify suitable materials and heat treatment.

Given	The torque-time function on the output shaft is as shown in Figure 8-3.  The required gear ratio is a 2.5:1 reduction in velocity from the input to the output shaft.  Output shaft velocity is 1500 rpm.

Velocity ratio

mG = 2.5;

Output shaft velocity

nout = 1500;    % rpm

Assumptions	A 10-year life of 1-shift operation is desired.  AGMA standard full-depth teeth will be used.   Based on the data in Tables 11-6 and 11-7, set

Qv = 10;

Both pinion and gear will be through-hardened steel.  Choose a 20-deg pressure angle for the pinion and gear.

Pressure angle

PHIdeg = 20;            % deg

PHI = 20 * (2*pi/360)   % rad

PHI =

0.3491

Solution	See Figures 8-1, 8-3, M-Book CASE7C.doc and M-File Case7c.m.

1	To keep the weight and size of the gearbox low we want to minimize the number of teeth on the pinion.  There are two factors to be considered in this decision.  One is the velocity ratio and the other is the manufacturing process that will be used to make the pinion.  The smallest number of teeth that a 20-deg pinion can have and mesh without interference with a gear having 2.5 times as many teeth is 14.  On the other hand, the smallest number of teeth that can be generated by a 20-deg hob without undercutting is 21.  Assuming that our pinion will be cut with a hob, we will choose   teeth since, with a velocity ratio of 2.5, the pinion must have an even number of teeth so that the gear will have an integral number of teeth.  The gear then has

Np = 22;        % teeth

Ng = Np * mG    % teeth

Ng =

55

teeth.  The pitch diameter of the pinion is

dp = @(pd) Np ./ pd;     % in (if pd [=] in^-1)

2	The time-varying torque on the output shaft is defined in Figure 8-3 as varying between -175 and +585 lb-in.  We will take the positive peak value to calculate the transmitted load, found from the known output torque and, as yet to be deter- mined, gear pitch radius.  The -175 lb-in torque causes a force that acts on the opposite sides of the teeth, loading both pinion and gear similar to an idler gear.  We will take that aspect of the loading into account with the application factor Ka.  The peak positive transmitted load, as a function of the unknown pinion pitch diameter is

Peak torque

Tmax = 585;     % lbf in

Transmitted load

Wt = @(pd) Tmax ./ (0.5 * dp(pd) * mG);  % lbf

where dp is the pinion pitch diameter and dp . mG is the gear pitch diameter.

3	The pinion (input) shaft velocity is

nin = nout * mG     % rpm

nin =

3750

and the pitch-line velocity of the pinion and the gear is

Vt = @(pd) (dp(pd) / 12) / 2 * nin * (2*pi); ...

% (in) (ft / in) (rev / min) (rad / rev) >>> ft / min

4	The velocity factor Kv (Cv) is calculated from equations 11.16 and 11.17 based on the assumed gear-quality index Qv and the pitch-line velocity Vt.

B = 0.25 * (12 - Qv) ^ (2/3)

A = 50 + 56 * (1 - B)

Kv = @(pd) (A ./ (A + sqrt(Vt(pd)))).^B;

B =

0.3969

A =

83.7764

5	A value of Km must be chosen from Table 11-16.  However, at this point, the face width is unknown so we will assume that the face width will be less than 2 in.  With this assumption the load distribution factor is

Km = 1.6;

6	The application factor Ka is intended to account for shockiness in the driving and driven machinery.  This machine has both, since it is driven by a single-cylinder engine and drives a single-cylinder compressor.  In many such cases, only an average value of the transmitted torque is known, based on the average transmitted power.  In this case, we have computed (in Case Study 7A) a fairly accurate torque-time function for the compressor, which in effect defines the overloads in the driven part of the system.  We used the peak torque rather than the average torque to define the transmitted load.  So, the full value of the application factor recommended in Table 11-17 may not be needed here.  We will use it to account for the partially reversed loading on the gear teeth (Figure 8-3) as well as the shock loading associated with the driver (engine) and estimate it at  and  .

Ka = 2;

Ca = Ka

Ca =

2

7	The bending geometry factor J for this combination is found in Table 11-9 for loading at the highest point of single-tooth contact (HPSTC) and is approximately:

J = 0.34;

8	The size factor Ks (Cs), the idler factor KI , and the rim bending factor KB are all 1 for these small gears.

Ks = 1;

KB = 1;

KI = 1;

9	The bending stresses in the pinion can now be computed.  (When the pinion and gear are made from the same material, the bending stress in the pinion will always be greater than in the gear).

SIGb = @(pd,FF) (Wt(pd) .* pd) ./ (FF * J) * ...

(Ka * Km) / Kv(pd) * Ks * KB * KI;

10	An estimate of the uncorrected bending-fatigue strength can be made from the curves of Figure 11-25.  We will try an AGMA Grade 1 steel, through-hardened to

HB = 250;

The uncorrected bending-fatigue strength is found from the lower curve of the figure:

Sprmfb = -274 + 167*HB - 0.152*HB^2     % psi

Sprmfb =

31976

11	This value needs to be corrected for certain factors using equation 11.24.  The life factor KL is found from the appropriate equation in Figure 11-24 based on the required number of cycles in the life of the gears.  The pinion sees the largest number of repeated tooth-loadings, so we calculate the life based on it.  First, calculate the number of cycles N for the required life of 10 years, one shift.

N = nin * 60 * 2080 * 10     % (rev/ min) (min/hr) (hr/yr) (yr)

N =

4.6800e+009

The value of KL is found from

KL = 1.3558 * N^-0.0178

KL =

0.9121

12	At the specified operating temperature,

KT = 1;

13	The gear-material data are all taken at a reliability level of 99%.  This is satisfactory in this case, making

KR = 1;

14	The corrected bending-fatigue strength is then

Sfb = KL / (KT * KR) * Sprmfb   % psi

Sfb =

2.9167e+004

15	Substituting this value of the bending-fatigue strength for the bending stress in equation k and solving for F gives a design equation that relates the face width F and the diametral pitch pd.

F = @(pd) (Wt(pd) .* pd) ./ (Sfb .* J) .* ...

(Ka * Km) ./ Kv(pd) .* Ks .* KB .* KI;   % (eqn p)

This equation can be solved for F by substituting standard values of the diametral pitch into the right-hand side until one is found that yields a value of F that falls between 8/pd and 16/pd.  Another way to find an appropriate combination of F and pd is to plot equation p and the upper and lower limits 8/pd and 16/pd. (This has been done in Figure 11-28.)  Then the values are determined visually from the graph.

FL = @(pd) 8 ./pd;

FU = @(pd) 16 ./ pd;	% (eqns q)

Equations p and q, are plotted in Figure 11-28 which can be created using the Matlab commands that follow.

Range of pd over which the equations are to be plotted

pd = 6:16;

Fplot = F(pd);

FUplot = FU(pd);

FLplot = FL(pd);

plot(pd,Fplot,'-', pd,FUplot,'--', pd,FLplot,':')

xlabel('Diametral Pitch (in^-1)')

ylabel('Face Width (in)')

ylim([0 2])

legend('F','F-U','F-L')

FIGURE 11-28. Face Width vs. Diametral Pitch

The solid line is the design equation p and the dashed lines are the upper and lower limits on F.

From the graph we see that a diametral pitch of 12 yields a face width that falls between the upper and lower limits.  Choosing this value,

pd = 12;       % in^-1

and substituting it into equation p gives a face width of

Fval = F(pd)   % in (Fval used to not overwrite the F function)

Fval =

1.1630

Using a value that is the decimal equivalent of a common fractional value, let

Fval = 1.250;  % in

16	The parameters that depend upon pd and F can now be evaluated (The "val" variable names have been used to prevent overwriting a fucntion name with a variable):

Pinion pitch diameter

dpval = dp(pd)  % in

dpval =

1.8333

Velocity factor

Kvval = Kv(pd)

Kvval =

0.8499

Pitch-line velocity

Vtval = Vt(pd)  % ft / min

Vtval =

1.7999e+003

Transmitted load

Wtval = Wt(pd)  % lbf

Wtval =

255.2727

Bending stress

SIGbval = SIGb(pd,Fval)   % psi

SIGbval =

2.7137e+004

17	Additional factors are needed for the surface stress calculation.  Table 11-18 shows an approximate elastic coefficient of 2300 for steel on steel.  In Example 11-6, we computed a more accurate value of

Cp = 2276;  % psi^0.5

The surface finish factor is

Cf = 1;

Also,

Cv = Kvval

Ca = Ka

Cm = Km

Cs = 1;

Cv =

0.8499

Ca =

2

Cm =

1.6000

18	The surface geometry factor I is calculated from equations 11.22:

rp = 0.5 * dpval    % in

rg = mG * rp        % in

rp =

0.9167

rg =

2.2917

RHOp = sqrt((rp + 1./pd).^2 - (rp * cos(PHI)).^2) - ...

pi./pd .* cos(PHI)    % in

RHOg = (rp + rg) * sin(PHI) - RHOp  % in

RHOp =

0.2619

RHOg =

0.8354

I = cos(PHI) ./ (((1./RHOp) + (1./RHOg)) .* dpval)

I =

0.1022

19	The surface stress in the pinion can now be computed.

SIGc = Cp .* sqrt((Wtval./(Fval.*I.*dpval)) .*...

((Ca .* Cm)./Cv) .* Cs .* Cf)  % psi (eqn x)

SIGc =

1.4579e+005

20	An estimate of the uncorrected surface fatigue strength can be made from the curves of Figure 11-27.  For an AGMA Grade 1 steel, through-hardened to 250HB, the strength is found from the lower curve of the figure:

Sprmfc = 26000 + 327 * HB   % psi

Sprmfc =

107750

21	This value needs to be corrected for certain factors using equation 11.25.  The life factor CL is found from the appropriate equation in Figure 11-26 based on the required number of cycles N found above.

CL = 1.4488 * N^-0.023

CL =

0.8681

22

CT = KT

CT =

1

and

CR = KR

CR =

1

23	Since the gears and pinion are of the same hardness material in this case,

CH = 1;

24	The corrected surface-fatigue strength is then

Sfc = (CL * CH) / (CT * CR) * Sprmfc    % psi

Sfc =

9.3543e+004

25	This value of surface-fatigue strength is less than the value of surface stress calculated in equation x.  We can either reduce the stress by decreasing the diametral pitch (resulting in a larger tooth) and increasing the face width, or we can change the tooth surface strength by induction hardening, case hardening, or a similar process.  Since surface hardening will allow us to use smaller, lighter gears, we will take that approach.

26	Substitute the value of the surface stress from equation x for the corrected surface-fatigue strength in equation 11.25 and solve for the uncorrected surface-fatigue strength.

Sprmfc = (CT * CR) / (CL * CH) * SIGc   % psi

Sprmfc =

1.6793e+005

27	Using the equation for Grade 1 material in Figure 11-27, solve for the required surface hardness.

HB = (Sprmfc - 26000) / 327

HB =

434.0505

28	These gears are safe against tooth breakage and surface endurance.  Based on the assumptions and calculations and, if properly lubricated, should have a 99% probability of lasting the required 10 years before pitting of the pinion begins.

29	Note that the change of gear pitch diameters over those assumed in Case Study 7B for the shaft design also increases the transverse gear loads on the shaft.  This will require another iteration of that shaft design.