CASE STUDY 3B

Automobile Scissors-Jack Stress and Deflection Analysis

Problem	Determine the stress and deflections at critical points in the scissors-jack assembly shown in Figure 3-5 and 4-52.

Given	The geometry and loading are known from the previous Case Study 3A (p. 118).  The design load is 2000 lb total or 1000 lb per side.

Support force

P = 1000;    % lbf

Link dimensions

Length

l = 6.00;    % in

Width

w = 1.032;   % in

Thickness

t = 0.15;    % in

Properties

E = 30e6;        % psi

Sy = 60*1000;    % psi

Pin diameter

dpin = 0.437;    % in

Screw:  1/2 - 13 UNC

droot = 0.406;   % in

dAB = 12.55;     % in

Gear tooth root width

wgear = 0.44;    % in

Link 2 forces and angles

F12 = 1025.5;    % lbf

THETA12deg = 31.12;              % deg

THETA12 = 31.12 * (2*pi/360)     % rad

F32 = 1009.3;    % lbf

THETA32deg = 234.46;             % deg

THETA32 = 234.46 * (2*pi/360)    % rad

F42 = 412.0;     % lbf

THETA42deg = 135.0;              % deg

THETA42 = 135.0 * (2*pi/360)     % rad

THETA12 =

0.5431

THETA32 =

4.0921

THETA42 =

2.3562

Link 2 angle

THETAdeg = 30;              % deg

THETA = 30 * (2*pi/360)     % rad

dDE = 0.9   % in

THETA =

0.5236

dDE =

0.9000

Assumptions	The most likely failure points are the links as columns, the holes in bearing where the pins insert, the connecting pins in shear, the gear teeth in bending, and the screw in tension.  There are two sets of links, one set on each side.  Assume the two sides share the load equally.  The jack is typically used for very few cycles over its lifetime so a static analysis is appropriate.

Solution	See Figures 4-52 and 4-53, M-Book CASE3B.doc and M-File Case3b.m.

1	The forces on this jack assembly for the position shown were calculated in Case Study 3A (p. 118).  Please see that section and M-Book CASE3A.doc for additional force data.

2	The block-nut with pin A carries the compressive forces from links 2 and 7 (and the corresponding links on the opposite side of the screw) and a tensile force from the screw.  This is shown below in a FBD of the block-nut.  From the symmetry argument that was made in Case Study 3A, we see that the magnitudes of F21 and F71 are equal and that they make equal angles with the screw axis (x-axis).  Thus, since the magnitude of F12 = F21, the tensile force in the screw is

P = 4 * F12 * cos(THETA12)  % lbf

P =

3.5117e+003

Figure 4-52 (c). FBD of block-nut at A.

The tensile stress is found from equation 4.7 using the root diameter of the thread to calculate the cross-sectional area.  This is a conservative assumption as we shall see when we analyze threaded fasteners in Chapter 14.  The screw cross-sectional area is

Ascrew = (pi * droot^2) / 4     % in2

Ascrew =

0.1295

and the tensile stress in the jack screw is

SIGx = P / Ascrew   % psi

SIGx =

2.7125e+004

The axial deflection of the screw is found from equation 4.8

DEL = (P * dAB) / (Ascrew * E)  % in

DEL =

0.0113

3	Link 2 is the most heavily loaded of the links due to the applied load P being slightly to the left of center, so we will calculate its stresses and deflections.  This link is loaded as a beam-column with both an axial compressive force between points C and D and a bending couple applied between D and E.  Note that the force F12 is virtually colinear with the link axis.  The axial load is equal to

P = F12 * cos(THETA12 - THETA)  % lbf

P =

1.0253e+003

and the bending couple created by F42 acting about point D is

M = F42 * dDE   % in lbf

M =

370.8000

This couple is equivalent to the axial load being eccentric at point D by

Load eccentricity

ee = M / P      % in (reserve e for powers of ten)

ee =

0.3616

The secant-column formula (Eq. 4.46c) can be used with this effective eccentricity accounting for the applied couple in the plane of bending.  Since it is a pinned-pinned column,

leff = l        % in

leff =

6

from Table 4-7.  The radius of gyration k is taken in the xy plane of bending for this calculation (Eq. 4.34):

k = sqrt(w^2 / 12)  % in

k =

0.2979

The slenderness ratio is

Sr = leff / k

Sr =

20.1401

and

A = w * t       % in2

A =

0.1548

the distance to the outer fiber in the xy plane is

c = 0.5 * w     % in

c =

0.5160

The secant formula can now be applied and solved for the value of the critical load. (See Figure 4-53).  The equation can be solved iteratively using Matlab's fzero function,

Pfunc = @(P) P - ((Sy * A) / (1 + ((ee*c)/k^2) * ...

sec(Sr * sqrt(P/(4*E*A)))));   % lbf

Pcrit = fzero(Pfunc,1025)   % lbf

Pcrit =

2.9284e+003

The column also has to be checked for concentric column buckling in the weaker out-of-plane (z) direction, now with

c = 0.5 * t   % in

c =

0.0750

The radius of gyration in the z direction is found from

k = sqrt(t^2 / 12)  % in

k =

0.0433

The slenderness ratio in the z direction is

Sr = leff/ k

Sr =

138.5641

This needs to be compared to the slenderness ratio (Sr)D at the tangency between the Euler and Johnson lines to determine which buckling equation to use for this column:

SrD = pi * sqrt((2*E)/Sy)

SrD =

99.3459

The Sr for this column is greater than (Sr)D, making it an Euler column (See Figure 4-53).  The critical Euler load is then found from equation 4.38a.

Moment of inertia

I = (w*t^3)/12  % in4

Pcr = (pi^2 * E * I) / leff^2   % lbf

I =

2.9025e-004

Pcr =

2.3872e+003

Thus, it is more likely to buckle in the weaker z direction than in the plane of the applied moment.  Its safety factor against buckling is

N = Pcr / P

N =

2.3283

4	The pins are all dpin = 0.437 in dia.  The bearing stress in the most heavily loaded hole at C is

Bearing area

Abear = t * dpin        % in2

Abear =

0.0655

Bearing stress

SIGbear = P / Abear     % psi

SIGbear =

1.5642e+004

The pins are in single shear and their worst shear stress is

Shear area

Ashear = (pi * dpin^2) / 4  % in2

Ashear =

0.1500

Shear stress

TAU = P / Ashear            % psi

TAU =

6.8360e+003

5	The gear tooth on link 2 is subjected to a force of F42 = 412.0 lbf applied at a point

a = 0.22;       % in

from the root of the cantilevered tooth.  The tooth is wgear = 0.440 in deep at the root and t = 0.150 in thick.  The bending moment and stress are

Bending moment

wgear = 0.440;  % in

t = 0.150;      % in

M = F42 * a     % lbf in

M =

90.6400

Moment of inertia

I = (t * wgear^3) / 12  % in4

I =

0.0011

Outer fiber

c = 0.5 * wgear         % in

c =

0.2200

Bending stress

SIG = (M * c) / I       % psi

SIG =

1.8727e+004

6	This analysis could be continued, looking at other points in the assembly and, more importantly, at stresses when the jack is in different positions.  We have used an arbitrary position for this case study but, as the jack moves to the lowered position, the link and pin forces will increase due to poorer transmission angles.  A complete stress analysis should be done for multiple positions.

This case study will be revisited in the next chapter for the purpose of failure analysis.