CASE STUDY 4B

Bicycle Brake Arm Stress Analysis

Problem	Determine the stresses at critical points in the bicycle brake arm shown in Figures 3-9 and 4-54.

Given	The geometry and loading are known from the previous Case Study 4A (p. 127).  The cast-aluminum arm is a tee-section curved beam whose dimensions are shown in Figure 4-54 and given below.  The pivot pin is ductile steel.  The loading is three-dimensional.

Forces (magnitudes, see Figure 4-54 for directions)

F12x = 1806;    % N

F12y = 319;     % N

F12z = 587;     % N

F32x = 353;     % N

F32y = 524;     % N

F32z = 0;       % N

F52x = 1453;    % N

F52y = 204;     % N

F52z = 587;     % N

Moments (magnitudes, see Figure 4-54 for directions)

M21x = 32.3;    % N m

M21y = 52.4;    % N m

Distances

dABx = 80.6;    % mm

dABy = 66.0;    % mm

dBCx = 18.2;    % mm

dBCy = 42.5;    % mm

Tee section dimensions

h1 = 7.5;       % mm

h2 = 7.5;       % mm

w1 = 20;        % mm

w2 = 10;        % mm

ri = 58;        % mm

ro = 73;        % mm

Hub section dimensions

lhub = 28.5;    % mm

dout = 25;      % mm

din = 11;       % mm

Assumptions	The most likely failure points are the arm as a double cantilever beam (one end of which is curved), the hole in the bearing, and the connecting pin in bending as a cantilever beam.  Since this is a marginally ductile (5% elongation to frature) material, we can ignore the stress concentration on the basis that local yielding will relieve it.

Solution	See Figures 4-54 to 4-56, M-Book CASE4B.doc and M-File Case4b.m.

1	The brake arm is a double-cantilever beam.  Each end can be treated separatly.  The curved-beam portion has a tee-shaped cross-section as shown in Section X-X of Figure 4-54.  The neutral axis of a curved beam shifts from the centroidal axis toward the center of curvature a distance e as described in Section 4-9 and in equation 4.12a.  To find e requires an integration of the beam cross-section and knowledge of its centroidal radius.  Figure 4-55 shows the tee-section broken into two rectangular segments, flange and web.  The radius of the centroid of the tee is found by summing the moments of area for each segment about the center of curvature:

M=A1⋅rc1+A2⋅rc2=At⋅rc

where

A1 = w1*h1      % mm2

A2 = w2*h2      % mm2

At = A1 + A2    % mm2

A1 =

150

A2 =

75

At =

225

rc1 = ri + 0.5*h1       % mm

rc2 = ri + h1 + 0.5*h2  % mm

rc1 =

61.7500

rc2 =

69.2500

solving for rc

rc = (A1*rc1 + A2*rc2) / At     % mm

rc =

64.2500

See Figures 4-53 and 4-54 for dimensions and variable names.  The integral dA/r for equation 4.12a can be evaluated in this case by adding the integrals for web and flange.

1r dA=A1rc1+A2rc2	(b)

The radius of the neutral axis and the distance e are then

rn = At / ((A1/rc1) + (A2/rc2)) % mm

ee = rc - rn    % mm

rn =

64.0627

ee =

0.1873

The bending moment acting on the curved section of the beam is found by taking the cross-product of the force F32 and its position vector RAB referenced to the pivot at B in Figure 4-54.

MAB = dABx*F32y + dABy*F32x     % N mm (cw)

MAB =

6.5532e+004

The stresses at the inner and outer fibers can now be found using equations 4.12b and 4.12c (with lengths in mm and moments in N-mm for proper unit balance to give a result in MPa).

Inner fiber

ci = rn - ri    % mm

SIGi = (MAB/(ee*At))*(ci/ri)    % MPa

ci =

6.0627

SIGi =

162.5756

Outer fiber

co = ro - rn    % mm

SIGo = -(MAB/(ee*At))*(co/ro)    % MPa

co =

8.9373

SIGo =

-190.4130

2	The hub cross-section, shown as Section B-B in Figure 4-54 is a possible location of failure since there is a combination of bending and axial tension stresses here and the pin hole removes substantial material.  The bending stress is due to the moment acting on the curved beam and the tensile stress is due to the y-component of the force at A.  There is also a shear stress due to transverse loading but this will be zero at the outer fiber where the sum of the bending and axial stresses is maximum.  The area and area moment of inertia of the hub cross-section are needed:

Ahub = lhub * (dout - din)           % mm2

Ihub = (lhub*(dout^3 - din^3)) / 12  % mm4

Ahub =

399

Ihub =

3.3948e+004

The stress on the left half of Section B-B is the sum of the bending and axial stress:

SIGhub = (MAB*0.5*dout)/Ihub + F32y/Ahub    % MPa

SIGhub =

25.4428

The stress on the right half of Section B-B is lower because the compression due to bending is reduced by the axial tension.

3	The straight portion of the brake arm is a cantilever beam loaded in two directions, in the xy plane and in the yz plane.  The section moduli and moments are different in these bending directions.  The z-moment in the xy plane is equal and opposite the moment on the curved section.  The cross-section at the root of the cantilever is a rectangle of 23 by 12 mm as shown in Figure 4-54.  The bending stress at the outer fiber of the 23-mm side due to this moment is

Section width

b = 23;         % mm

Section height

h = 12;         % mm

Outer fiber

c = 0.5 * h     % mm

c =

6

Moment of inertia

I = (b * h^3) / 12    % mm4

I =

3312

Stress

SIGy1 = (MAB*c)/I     % MPa

SIGy1 =

118.7181

The x-moment is due to force F52z acting at the 42.5-mm distance from B to C, bending the link in the z-direction.  The bending stress at the surface of the 12-mm side is

Section width

b = 12;         % mm

Section height

h = 23;         % mm

Outer fiber

c = 0.5 * h     % mm

c =

11.5000

Moment of inertia

I = (b * h^3) / 12  % mm4

I =

12167

Bending moment

M = dBCy*F52z       % N mm

M =

2.4948e+004

Stress

SIGy2 = (M*c)/I     % MPa

SIGy2 =

23.5799

These two y-direction normal stresses add at the corners of the two faces to give

SIGy = SIGy1 + SIGy2    % MPa

SIGy =

142.2980

4	Another possible failure point is the slot in the cantilever arm.  Though the moment is zero there, the shear force is present and can cause tearout in the z-direction.  The tearout area is the shear area between the slot and the edge.

Arm thickness

t = 8;   % mm

Arm width

w = 4;   % mm

Tearout area

Atear = t*w         % mm2

Atear =

32

Shear stress

TAU = F52z / Atear  % MPa

TAU =

18.3438

5	The pivot pin is subjected to force F21, which has both x and y components, and to a couple M21 due to forces F12z and F52z.  The force F21 creates a bending moment having components F21xl and F21yl in the yz and xz planes, respectively, where l is the length of the pin.  Figure 4-56a shows the moment of the couple M21 and Figure 4-56b shows the moment of the the force F21.  Their combination is shown in Figure 4-56c.

Length of pin

l = 29;  % mm

Pin diameter

d = 11;  % mm

Outer fiber

c = 0.5*d   % mm

c =

5.5000

Moment of inertia

I = (pi * d^4) / 64   % mm4

I =

718.6884

From Figure 4-56, the total moment at the base of the pin is

Mpin = sqrt((M21x - F12y * (l/1000))^2 + ...

(-M21y + F12x * (l/1000))^2) % N m

Mpin =

23.0490

It is this combined bending moment that creates the largest bending stresses in the pin.  The maximum bending stress in the pin is

SIGpin = (Mpin*c)/I *1000   % MPa

SIGpin =

176.3902

6	A more complete analysis could be done using finite-element methods to determine the stresses and deformations at many other locations on the part.