CASE STUDY 2A

Hand-Operated Crimping-Tool Loading Analysis

Problem	Determine the forces on the elements of the crimping tool shown in Figure 3-3 during a crimp operation.

Given	The geometry is known and the tool develops a crimp force of 2000 lb (8896 N) at closure in the position shown.

Applied crimp force

Fc4x = -1956.30;    % lbf

Fc4y = 415.82;      % lbf

Positon vector components

Rc4x = 0.454;   % in

R12x = 1.399;   % in

R32x = 2.199;   % in

Rc4y = 0.337;   % in

R12y = 0.049;   % in

R32y = 0.077;   % in

R23x = -0.602;  % in

R43x = 0.602;   % in

R14x = -0.161;  % in

R23y = 0.127;   % in

R43y = -0.127;  % in

R14y = -0.758;  % in

R34x = 0.161;   % in

R34y = 0.758;   % in

Rhand = -4.40;  % in

Assumptions	The accelerations are negligible.  All forces are coplanar and two-dimensional.  A class 1 load model is appropriate and a static analysis is acceptable.

Solution	See Figures 3-3, 3-4, M-Book CASE2A.doc and M-File Case2a.m.

1	Figure 3-3 shows the tool in the closed position, in the process of crimping a metal connector onto a wire.  The user's hand provides the input forces between links 1 and 2, shown as the reaction pair Fhand.  The user can grip the handle anywhere along its length but we are assuming a nominal moment arm of Rhand for the application of the resultant of the user's grip force (see Figure 3-4).  The high mechanical advantage of the tool transforms the grip force to a large force at the crimp.

Figure 3-3 is a free-body diagram of the entire assembly,neglecting the weight of the tool, which is small compared to the crimp force.  There are four elements, or links, in the assembly, all pinned together.  Link 1 can be considered to be the "ground" link, with the other links moving with respect to it as the jaw is closed.  The desired magnitude of the crimp force Fc is defined and its direction will be normal to the surfaces at the crimp.

2	Figure 3-4 shows the elements of the crimping tool assembly separated and drawn as free-body diagrams with all forces applied to each element, again neglecting their weights as being insignificant compared to the applied forces.  The centers of gravity of the respective elements are used as the origins of the local, nonrotating coordinate systems in which the points of application of all forces on the element are located.  (See footnote on page 116 of the text).

3	We will consider link 1 to be the ground plane and analyze the remaining moving links.  Note that all unknown forces and moments are initially assumed positive.  Link 4 has three forces acting on it: Fc4 is the known (desired) force at the crimp, and F14 and F34 are the reaction forces from links 1 and 3, respectively.  The magnitudes of these two forces are unknown as is the direction of F14.  The direction of F34 will be the same as link 3, since it is a two-force member.  Writing equations 3.3b for this element:

Fx=F14x+F34x+Fc4x=0Fx=F14y+F34y+Fc4y=0Mz=R14x⋅F14y−R14y⋅F14x ...+R34x⋅F34y−R34y⋅F34x ...+Rc4x⋅Fc4y−Rc4y⋅Fc4x=0	(a)

We have three equations and four unknowns (F14x, F14y, F34x, F34y) at this point, so we need another equation.  It is available from the fact that the direction of F34 is known.  We can express one component of the force F34 in terms of its other component and the known angle θ3 of link 3.

F34y=F34x⋅tan(θ3)	(b)

where

THETA3deg = 168             % deg

THETA3 = 168 * (2*pi/360)   % rad

THETA3deg =

168

THETA3 =

2.9322

Coeff = [1 0 1 0;...

0 1 0 1;...

-R14y R14x -R34y R34x;...

0 0 -tan(THETA3) 1];

rhs = [-Fc4x; -Fc4y; -(Rc4x*Fc4y - Rc4y*Fc4x); 0];

Soln = Coeff \ rhs;   % lbf

Components of the unknown forces F14, and F34

F14x = Soln(1)   % lbf

F14y = Soln(2)

F34x = Soln(3)

F34y = Soln(4)

F14x =

442.9117

F14y =

-94.1394

F34x =

1.5134e+003

F34y =

-321.6806

4	Link 3 has two forces on it, F23 and F43.  Because this is a two-force link, these two forces are equal in magnitude and opposite in direction.  Also, from Newton's third law, F43 = - F34.  Thus,

F43x = -F34x   % lbf

F23x = -F43x

F43y = -F34y

F23y = -F43y

F43x =

-1.5134e+003

F23x =

1.5134e+003

F43y =

321.6806

F23y =

-321.6806

5	Link 2 has three forces acting on it: Fhand is the unknown force from the hand, and F12 and F32 are the reaction forces from links 1 and 3, respectively.  Force F12 is provided by part 1 on part 2 at the pivot pin and force F32 is provided by part 3 acting on part 2 at their pivot pin.  The magnitude and direction of F32 is known and the direction of Fhand is known.  Using equations 3.3b, we can solve for the magnitude of Fhand and the two components of F12.  From the third law,

F32x = -F23x   % lbf

F32y = -F23y

F32x =

-1.5134e+003

F32y =

321.6806

Coeff = [1 0 0; 0 1 1; -R12y R12x Rhand];

rhs = [-F32x; -F32y; -(R32x*F32y - R32y*F32x)];

Soln = Coeff \ rhs;   % lbf

F12x = Soln(1)

F12y = Soln(2)

Fhand = Soln(3)

F12x =

1.5134e+003

F12y =

-373.3653

Fhand =

51.6847

6	The four forces on link 1 can now be determined using the third law.

F21x = -F12x   % lbf

F21y = -F12y

F41x = -F14x

F41y = -F14y

Fc1x = -Fc4x

Fc1y = -Fc4y

F21x =

-1.5134e+003

F21y =

373.3653

F41x =

-442.9117

F41y =

94.1394

Fc1x =

1.9563e+003

Fc1y =

-415.8200

7	The solution to this problem for the scaled dimensions in Figure 3-3 assuming a 2000-lb (8896-N) force applied at the crimp, normal to the crimp surface, is given above.  The total forces at the pivot points are:

Pivot A

F12 = sqrt(F12x^2 + F12y^2)   % lbf

F12 =

1.5588e+003

Pivot B

F32 = sqrt(F32x^2 + F32y^2)   % lbf

F32 =

1.5472e+003

Pivot C

F43 = sqrt(F43x^2 + F43y^2)   % lbf

F43 =

1.5472e+003

Pivot D

F14 = sqrt(F14x^2 + F14y^2)   % lbf

F14 =

452.8057

The moment that must be applied to the handles to generate the crimp force of

Crimp force

Fc4 = sqrt(Fc4x^2 + Fc4y^2)   % lbf

Fc4 =

2.0000e+003

Moment

Mh = abs(Rhand*Fhand)   % in lbf

Mh =

227.4125

This moment can be obtained with a force of Fhand = 52 lbf applied at mid-handle.   This force is within the physiological grip-force capacity of the average human.

8	As an exercise, move the point of application of the crimp force along the jaw by changing the value assigned to Rhand and observe the changes to the forces and moments.