CASE STUDY 3A

Automobile Scissors-Jack Loading Analysis

Problem	Determine the forces on the elements of the scissors-jack in the position shown in Figure 3-5.

Given	The geometry is known and the jack supports a force of 1000 lb (4448 N) in the position shown.

Support force

Px = 0.0;    % lbf

Py =  -1000; % lbf

Positon vector components

Rpx = -0.50;     % in

R12x = -3.12;    % in

R32x = 2.08;     % in

Rpy = 0.87;      % in

R12y = -1.80;    % in

R32y = 1.20;     % in

R42x = 2.71;     % in

R23x = -0.78;    % in

R43x = 0.78;     % in

R42y = 1.00;     % in

R23y = -0.78;    % in

R43y = -0.78;    % in

R14x = 3.12;     % in

R24x = -2.58;    % in

R34x = -2.08;    % in

R14y = -1.80;    % in

R24y = 1.04;     % in

R34y = 1.20;     % in

Angle of gear teeth common normal

THETAdeg = -45.0;           % deg

THETA = -45.0 * (2*pi/360)  % rad

THETA =

-0.7854

Assumptions	The accelerations are negligible.  The jack is on level ground.  The angle of the elevated car chassis does not impart an overturning moment to the jack.  All forces are coplanar and two-dimensional.  A class 1 load model is appropriate and a static analysis is acceptable.

Solution	See Figures 3-5 through 3-8, M-Book CASE3A.doc and M-File Case3a.m.

1	Figure 3-5 shows a schematic of a simple scissors jack used to raise a car.  It consists of six links that are pivoted and/or geared together and a seventh link in the form of a lead screw that is turned to raise the jack.  While this is clearly a three-dimensional device, it can be analyzed as a two-dimensional one if we assume that the applied load (from the car) and the jack are exactly vertical (in the z direction).  If so, all forces will be in the xy plane.  This assumption is valid if the car is jacked from a level surface.  If not, then there will be some forces in the yz and xz planes as well.  The jack designer needs to consider the more general case, but for our simple example we will initially assume two-dimensional loading.  For the overall assembly as shown in Figure 3-5, we can solve for the reaction force Fg, given force P, by summing forces:  Fg = -P.

2	Figure 3-6 shows a set of free-body diagrams for the entire jack.  Each element or subassembly of interest has been separated from the others and the forces and moments shown acting on it (except for its weight, which is small compared to the applied forces and is thus neglected for this analysis).  The forces and moments can be either internal reactions at interconnections with other elements or external loads from the "outside world."  The centers of gravity of the respective elements are used as the origins of the local, nonrotating coordinate systems in which the points of application of all forces on the element are located.  In this design, stability is achieved by the mating of two pairs of crude (noninvolute) gear segments acting between links 2 and 4 and between links 5 and 7.  These interactions are modeled as forces acting along a common normal shared by the two teeth.  This common normal is perpendicular to the common tangent at the contact point.

There are 3 second-law equations available for each of the seven elements allowing 21 unknowns.  An additional 10 third-law equations will be needed for a total of 31.  This is a cumbersome system to solve for such a simple device, but we can use its symmetry to advantage in order to simplify the problem.

3	Figure 3-7 shows the upper half of the jack assembly.  Because of the mirror symmetry between the upper and lower portions, the lower half can be removed to simplify the analysis.  The forces calculated for this half will be duplicated in the other.  If we wished, we could solve for the reaction forces at A and B using equations 3.3b from this free-body diagram of the half-jack assembly.

4	Figure 3-8a shows the free-body diagrams for the upper half of the jack assembly, which are essentially the same as those of Figure 3-6.  We now have four elements but can consider the subassembly labeled 1 to be the "ground," leaving three elements on which to apply equations 3.3.  Note that all forces and moments are initially assumed positive in the equations.

5	Link 2 has three forces acting on it: F42 is the unknown force at the gear tooth contact with link 4; F12 and F32 are the unkown reaction forces from links 1 and 3, respectively.  Force F12 is provided by part 1 on part 2 at the pivot pin and force F32 is provided by part 3 acting on part 2 at their pivot pin.  The magnitudes and the directions of these pin forces and the magnitude of F42 are unknown.  The direction of F42 is along the common normal shown in Figure 3-8b.  Write equations 3.3b for this element to sum the forces in the x and y directions and sum moments about the CG (with the cross products expanded).

Fx=F12x+F32x+F42x=0Fy=F12y+F32y+F42y=0	(a)

Mz=R12x⋅F12y−R12y⋅F12x ...+R32x⋅F32y−R32y⋅F32x ...+R42x⋅F42y−R42y⋅F42x=0

6	Link 3 has three forces acting on it: P, F23 and F43.  Only P is known.  Writing equations 3.3b for this element gives

Fx=F23x+F43x+Px=0Fy=F23y+F43y+Py=0	(b)

Mz=R23x⋅F23y−R23y⋅F23x ...+R43x⋅F43y−R43y⋅F43x ...+Rpx⋅Py−Rpy⋅Px=0

7	Link 4 has three forces acting on it: F24 is the unknown force fromlink 2; F14 and F34 are the unkown reaction forces from links 1 and 3, respectively.

Fx=F14x+F24x+F34x=0Fy=F14y+F24y+F34y=0	(c)

Mz=R14x⋅F14y−R14y⋅F14x ...+R24x⋅F24y−R24y⋅F24x ...+R34x⋅F34y−R34y⋅F34x=0

8	The nine equations in sets a through c have 16 unknowns in them, F12x, F12y, F32x, F32y, F23x, F23y, F43x, F43y, F14x, F14y, F34x, F34y, F24x, F24y, F42x, F42y.  We can write the third-law relationships between action-reaction pairs at each of the joints to obtain six of the seven additional equations needed:

F32x=−F23x F32y=−F23yF34x=−F43x F34y=−F43yF42x=−F24x F42y=−F24y	(d)

9	The last equation needed comes from the relationship between the x and y components of the force F24 (or F42) at the tooth/tooth contact point.  Such a contact (or half) joint can transmit force (excepting friction force) only along the common normal[4] , which is perpendicular to the joint's common tangent as shown in Figure 3-8b.  The common normal is also called the axis of transmission.

The tangent of the angle of this common normal relates the two components of the force at the joint:

F24y=F24x⋅tan(θ)	(e)

10	Equations (d) and (e) will be substituted into equations (a) through (c) to create a set of nine simultaneous equations for solution.

Coeff = [1 0 0 0 -1 0 -1 0 0; ...

0 1 0 0 0 -1 -tan(THETA) 0 0; ...

-R12y R12x 0 0 R32y -R32x (-R42x*tan(THETA)+R42y) 0 0; ...

0 0 0 0 1 0 0 1 0; ...

0 0 0 0 0 1 0 0 1; ...

0 0 0 0 -R23y R23x 0 -R43y R43x;...

0 0 1 0 0 0 1 -1 0; ...

0 0 0 1 0 0 tan(THETA) 0 -1; ...

0 0 -R14y R14x 0 0 (R24x*tan(THETA)-R24y) R34y -R34x];

rhs = [0; 0; 0; -Px; -Py; (-(Rpx*Py - Rpy*Px)); 0; 0; 0];

Soln = Coeff \ rhs;

Results:

F12x = Soln(1)   % lbf

F12y = Soln(2)   % lbf

F14x = Soln(3)   % lbf

F14y = Soln(4)   % lbf

F23x = Soln(5)   % lbf

F23y = Soln(6)   % lbf

F24x = Soln(7)   % lbf

F43x = Soln(8)   % lbf

F43y = Soln(9)   % lbf

F12x =

877.7874

F12y =

530.4055

F14x =

-877.7874

F14y =

469.5945

F23x =

587.6801

F23y =

820.5128

F24x =

290.1073

F43x =

-587.6801

F43y =

179.4872

Additional Results:

F24y = F24x * tan(THETA)    % lbf

F34x = -F43x                % lbf

F34y = -F43y                % lbf

F32x = -F23x                % lbf

F32y = -F23y                % lbf

F42x = -F24x                % lbf

F42y = -F24y                % lbf

F24y =

-290.1073

F34x =

587.6801

F34y =

-179.4872

F32x =

-587.6801

F32y =

-820.5128

F42x =

-290.1073

F42y =

290.1073

11	The forces on link 1 can now be determined using the third law.

FAx = F12x  % lbf

FBx = F14x  % lbf

FAy = F12y  % lbf

FBy = F14y  % lbf

FAx =

877.7874

FBx =

-877.7874

FAy =

530.4055

FBy =

469.5945

12	As an exercise, move the point of application of P along the x direction by changing Rpx and Rpy and observe the changes to the forces on the links.  What happens when the vertical force P is centered on link 3?