CASE STUDY 2B

Crimping-Tool Stress and Deflection Analysis

Problem	Determine the stresses and deflections at critical points in the crimping tool shown in Figures 3-3 and 4-49.

Given	The geometry and loading are known from the previous Case Study 2A on p. 113.

Applied crimp force

Fcrimp = 2000;   % lbf

Inter-part forces

Part 1

F21 = 1560;  % lbf

F41 = 452;   % lbf

Part 2

F12 = 1560;  % lbf

F32 = 1548;  % lbf

Fhand = 52.2;    % lbf

Part 3

F23 = 1548;  % lbf

F43 = 1548;  % lbf

Part 4

F14 = 452;   % lbf

F34 = 1548;  % lbf

Distances

Part 1

Throat depth

h1 = 1.0;    % in

Inner radius

r1i = 0.6;   % in

Distance to load

a1 = 0.7;    % in

Part 2

Distance A-B

dAB = 0.80;  % in

Part 3

Distance B-C

dBC = 1.228; % in

Part 4

Distance C-D

dCD = 1.55;  % in

Distance to load

a4 = 0.35;   % in

Thickness and width

Part 1

t1 = 0.313;      % in

dpin = 0.250;    % in

Part 2

t2 = 0.187;      % in

Part 3

t3 = 0.125;      % in

w3 = 0.50;       % in

Part 4

t4 = 0.187;      % in

w4 = 0.75;       % in

Material properties (AISI 1095 steel, as-rolled)

Yield strength

Sy = 83*1000;    % psi

Young's modulus

E = 30e6;    % psi

Assumptions	The most likely failure points are link 3 as a column, the holes where the pins insert, the connecting pins in shear, and link 4 in bending.  The number of cycles expected over the life of the tool is low so a static analysis is acceptable.  Stress concentration can be ignored due to the material's ductility and the static loading assumption.

Solution	See Figures 3-3, 4-49 to 4-51, M-Book CASE2B.doc and M-file Case2b.m.

1	Link 3 is a pinned-pinned column loaded with F43 = 1548 lbf as calculated in Case Study 2A (p. 113) and shown in M-Book CASE2A.doc.  Note that, from Table 4-7, leff = dBC, the distance from B to C.  We need to first check the slenderness ratio of link 3 as a column.  This requires the radius of gyration (Eq. 4.34) for the weakest buckling direction, which is in the z-direction (out of the plane of the figure) in this case.  Even a small amount of clearance in the holes will prevent the pins from acting as a moment joint along their axes, thus creating an effective pinned-pinned joint both in-plane and out-of-plane.

leff = dBC  % in

leff =

1.2280

Depth of section

h = t3      % in

h =

0.1250

Radius of gyration	k=IA=b⋅h312⋅b⋅h=h212

k = sqrt(h^2 / 12)  % in

k =

0.0361

The slenderness ratio for the z-direction buckling is then

Sr = leff / k

Sr =

34.0313

which is greater than 10 making it other than a short column.  Calculate the slenderness ratio of the tangent point between the Johnson and Euler lines of Figure 4-42.

SrD = pi * sqrt((2*E)/Sy)

SrD =

84.4669

The slenderness ratio of this column is less than that of the tangent point between the Johnson and Euler lines shown in Figure 4-42.  It is thus an intermediate-column and the Johnson-column formula (Eq. 4.43) should be used to determine the critical load.

Cross-section area

A3 = t3 * w3    % in2

A3 =

0.0625

Critical load

Pcr = A3 * (Sy - (1/E)*((Sy*Sr)/(2*pi))^2)  % lbf

Pcr =

4.7665e+003

Ratio of critical load to applied load

N = Pcr / F43

N =

3.0791

The critical load is N = 3.08 times larger than the applied load.  It is safe against buckling.  Link 2 is a shorter, wider column than link 3 and has lower axial forces so can be assumed to be safe against buckling based on link 3 calculations.

2	Since it does not buckle, the deflection in link 3 in axial compression is (from Eq. 4.7):

DEL = (F43 * dBC) / (A3 * E)    % in

DEL =

0.0010

3	Any of the links could also fail in bearing in the 0.25-dia holes.  The largest force on any pin is F12 = 1560 lbf.  However, this force is applied at point A on links 1 and 2, which are thicker than link 3, where the applied force is only slightly less. This worst-case bearing stress (Eqs. 4.7 and 4.10) is on link 3 and is then

Bearing area

Abearing = t3 * dpin    % in2

Abearing =

0.0313

Bearing stress

SIGb = F43 / Abearing   % psi

SIGb =

49536

There is no danger of tearout failure in links 2 or 3 since the loading is toward the center of the part.  Link 1 has ample material around the holes to prevent tearout.

4	The 0.25-in diameter pins are in single shear.  The worst-case direct shear stress, from equation 4.9, is at A where F12 = 1560 lbf:

Shear area

Ashear = (pi/4) * dpin^2    % in2

Ashear =

0.0491

Pin shear stress

TAU = F12 / Ashear    % psi

TAU =

3.1780e+004

5	Link 4 is a 1.55-in long beam, simply supported at the pins and loaded with the 2000-lb crimp force at 0.35 in from C.  Write the equations for the load, shear, moment, slope, and deflection using singularity functions, noting that the integration constants C1 and C2 will be zero:

q(x) = R1<x - 0>-1 - F<x - a>-1 + R2<x - l>-1

V(x) = R1<x - 0>0 - F<x - a>0 + R2<x - l>0 	(h)

M(x) = R1<x - 0>1 - F<x - a>1 + R2<x - l>1

θ(x) = (R1<x - 0>2/2 - F<x - a>2/2 + R2<x - l>2/2  + C3) / EI	(i)

y(x) = (R1<x - 0>3/6 - F<x - a>3/6 + R2<x - l>3/6  + C3x + C4) / EI

6	The reaction forces R1 and R2 can be found by substitution of the boundary conditions x = l+, V = 0, M = 0 into the shear and moment equations.

F = 2000;    % lbf

M(l) = R1<l - 0>1 - F<l - a>1 + R2<l - l>1 = 0,  where

M = R1 dCD – F (dCD – a4) = 0

R1 = (F*(dCD - a4)) / dCD   % lbf

R1 =

1.5484e+003

V(l) = R1<l - 0>0 - F<l - a>0 + R2<l - l>0  = 0

V = R1 – F + R2 = 0

R2 = F - R1  % lbf

R2 =

451.6129

Note that these forces are consistent with those found in Case Study 2A.  The maximum moment is

Mmax = R1 * a4   % lbf in

Mmax =

541.9355

at the applied load.  The shear and moment diagrams are shown in Figure 4-50.

7	The bending stress at the point of maximum moment is then

Beam depth

h = w4      % in

h =

0.7500

Beam width

b = t4      % in

b =

0.1870

Moment of inertia

I = (b*h^3) / 12    % in4

I =

0.0066

Stress

SIG = (Mmax * 0.5 * h) / I   % psi

SIG =

3.0913e+004

8	The beam slope and deflection functions require calculation of the integration constants C3 and C4, which are found by substituting the boundary conditions x = 0, y = 0 and x = l, y = 0 in the deflection equation.

y(0) = 0 = R1<0 - 0>3/6 - F<0 - a>3/6 + R2<0 - l>3/6  + C3(0) + C4

C4 = 0;     % lbf in3

y(l) = 0 = R1<l - 0>3/6 - F<l - a>3/6 + R2<l - l>3/6  + C3 l

C3 = (1/dCD) * ((F/6)*(dCD-a4)^3 - (R1/6) * dCD^3)  % lbf in2

C3 =

-248.3871

9	From Figure 4-50 we see that the maximum deflection is close to the center of the span and occurs where the slope is zero.  Let the value of x where θ = 0 be c, then

θ(c)=1E⋅I⋅R12⋅c2−F2⋅c−a42+C3=0

Expanding and collecting terms in powers of c,

R1−F⋅c2+2⋅F⋅a4⋅c+2⋅C3−F⋅a42=0

Using the binomial equation to solve for c,

c = (1/(2 * (R1 - F))) * ...

((-2*F*a4) + (2*sqrt(-2*R1*C3 + R1*F*a4^2 + 2*F*C3))) % in

c =

0.6782

The maximum deflection, at x = c is

y = (1/(E*I)) * ((R1/6)*c^3 - (F/6)*(c-a4)^3 + C3*c)    % in

y =

-5.0571e-004

Only a very small deflection is allowed in link 4 to guarantee the proper crimp stroke, and this amount is acceptable.  The slope and deflection diagrams are shown in Figure 4-50.

10	Link 1 is relatively massive compared to the others and the only area of concern is the jaw, which is loaded by the 2000-lb crimp force and has a hole in the cross-section at its root.  While the shape of this element is not exactly that of a curved beam with concentric inside and outside radii, this assumption will be acceptably conservative if we use an outer radius equal to the smallest section dimension as shown in Figure 4-49.  This makes its inside radius 0.6 in and its approximate outside radius 1.6 in.  The eccentricity e of the curved beam's neutral axis versus the beam's centroidal axis rc is found from equation 4.12a.  Since this cross-section is rectangular, we can use the equation given in the footnote on p. 195.

Centroidal radius

rc = r1i + 0.5*h1   % in

rc =

1.1000

Outside radius

r1o = r1i + h1      % in

r1o =

1.6000

Radius to inner edge of hole

rhi = rc - 0.5*dpin % in

rhi =

0.9750

Radius to outer edge of hole

rho = rc + 0.5*dpin % in

rho =

1.2250

Section area

Anet = t1 * (h1 - dpin)   % in2

Anet =

0.2348

Eccentricity

e = rc - Anet / (t1 * ((log(rhi)-log(r1i)) + ...

(log(r1o)-log(rho))))  % in

e =

0.1034

The other dimensions necessary for a curved beam analysis are:

Neutral axis radius

rn = rc - e     % in

rn =

0.9966

Inner fiber radius

ci = rn - r1i   % in

ci =

0.3966

Outer fiber radius

co = r1o - rn   % in

co =

0.6034

11	The applied bending moment on the curved beam section is taken as the applied load times its distance to the beam's centroidal axis.

Bending moment

M = F * (a1 - r1i + rc) % lbf in

M =

2.4000e+003

12	Find stresses at the inner and outer fibers from equations 4.12b and 4.12c.  Reduce the beam cross sectional area by the hole area.

SIGi = (M / (e*Anet)) * (ci / r1i)  % psi

SIGo = -(M / (e*Anet)) * (co / r1o)  % psi

SIGi =

6.5344e+004

SIGo =

-3.7283e+004

13	There also is a direct axial tensile stress, which adds to the tensile bending stress in the inner fiber at point P:

Tensile stress

SIGa = F / Anet         % psi

SIGa =

8.5197e+003

Max stress

SIGmax = SIGi + SIGa    % psi

SIGmax =

7.3863e+004

This is the principal stress for point P since there is no applied shear or other normal stress at this edge point.  The maximum shear stress at point P is

TAUmax = 0.5 * SIGmax   % psi

TAUmax =

3.6932e+004

The bending stress at the outer fiber is compressive and thus subtracts from the axial tensile stress for a net of

Outer fiber net stress

SIGonet = SIGo + SIGa   % psi

SIGonet =

-2.8764e+004

14	The stress at the hole also needs to be checked as there is significant stress concentration with the axial stress of σa = 8.52 ksi.  The theoretical stress concentration factor for the case of a circular hole in an infinite plate is

kt = 3;

as defined in equation 4.32a and Figure 4-35.  For a circular hole in a finite plate, kt is a function of the ratio of the hole diameter to the plate width.  Peterson gives a chart of stress-concentration factors for a round hole in a flat plat under tension[5] from which we find that

kt = 2.42;

for a diameter / width ratio of 1/4.  The local principal stress at the hole is then

Local stress at hole

SIGhole = kt * SIGa     % psi

SIGhole =

2.0618e+004

which is still less than the stress at the inner fiber.

15	While this is far from a complete stress and deflection analysis of these parts, the calculations done do address the areas judged to be most likely to fail or to have problem deflections.  The stresses and deflections in link 1 were also computed using the ANSYS finite element analysis program, which gave an estimated maximum principal stress at point P of 66.2 ksi compared to our estimate of σmax = 73.9 ksi.  The FEA mesh and stress distribution calculated by the ANSYS model is shown in Figure 4-51.

Our analysis simplified the part geometry in order to allow the use of a known closed-form model (the curved beam) whereas the ANSYS FEA model included all the material in the actual part but discretized its geometry.  Both analyses should be recognized as only estimates of the stress states in the part, not exact solutions.

16	Redesign may be needed to reduce these stresses and deflections, based on a failure analysis.  This case study will be revisited in the next chapter after various failure theories are presented.