CASE STUDY 2C

Crimping-Tool Failure Analysis

Problem	Determine the factors of safety at critical points in the crimping tool shown in Figures 3-3 and 5-19.

Given	The stresses are known from the previous Case Study 2B on p. 255.  All material is AISI 1095 steel Q & T @ 800 F with Sy = 112 ksi.  It is an even material.

Yield strength

Sy = 112*1000;   % psi

Assumptions	The most likely failure points are link 3 as a column, the holes where the pins insert, the connecting pins in shear, and link 4 in bending.

Solution	See Figures 3-3 and 5-19, M-Book CASE2C.doc and M-File Case2c.m.

1	The previous case study found the critical column load in link 3 to be 3.1 times larger than the applied load.  This is the safety factor against buckling, which is expressed in terms of load rather than stress.

2	Any link can fail in bearing in the 0.25-in-dia holes.  The bearing stress (step 3 in Case Study 2B) is

SIGb = 49.5 * 1000;  % psi

3	As the only applied stress at this element, this is the principal stress and also the von Mises stress.  Thus,

SIGprm = SIGb       % psi

SIGprm =

49500

The safety factor for bearing stress on either hole or pin is then

N = Sy / SIGprm

N =

2.2626

4	The 0.25-in-dia pins are in single shear.  The worst-case direct-shear stress from Case Study 2B (step 4) is

TAU = 31.8 * 1000;   % psi

As the only stress on this section, this is also the maximum shear stress.  The safety factor for the pins in single shear from equation 5.9a is

N = (0.577*Sy) / TAU

N =

2.0322

5	Link 4 is a 1.55-in-long beam, simply supported at the pins with the 2 000-lb crimp force at 0.35 in from point C.  The bending stress (Case Study 2B, step 7) is

SIG = 30.9 * 1000;   % psi

As the only applied stress on this element at the outer fiber of the beam, this is the principal stress and also the von Mises stress.  Thus,

SIGprm = SIG        % psi

SIGprm =

30900

The safety factor for link 4 in bending is then

N = Sy / SIGprm

N =

3.6246

6	Link 1 has a tensile stress due to bending in the inner fiber at point P on the curved beam superposed on an axial tensile stress at the same point.  Their sum is the maximum principal stress (Case Study 2B, step 13) is

SIG1 = 73.9 * 1000;  % psi

There is no applied shear stress so this is the principal stress and also the von Mises stress.  The safety factor for bending at the inner fiber of the curved beam at point P from equation 5.8a is

N = Sy / SIG1

N =

1.5156

7	At the hole in link 1, there is only the axial stress

SIGa = 20.6 * 1000;  % psi

from step 13 of Case Study 2B where

SIGprm = SIGa

SIGprm =

20600

and the safety factor is found from equation 5.8a:

N = Sy / SIGa

N =

5.4369

Note that there is also a transverse shear stress at the hole which when combined with the axial tension reduces the safety factor at the hole to about 3.7.

8	Some of these safety factors, such as the N = 1.23 for bending in link 1 at point P, are low.  The safety factors for the pins in shear could also be increased to guard against user-induced overloads.  Either a stronger steel such as SAE 4140 could be selected or the section sizes of the parts could be increased slightly.  A small change in link thickness would achieve acceptable safety factors in the existing material.  Note that the geometry of this tool has been simplified for this example from that of the actual device.  The stresses and safety factors calculated here are not necessarily the same as those in the actual tool, which is a well tested and safe design.