CASE STUDY 1C

Bicycle Brake Lever Failure Analysis

Problem	Determine the factors of safety at critical points in the brake lever shown in Figures 3-1 and 5-18.

Given	The stresses are known from the previous Case Study 1B (p. 252).  The material of the brake lever is die-cast aluminum alloy ASTM G8A.  The elongation to fracture is 8% making it a marginally ductile material.

Yield strength

Sy = 186;    % MPa

Tensile strength

Sut = 310;   % MPa

Bending moment at P

MP = 20.3;   % N m

Moment of inertia at P

IP = 2053;   % mm4

Distance to outer fiber at P

cP = 7.150;  % mm

Shear load at P, Q

V = 267;     % N

Cross-section area at P, Q

A = 160.61;  % mm2

Assumptions	The most likely failure points are the two holes where the pins insert and at the root of the cantilever-beam lever handle.

Solution	See Figure 5-18, M-Book CASE1C.doc and M-File Case1c.m.

1	The bending stress at point P in Figure 5-18 at the root of the cantilever was found from equation 4.11b in Case Study 1B and is

SIGx = (MP * cP) / IP * 1000  % MPa

SIGx =

70.6990

2	This is the only applied stress at this point so it is also the principal stress.  The von Mises effective stress

SIGprm = SIGx   % MPa

SIGprm =

70.6990

and the principal stress is

SIG1 = SIGprm   % MPa

SIG1 =

70.6990

in this case (see equation 5.8c).  The safety factor against yielding at point P is then (from Eq. 5.8a),

Nyield = Sy/SIGprm   % Eqn b

Nyield =

2.6309

This design is safe at the average load but there is not a large margin to protect from overloads.  Note that in this simple stress situation, the distortion-energy theory gives identical results to the maximum shear theory because the failure boundaries (ellipse and hexagon) are coincident at the point x = σ1 , y = 0 in Figure 5-4.

3	Since this is a cast material with limited ductility, it would be interesting to also compute the modified-Mohr safety factor against brittle fracture from equation 5.12a.  It could be argued that the brake handle might still be usable despite slight yielding having occured:

Nfracture = Sut / SIG1

Nfracture =

4.3848

Note that we have not accounted for stress concentrations at the root of the cantilever that could reduce this fracture safety factor.

4	The transverse shear stress at point Q in Figure 5-18 was calculated from equation 4.15c as

TAUxy = (4*V)/(3*A)   % MPa

TAUxy =

2.2165

This shear stress is also the maximum since no other stresses act at that point.  The safety factor using the distortion-energy theory for pure shear at point Q is

Nshear = (0.577 * Sy) / TAUxy    % Eqn (e)

Nshear =

48.4185

Clearly, there is no danger of transverse-shear failure at point Q.

5	The compressive bearing stress in the hole at point A in Figure 5-18 is

Hole diameter

d = 8;       % mm

Single thickness at A

tA = 6.4;    % mm

Total bearing area

Abearing = d * (2*tA)           % mm2

Abearing =

102.4000

Load

F12 = 1100;                     % N

Bearing stress

SIGbearing = F12 / Abearing     % MPa

SIGbearing =

10.7422

and this stress, acting alone, is also the principal and the von Mises stress.  Assuming that the compressive strength of this material is equal to its tensile strength (an even material), the safety factor against bearing failure in the hole is

Nbearing = Sy / SIGbearing

Nbearing =

17.3149

6	Tearout in this case requires that four 6.4-mm-thick sections fail in shear through the material between hole A and the edge.  (See also Figure 4-13 on p. 192.)

Hole A edge distance

dedge = 7.1;                % mm

Tearout shear area

Atearout = dedge * (4*tA)   % mm2

Atearout =

181.7600

Shear stress

TAUtearout = F12 / Atearout % MPa

TAUtearout =

6.0519

This is a pure shear case and the safety factor is found from

Ntearout = (0.577*Sy) / TAUtearout

Ntearout =

17.7335

7	The cable-end inserts in a blind hole that is half-slotted to allow the cable to pass through at assembly as shown in Figure 5-18.  This slot weakens the part and makes the section at R the most likely failure location at this joint.  The bending stress at the outer fiber is

Cable force

Fc2 = 1047;     % N

Force at R

FR = 0.5 * Fc2  % N

FR =

523.5000

Distance from support to force at R

a = 0.5 * d     % mm

a =

4

Depth of section

h = 5;          % mm

Width of section

b = 10;         % mm

Outer fiber distance

c = 0.5*h       % mm

c =

2.5000

Bending moment

M = FR * a      % N mm

M =

2094

Moment of inertia

I = (b*h^3)/12  % mm4

I =

104.1667

Bending stress

SIGx = (M*c)/I  % MPa

SIGx =

50.2560

As the only applied stress at the outer fiber of this section, This is also the von Mises effective stress

SIGprm = SIGx   % MPa

SIGprm =

50.2560

and the principal stress is

SIG1 = SIGprm   % MPa

SIG1 =

50.2560

The bending safety factor at point R is

NQyield = Sy/SIGprm

NQyield =

3.7011

8	The shear stress due to transverse loading at the neutral axis in the section at R is (Eq. 4.14b)

Shear load

V = 0.5 * Fc2       % N

V =

523.5000

Cross-section area

A = b*h             % mm2

A =

50

Transverse shear stress (rectangular section)

TAUxy = (3*V)/(2*A) % MPa

TAUxy =

15.7050

This is the maximum shear stress at the neutral axis and the transverse-shear safety factor at point R is

TAUmax = TAUxy      % MPa

Sys = 0.577*Sy      % MPa

N = Sys / TAUmax

TAUmax =

15.7050

Sys =

107.3220

N =

6.8336

It is interesting to note that the transverse-shear safety factor is only about twice the bending safety factor at point R because the beam is so short.  Compare this result with that at point P in equations b and e where the bending and transverse shear safety factors differ by about a factor of 18 in the longer beam.  See Figure 5-18.

9	The die-cast aluminum alloy chosen is one of the strongest available.  If additional protection against overloads (as from the bicycle falling) is desired, either a change of geometry to increase the section size and/or reduce stress concentrations, or a change in material or manufacturing method could be made.  A forged-aluminum part would be stronger but would increase the cost.  Thicker sections would increase the weight slightly, but probably not prohibitively.  Increasing the diameter of the handle around point P by 26% to 18 mm (with perhaps a more generous transition radius also) would double the safety factor there since the section modulus is a function of d3.

Even though some of the other safety factors may seem excessive, it may be impratical to reduce those sections due to difficulties in casting thin sections.  Other considerations may take into account the appearance of a part intended for a consumer application such as a bicycle.  If the dimensions do not look "right" to the customer, it may give an undesired impression of cheapness.  Sometimes it is better economics to provide more thickness than is necessary for a suitable safety factor in order to provide a quality appearance.