CASE STUDY 9A

Preliminary Design of a Cam Dynamic Test Fixture (CDTF)

Problem	Define a preliminary design concept to satisfy the general constraints of the problem listed above.  Determine the force-time function acting on the follower and the torque-time function on the camshaft during any one cycle.  Define the drive ratio and the power and torque requirements for the motor.

Given	The four-dwell cams have a minimum diameter of 6 in and a maximum diameter of 8 in.  The rise is 1 in.  The roller follower is 2-in dia.  The cams are run at 180 rpm.  The cam shape is shown in Figure 8-7.

Assumptions	Plain bearings must be used throughout since rolling-element bear- ings introduce too much noise.  A speed-controlled DC motor will be used.

Solution	See Figures 8-7 through 8-13, M-Book CASE9A.doc and M-File Case9a.m.

1	A preliminary design is shown in Figure 8-8.  The camshaft is tapered to receive a matching taper in the cam.  This avoids the use of keyways, which can introduce vibration and noise on torque reversal.  The cam will be axially clamped to the shaft for concentric location.  A dowel pin at a large radius keys the cam to the hub to establish a zero position.  This arrangement allows quick removal and installation when changing cams.

2	A flywheel is attached to the camshaft to provide modulation of the speed oscillations during torque variations.  The flywheel also serves as a sheave for a flat belt from the motors smaller driving sheave in order to reduce the camshaft speed appropriately.

3	The follower arm is pivoted 12 in from the camshaft and carries a commercial roller follower running in a plain bearing.  A helical-coil tension spring loads the follow- er-arm roller against the cam.  This spring must be de-tensioned and removed to replace a cam and then re-tensioned for the new cam.  The cover, which is pivoted at the follower-arm pivot, applies tension to the spring when closed and releases it when opened.

4	Accelerometers and force transducers are fitted on and between the roller pivot and follower arm to measure the desired parameters.

5	The whole is mounted on a box-structure base that supports it, provides rigidity, and also contains an openable oil chamber around the cam.  The base can be supported either on casters for mobility, or on jacking legs for stability.  The motor is mounted to the base on rubber vibration isolators.  The electronics for motor control and instrumentation are contained within the base.

6	The design of the cam itself along with its rotational velocity determines the magnitudes and shapes of the acceleration of the follower arm.  This follower-acceleration function, generated from program Dynacam,[2] is shown in Figure 8-9.  The acceleration function multiplied by the effective mass of the follower is one component of the dynamic force needed for stress calculations.  The dynamic system of cam and follower can be modeled as a linearized, lumped parameter, single-degree-of-freedom system as shown in Figure 8-10.  The motion of the roller centerline on the rotating follower arm is actually along an arc, but that arc is quite flat in this design due to the length of the arm's radius.  The error in assuming the roller's motion to be linear over its short excursion is minimal.  A portion of the moving follower mass is then considered to be lumped at the roller so as to be dynamically equivalent.  From Newton's 2nd law, the differential equation for the lumped system in Figure 8-10 is [3]

F=m aF(t)−Fd−Fs=m⋅y''

where

Fd=c⋅y' Fs=k⋅y+Fpl

and

F(t)=m⋅y''+c⋅y'+k⋅y+Fpl	(a)

As in Case 8a, y'=ddty and y''=d2dt2y

The spring force Fs has two components.  The spring constant k times the deflection y is added to whatever initial force Fpl was created by preloading the spring at installation. The damping force Fd is proportional to velocity by a damping coefficient c.

7	Equation (a) can be solved kinetostatically in this case since we are maintaining a constant angular velocity and the displacement (y), velocity (y' ), and acceleration (y'' ) are defined functions of time.  The value of m will depend on our design of the follower arm and anything attached to it as moving mass such as the roller.  The value of the damping factor c is sometimes difficult to predict and the usual way to estimate its value is to define an expected damping ratio z for the type of system and calculate damping from

c=2⋅ζ⋅m⋅ωn	(b)

where  ωn=km

Koster[4] found that a typical value of z for cam follower systems is about 0.06.  The term wn is the undamped natural frequency of the system.

The value of the spring constant k is under the control of the designer as is the amount of spring preload Fpl.  We will later design a suitable spring for this system that will provide appropriate values for these variables.  Note that we cannot numerically calculate the dynamic load on the system until we have preliminary designs of the moving parts in order to define their masses.  At that point we can define a desired spring k and Fpl and then try to design a viable spring that delivers those values.

8	For a given mass, damping ratio, and acceleration function, the choice of k and Fpl determine whether the follower will jump off the cam during the fall.  Figure 8-11a shows the effect on the dynamic force F(t) of a too-small combination of k and Fpl. The shaded areas highlight portions where the dynamic force is negative.  A cam-follower joint cannot deliver a negative (tension) force any more than the rope of the previous case study could support a compressive force.  Thus the spring constant and preload must be increased in some combination until the dynamic-force function remains positive throughout the cycle, as shown in Figure 8-11b.

9	For this case study we will define the follower-arm geometry as shown in Figure 8-12.  It is a solid 2 in x 2.5 in rectangular cross section of aluminum, relieved internally around the follower for clearance.  The distance from pivot to roller follower is 12 in with a 10 in extension beyond the pivot for balance.  It extends 6 in beyond the roller follower to attach the spring.  The effective mass of the follower arm reflected to the roller centerline, plus the mass of roller and its pivot is 0.02 lb-sec2/in.  Using this value of effective mass and applying a spring constant at the end of the arm of k = 25 lb/in and a spring preload Fpl=25 lb, (which translates to effective values of k=56.25 and Fpl=37.5 lb at the follower) to achieve the dynamic force function shown in Figure 8-11b.  The peak dynamic force is 110 lb and the minimum force is 13 lb at the cam follower.  The deflection at the spring is 1.5 in.

10	The camshaft torque can be found from [5]

T(t)=F(t)⋅y'ω	(c)

This torque function for the values assumed above is shown in Figure 8-13.  The maximum torque is 176 in-lb and the minimum torque is 204 in-lb.  The average torque is 7 lb-in.

11	The flywheel is 24-in-dia and 1.88-in-thick solid steel. Its mass moment of inertia is

I = 44;     % lbf sec2 in

The coefficient of fluctuation Cf for this flywheel is found by integrating the torque-time function of Figure 8-13, pulse by pulse, to find the maximum energy oscillation E over one cycle.  That integration was done numerically with program Dynacam[2] and gives

E = 3980;   % in lbf

of energy over one cycle.  The coefficient of fluctuation is then[6]

OMEGA = 180;                % rpm

OMEGA = 180 * (2*pi) / 60   % rad / sec

Cf = E / (I * OMEGA^2)

OMEGA =

18.8496

Cf =

0.2546

Despite the relatively large size and weight (220 lb) of this flywheel, it gives only a 75% reduction in peak torque because its angular velocity is so low.  Flywheels need either high speed or very large mass to be effective.  The maximum torque with this flywheel is reduced to 47 in-lb and the minimum torque is now 48 in-lb.  The average torque is unchanged at 7 lb-in.  The shape of the torque function is the same as Figure 8-13 with these reduced peak values.

12	The average power required is very low (about 0.02 hp), but it is necessary to size the motor to handle the peak torque of

Tpeak = 47;    % in lbf

in order to maintain the constant speed required.  Using the peak torque value with the flywheel and the camshaft operating speed gives a minimum power level of

Ppeak = Tpeak * OMEGA               % in lbf / sec

Ppeak = Tpeak * OMEGA * 1.515e-4    % hp

Ppeak =

885.9291

Ppeak =

0.1342

Since frictional losses were only crudely estimated in this case and other cams will operate at higher speeds, a 1/2-hp speed-controlled DC motor was selected to drive the camshaft.  This uses 110-V AC power to the motor's rectifier/speed controller.

13	We need to select a speed ratio for the belt drive from motor to flywheel.  Since the motor is speed-controlled over an operating range of 0-1800 rpm, we can afford to allow a broader range of operating speeds than dictated by this particular cam, which needs 180 rpm.  A range of 0-400 rpm is reasonable since that will put the motor in the middle of its speed range for this cam and allow other cams to be run either faster or slower.  The drive pulley diameter is then

dout = 24;      % in

OMEGAout = 400; % rpm

OMEGAin = 1800; % rpm

din = dout * (OMEGAout/OMEGAin)   % in

din =

5.3333