CASE STUDY 3C

Automobile Scissors-Jack Failure Analysis

Problem	Determine the safety factors at critical points in the scissors-jack shown in Figures 3-5 and 5-20.

Given	The stresses are known from the previous Case Study 3B (p. 261).  The material of all parts is ductile steel.

Yield strength

Sy = 60*1000;    % psi

Assumptions	The most likely failure points are the links as columns, the holes in bearing where the pins insert, the connecting pins in shear, the gear teeth in bending, and the screw in tension.  There are two sets of links, one set on each side.  Assume the two sides share the load equally.  The jack is typically used for very few cycles over its lifetime so a static analysis is appropriate.

Solution	See Figures 3-5 and 5-20, M-Book CASE3C.doc and M-File Case3c.m.

1	The stresses on this jack assembly for the position shown were calculated  in Case Study 3B (p. 261).  Please see that section and Mathcad file CASE3B for additional stress data.

2	The jack screw is in axial tension.  The tensile stress was found in Case Study 3B (step 2) to be

SIGx = 27.2*1000;   % psi

This is a uniaxial tension stress, so is also the principal and the von Mises stress, thus

SIGprm = SIGx       % psi

SIGprm =

27200

The safety factor is

N = Sy / SIGprm

N =

2.2059

3	Link 2 is loaded as a beam-column.  Its safety factor against buckling was calculated in the last installment of this case study (step 3) and is N = 2.3.

4	The bearing stress in the most heavily loaded hole at C is (Case Study 3B, step 4)

SIGbear = 15.7*1000 % psi

SIGbear =

15700

This is a uniaxial compression stress, so is also the principal and the von Mises stress, thus

SIGprm = SIGbear    % psi

SIGprm =

15700

The safety factor is

N = Sy / SIGprm

N =

3.8217

The pins' shear stress is (Case Study 3B, step 4)

TAU = 6.85*1000;     % psi

This is a pure shear stress, so is also the maximum shear stress.  The safety factor is

N = (0.577 * Sy) / TAU

N =

5.0540

5	The bending stress at the root of the gear tooth on link 2 is (Case Study 3B, step 5)

SIG = 18.2*1000;     % psi

This is a uniaxial bending stress, so is also the principal and the von Mises stress, thus

SIGprm = SIG        % psi

SIGprm =

18200

The safety factor is

N = Sy/SIGprm

N =

3.2967

6	This analysis could be continued, looking at other points in the assembly and, more importantly, at stresses and safety factors when the jack is in different positions.  We have used an arbitrary position for this case study but, as the jack moves to the lowered position, the link and pin forces will increase due to poorer transmission angles.  A complete stress and safety factor analysis should be done for multiple positions.