CASE STUDY 8B

Design of a Wormset Speed Reducer for a Winch Lift

Problem	Size the worm and worm gear for the winch lift defined in Case Study 8A (p. 542) as shown in Figure 8-4 repeated here.

Given	The force-time function was estimated in the previous study to be as shown in Figure 8-6b (repeated here).  For an assumed winch-drum radius of 10 in, the peak torque will be about 7800 lb-in.  The average output power required was calculated to be about  0.6 hp.

Worm speed

nw = 1725;   % rpm

Gear ratio

mG = 75;

Assumptions	A single start worm with a 20 deg pressure angle will be tried.  The worm will be of steel case hardened to 58HRC and the worm gear of chill-cast phosphor bronze.  A self- locking wormset is needed.

Pressure angle

PHIdeg = 20;            % deg

PHI = 20 * (2*pi/360)   % rad

PHI =

0.3491

Threads on worm

Nw = 1;

Solution	See Figures 8-4, 8-6, M-Book CASE8B.doc and M-File Case8b.m.

1	A single-start worm will require a 75-tooth worm gear for the desired 75:1 ratio.  This number of worm-gear teeth is well above the minimum recommended in Table 12-7.

Ng = Nw * mG    % teeth

Ng =

75

2	Assume a center distance of 5.5 in for a trial calculation and find a suitable worm diameter based on that assumption from equation 12.16b.

C = 5.5;    %in

dw = (1/2.2)*C^0.875    % in (assumes C [=] in)

dw =

2.0202

3	Find a suitable worm gear diameter from equation 12.17.

dg = 2*C - dw   % in

dg =

8.9798

4	Find the lead from equation 12.13.

L = pi * dg * Nw / Ng   % in

L =

0.3761

5	Find the lead angle from equation 12.12.

LAMDA = atan(L / (pi * dw))                     % rad

LAMDAdeg = atan(L / (pi * dw)) * 360 / (2*pi)   % deg

LAMDA =

0.0592

LAMDAdeg =

3.3918

6	Find the maximum recommended face width from equation 12.19.

Fmax = 0.67 * dw    % in

Fmax =

1.3535

7	Find the materials factor Cs from equation 12.24.  Since C < 8 in,

Cs = 1000;  % since C < 8

8	Find the ratio correction factor Cm from equations 12.25.  Based on mG = 75, the second of the expressions in that equation set will be used.

Cm = 0.0107 * sqrt(-mG^2 + 56*mG + 5145)

Cm =

0.6526

9	Find the tangential velocity Vt from equation 12.27.

Vt = (nw*dw) / (2*cos(LAMDA)) /12 * (2*pi)   % fpm

Vt =

913.9318

10	Use this velocity to find the velocity factor Cv from equations 12.26.  For this value of Vt, the second of these equations is appropriate.

Cv = 13.31 * Vt^-0.571  % assumes Vt [=] fpm

Cv =

0.2713

11	Find the tangential load Wt from equation 12.23.

Wtg = Cs * Cm * Cv * dg^0.8 * Fmax  % lbf (assumes dg [=] in)

Wtg =

1.3875e+003

12	Find the coefficient of friction from the third expression in equation 12.29.

MU = 0.103 * exp(-0.110*Vt^0.450) + 0.012 % assumes Vt [=] fpm

MU =

0.0217

13	Find the friction force Wf from equation 12.28.

Wf = (MU * Wtg) / (cos(LAMDA)*cos(PHI))   % lbf

Wf =

32.0652

14	Find the rated output power from equation 12.21.

PHIo = (nw/mG)*(dg/2)*Wtg         % rev lbf in / min

PHIo = (nw/mG)*(dg/2)*Wtg * 2.525e-6 * (2*pi)   % hp

PHIo =

1.4328e+005

PHIo =

2.2732

15	Find the power lost in the mesh from equation 12.22.

PHIl = Vt * Wf              % ft lbf / min

PHIl = Vt * Wf * 3.03e-5    % hp

PHIl =

2.9305e+004

PHIl =

0.8880

16	Find the rated input power from equation 12.20.

phi = PHIo + PHIl   % hp (phi saves PHI for pressure angle)

phi =

3.1612

17	The efficiency of the gearset is

eff = PHIo/phi  % fraction (e saved for powers of ten)

eff =

0.7191

18	Find the rated output torque from equation 12.31.

Tg = Wtg * dg / 2   % lbf in

Tg =

6.2298e+003

19	While the power rating appears to be adequate for this application, the output torque rating falls short of the projected peak torque of 7800 lb-in modeled in Case Study 8A; thus some redesign is in order.

20	The original assumption for center distance was increased to 6.531 in and the model recalculated.  The center distance was also adjusted slightly to give an integer diametral pitch of 7 in-1. This increased the worm gear diameter to 10.714 in and the output torque rating to 9131 lb-in.  The new input power rating is 4.52 hp and the power loss is 1.18 hp for an efficiency of 73.8 %.  The output power rating is 3.33 hp.  The new lead angle is 3.48 deg, so is still self-locking.

21	While this new design appears feasible based on the loading calculations done in the previous case study, one of the original assumptions regarding the electric motor size will need to be revised.  The average net power required was estimated to be 0.62 hp.  It was hoped that a 1- to 1.25-hp motor would be adequate which would allow 110-V operation.  This now appears impossible due to the 1.18-hp loss in the wormset which would leave too little power available to lift the load even if a 1.25-hp motor were used.  The flywheel effect of the rotating winch drum can supply bursts of energy to get past the peaks of load oscillation shown in Figure 8-6, but cannot provide a sustained increase in power above the average available.  So a 220-V motor of about 2 or 2.25 hp appears to be necessary for this design.  The input power rating of the gearset should easily accommodate that level of power with no overheating problems.

22	The reader is encouraged to open the Matlab M-Book CASE8B.doc and make the changes suggested in step 20 to see how that alters the results.