CASE STUDY 5A

Fourbar Linkage Loading Analysis

Problem	Determine the theoretical rigid-body forces acting in two dimensions on the fourbar linkage shown in Figure 3-11.

Given	Link dimensions:

Link 2

a = 6;                      % in

a = 6 * 25.4 / 1000;        % m

r2CG = 3.36;                % in

r2CG = 3.36 * 25.4 /1000;   % m

DEL2deg = 26.7;             % deg

DEL2 = 26.7 * (2*pi/360)    % rad

DEL2 =

0.4660

Link 3

b = 16;                     % in

b = 16 * 25.4 / 1000;       % m

r3CG = 9.17;                % in

r3CG = 9.17 * 25.4 /1000;   % m

DEL3deg = 15.5;             % deg

DEL3 = 15.5 * (2*pi/360)    % rad

DEL3 =

0.2705

Link 4

c = 12;                     % in

c = 12 * 25.4 / 1000;       % m

r4CG = 4.05;                % in

r4CG = 4.05 * 25.4 /1000;   % m

DEL4deg = -10.0;            % deg

DEL4 = -10.0 * (2*pi/360)   % rad

DEL4 =

-0.1745

Link 1

d = 18;                     % in

d = 18 * 25.4 / 1000;       % m

Angle, link 2

THETA2deg = 30;             % deg

THETA2 = 30 * (2*pi/360)    % rad

THETA2 =

0.5236

Input velocity

OMEGA2 = 120;               % rpm

OMEGA2 = 120 * 2*pi / 60    % rad/sec

OMEGA2 =

12.5664

Input acceleration

ALPHA2 = 0;  % rad/sec2

Mass, link 2

m2 = 0.003;             % blob

m2 = 0.003 * 175.12     % kg

m2 =

0.5254

Mass, link 3

m3 = 0.006;             % blob

m3 = 0.006 * 175.12     % kg

m3 =

1.0507

Mass, link 4

m4 = 0.006;             % blob

m4 = 0.006 * 175.12     % kg

m4 =

1.0507

Mass mom, link 2

IG2 = 0.5;          % blob in2

IG2 = 0.5 * 0.113   % kg m2

IG2 =

0.0565

Mass mom, link 3

IG3 = 0.1;          % blob in2

IG3 = 0.1 * 0.113   % kg m2

IG3 =

0.0113

Mass mom, link 4

IG4 = 4.0;          % blob in2

IG4 = 4.0 * 0.113   % kg m2

IG4 =

0.4520

Assumptions	The accelerations are significant.  A class 4 load model is appropriate and a dynamic analysis is required.  There are no external loads on the system, all loads are due to the accelerations of the links.  The weight forces are insignificant compared to the inertial forces and will be neglected.  The links are assumed to be ideal rigid bodies. Friction and the effects of clearances in the pin joints also will be ignored.

Solution	See Figures 3-11 through 3-13, M-Book CASE5A.doc and M-File Case5a.m.

1	Figure 3-11 shows the fourbar linkage demonstrator model.  It consists of three moving elements (links 2, 3, and 4) plus the frame or ground link (1).  The motor drives link 2 through a gearbox.  The two fixed pivots are instrumented with piezoelectric force transducers to measure the dynamic forces acting in x and y directions on the ground plane.  A pair of accelerometers is mounted to a point on the floating coupler (link 3) to measure its accelerations.

2	Figure 3-12 shows a schematic of the linkage.  The links are designed with lightening holes to reduce their masses and mass moments of inertia.  The input to link 2 can be an angular acceleration (or a constant angular velocity) plus a torque.  Link 2 rotates fully about its fixed pivot at O2.  Even though link 2 may have a zero angular acceleration α2, if run at constant angular velocity ω2, there still will be time-varying angular accelerations on links 3 and 4 since they oscillate back and forth.  In any case, the CGs of the links will experience time-varying linear accelerations as the linkage moves.  These angular and linear accelerations will generate inertia forces and torques as defined by Newton's second law.  Thus, even with no external forces or torques applied to the links, the inertial forces will create reaction forces at the pins.  It is these forces that we wish to calculate.

3	Figure 3-13 shows the free-body diagrams of the individual links.  The local, nonrotating, coordinate system for each link is set up at its CG.  The kinematic equations of motion must be solved to determine the linear accelerations of the CG of each link and the link's angular acceleration for every position of interest during the cycle.  (See reference 1 for an explanation of this proceedure.)  These accelerations, AGn and αn, are shown acting on each of the n links and are given below.  The forces at each pin connection are shown as xy pairs, numbered as before, and are initially assumed to be positive.

From the kinematic analysis the link angles and accelerations are:

Link angles:

THETA3deg = 34.195471;          % deg

THETA3 = 34.195471 * (2*pi/360) % rad

THETA4deg = 87.945733;          % deg

THETA4 = 87.945733 * (2*pi/360) % rad

THETA3 =

0.5968

THETA4 =

1.5349

Accelerations:

ALPHA3 = 56.653182;     % rad/sec2

ALPHA4 = 137.95907;     % rad/sec2

AG2x = -291.306;                % in/sec2

AG2x = -291.306 * 25.4 / 1000   % m/sec2

AG2y = -432.471;                % in/sec2

AG2y = -432.471 * 25.4 / 1000   % m/sec2

AG2x =

-7.3992

AG2y =

-10.9848

AG3x = -1362.219;               % in/sec2

AG3x = -1362.219 * 25.4 / 1000  % m/sec2

AG3y = -309.2232;               % in/sec2

AG3y = -309.2232 * 25.4 / 1000  % m/sec2

AG3x =

-34.6004

AG3y =

-7.8543

AG4x = -546.6891;               % in/sec2

AG4x = -546.6891 * 25.4 / 1000  % m/sec2

AG4y = 115.39814;               % in/sec2

AG4y = 115.39814 * 25.4 / 1000  % m/sec2

AG4x =

-13.8859

AG4y =

2.9311

Calculating the x and y components of the position vectors:

Link 2:

R12x = -r2CG * cos(THETA2 + DEL2)   % m

R12y = -r2CG * sin(THETA2 + DEL2)   % m

R12x =

-0.0469

R12y =

-0.0713

R32x = a * cos(THETA2) + R12x       % m

R32y = a * sin(THETA2) + R12y       % m

R32x =

0.0851

R32y =

0.0049

Link 3:

R23x = -r3CG * cos(THETA3 + DEL3)   % m

R23y = -r3CG * sin(THETA3 + DEL3)   % m

R23x =

-0.1507

R23y =

-0.1776

R43x = b * cos(THETA3) + R23x       % m

R43y = b * sin(THETA3) + R23y       % m

R43x =

0.1855

R43y =

0.0508

Link 4:

R14x = -r4CG * cos(THETA4 + DEL4)   % m

R14y = -r4CG * sin(THETA4 + DEL4)   % m

R14x =

-0.0215

R14y =

-0.1006

R34x = c * cos(THETA4) + R14x       % m

R34y = c * sin(THETA4) + R14y       % m

R34x =

-0.0106

R34y =

0.2040

4	Equations 3.1 can be written for each moving link in the system.  The masses and the mass moments of inertia of each link about its CG must be calculated for use in these equations.

5	For link 2:

Fx=F12x+F32x=m2⋅AG2xFy=F12y+F32y=m2⋅AG2yMz=T2+R12x⋅F12y−R12y⋅F12x+R32x⋅F32y−R32y⋅F32x=IG2⋅α2	(a)

6	For link 3:

Fx=F32x+F43x=m3⋅AG3xFy=F32y+F43y=m3⋅AG3yMz=R23x⋅F23y−R23y⋅F23x+R43x⋅F43y−R43y⋅F43x=IG3⋅α3	(b)

7	For link 4:

Fx=F14x+F34x=m4⋅AG4xFy=F14y+F34y=m4⋅AG4yMz=R14x⋅F14y−R14y⋅F14x+R34x⋅F34y−R34y⋅F34x=IG4⋅α4	(c)

8	There are 13 unkowns in these nine equations: F12x, F12y, F32x, F32y, F23x, F23y, F43x, F43y, F14x, F14y, F34x, F34y, and T2.  Four third-law equations can be written to equate the action-reaction pairs at the joints.

F32x=−F23x F32y=−F23yF34x=−F43x F34y=−F43y	(d)

9	The set of thirteen equations in a through d can be solved simultaneously to determine the forces and driving torque either by matrix reduction or by iterative root-finding methods.  After reducing the number of unknowns to nine by substitution, we solve using matrix methods.

Coeff = [1 0 0 0 0 0 -1 0 0;...

0 1 0 0 0 0 0 -1 0;...

-R12y R12x 0 0 0 0 R32y -R32x 1;...

0 0 0 0 1 0 1 0 0;...

0 0 0 0 0 1 0 1 0;...

0 0 0 0 -R43y R43x -R23y R23x 0;...

0 0 1 0 -1 0 0 0 0;...

0 0 0 1 0 -1 0 0 0;...

0 0 -R14y R14x R34y -R34x 0 0 0];

rhs = [(m2*AG2x); (m2*AG2y); (IG2*ALPHA2);...

(m3*AG3x); (m3*AG3y); (IG3*ALPHA3);...

(m4*AG4x); (m4*AG4y); (IG4*ALPHA4)];

Soln = Coeff \ rhs;

F12x = Soln(1)  % N

F12y = Soln(2)  % N

F14x = Soln(3)  % N

F14y = Soln(4)  % N

F43x = Soln(5)  % N

F43y = Soln(6)  % N

F23x = Soln(7)  % N

F23y = Soln(8)  % N

T2 = Soln(9)    % N m

F12x =

-255.8752

F12y =

-177.9592

F14x =

201.0425

F14y =

167.0154

F43x =

215.6327

F43y =

163.9356

F23x =

-251.9880

F23y =

-172.1882

T2 =

-3.5174

Third-law equations:

F32x = -F23x    % N

F32y = -F23y    % N

F34x = -F43x    % N

F34y = -F43y    % N

F32x =

251.9880

F32y =

172.1882

F34x =

-215.6327

F34y =

-163.9356

Forces and torque:

Link 2:

F12x

F12y

F12x =

-255.8752

F12y =

-177.9592

F32x

F32y

F32x =

251.9880

F32y =

172.1882

Link 3:

F23x

F23y

F23x =

-251.9880

F23y =

-172.1882

F43x

F43y

F43x =

215.6327

F43y =

163.9356

Link 4:

F34x

F34y

F34x =

-215.6327

F34y =

-163.9356

F14x

F14y

F14x =

201.0425

F14y =

167.0154

Torque:

T2

T2 =

-3.5174