CASE STUDY 1A

Bicycle Brake Lever Loading Analysis

Problem	Determine the forces on the elements of the bicycle brake lever assembly shown in Figure 3-1 during braking.

Given	The geometry of each element is known.  The average human's hand can develop a grip force of about 267 N (60 lb) in the lever position shown.

Magnitude of handle force  Fb2

Fb2 = 267;                  % N

Direction of handle force  Fb2

THETAb2deg = 270;           % deg

THETAb2 = 270 * (2*pi/360)  % deg

THETAb2 =

4.7124

Direction of cable force  Fc2

THETAc2deg = 184;           % deg

THETAc2 = 184 * (2*pi/360)  % deg

THETAc2 =

3.2114

Direction of cable force  Fcable

THETAcabledeg = 180;           % deg

THETAcable = 180 * (2*pi/360)  % deg

THETAcable =

3.1416

Positon vector components

Rb2x = 19/1000;    % m

Rc2x = -25/1000;   % m

R12x = -12/1000;   % m

Rb2y = -4/1000;    % m

Rc2y = 0/1000;     % m

R12y = -7/1000;    % m

R21x = 7/1000;     % m

Rb1x = 47.5/1000;  % m

R31x = -27/1000;   % m

R21y = 19/1000;    % m

Rb1y = -14/1000;   % m

R31y = 30/1000;    % m

Assumptions	The accelerations are negligible.  All forces are coplanar and two-dimensional.  A class 1 load model is appropriate and a static analysis is acceptable.  The higher applied load will be used as a worst case, assuming that it can be reached before bottoming the tip of the handle on the handgrip.  If that occurs, it will change the beam's boundary conditions and the analysis.

Solution

See Figures 3-1, 3-2, M-Book CASE1A.doc and M-File Case1a.m.

1	Figure 3-1 shows the handbrake lever assembly, which consists of three subassemblies: the handlebar (1), the lever (2), and the cable (3).  The lever is pivoted to the handlebar and the cable is connected to the lever.  The cable runs within a plastic-lined sheath (for low friction) down to the brake caliper assembly at the bicycle's wheel rim.  The user's hand applies equal and opposite forces at some point on the lever and handgrip.  These forces are transformed to a larger force in the cable by reason of the lever ratio of part 2.

Figure 3-1 is a free-body diagram of the entire assembly since it shows all the forces and moments acting on it except for its weight, which is small compared to the applied forces and is thus neglected for this analysis.  The "broken away" portion of the handlebar provides internal x and y force components and a moment.  These are arbitrarily shown as positive in sign.  Their actual signs will "come out in the wash" in the calculations.  The known applied forces are shown in their actual directions and senses.

2	Figure 3-2 shows the three subassembly elements separated and drawn as free-body diagrams with all relevant forces and moments applied to each element, again neglecting the weights of the parts.  The lever (part 2) has three forces on it, Fb2, Fc2, and F12.  The two-character subscript notation used here should be read as, force of element 1 on 2 (F12) or force at B on 2 (Fb2), etc.  This defines the source of the forces (first subscript) and the element on which it acts (second subscript).

This notation will be used consistently throughout this text for both forces and position vectors such as Rb2, Rc2, and R12 in Figure 3-2, which serve to locate the above three forces in a local, nonrotating coordinate system whose origin is at the center of gravity (CG) of the element or subassembly being analyzed.  (See foot note on page 108 of the text).

On this brake lever, Fb2 is an applied force whose magnitude and direction are known.  Fc2 is the force in the cable.  Its direction is known but not its magnitude.  Force F12 is provided by part 1 on part 2 at the pivot pin.  Its magnitude and direction are both unknown.  We can write equations 3.3b for this element to sum forces in the x and y directions and sum moments about the CG.  Note that all unknown reactive forces and moments are initially assumed positive in the equations.  Their true signs will come out in the calculation.  (See foot note on page 109 of the text).

Fx=Fb2x+Fc2x+F12x=0Fy=Fb2y+Fc2y+F12y=0Fx=R12×F12+Rb2×Fb2+Rc2×Fc2=0	(a)

The cross products in the moment equation represent the "turning forces" or moments created by the application of these forces at points remote from the CG of the element.  Recall that these cross products can be expanded to

Mz=R12x⋅F12y−R12y⋅F12x ...+Rb2x⋅Fb2y−Rb2y⋅Fb2x ...+Rc2x⋅Fc2y−Rc2y⋅Fc2x=0	(b)

We have three equations and four unknowns (F12x, F12y, Fc2x, Fc2y) at this point, so we need another equation.  It is avalable from the fact that the direction of Fc2 is known.  (The cable can pull only along its axis).  We can express one component of the cable force Fc2 in terms of its other component and the known angle θc2 of the cable.

Fc2y=Fc2x⋅tan(θc2)	(c)

We now have four linear equations in four unknowns and we can use matrix methods to solve the equations directly.

Calculate components of Fb2

Fb2x = Fb2 * cos(THETAb2)   % N

Fb2y = Fb2 * sin(THETAb2)   % N

Fb2x =

-4.9047e-014

Fb2y =

-267

Establish the coefficient matrix describing the four equations to be solved.

Coeff = [1 0 1 0; 0 1 0 1; ...

-R12y R12x -Rc2y Rc2x; ...

0 0 -tan(THETAc2) 1];

Set the right-hand-side vector for the problem.

rhs = [-Fb2x; -Fb2y; -Rb2x*Fb2y + Rb2y*Fb2x; 0];

Solve the system of simultaneous linear equations.

Soln =Coeff \ rhs;

Components of the unknown forces F12, and Fc2

F12x = Soln(1)

F12y = Soln(2)

Fc2x = Soln(3)

Fc2y = Soln(4)

F12x =

1.0465e+003

F12y =

340.1800

Fc2x =

-1.0465e+003

Fc2y =

-73.1800

3	Part 3 in Figure 3-2 is the cable that passes through a hole in part 1.  This hole is lined with a low friction material, which allows us to assume no friction at the joint between parts 1 and 3.  We will further assume that the three forces F13, Fc3, and Fcable form a concurrent system of forces acting through the CG and thus create no moment.  With this assumption, only a summation of forces is necessary for this element.

Fx=Fcablex+F13x+Fc3x=0Fy=Fcabley+F13y+Fc3y=0	(d)

Using Newton's third law, we have

Fc3x = -Fc2x    % N

Fc3y = -Fc2y    % N

Fc3x =

1.0465e+003

Fc3y =

73.1800

We also assume that the cable entering from the left is horizontal and that the reaction F13 is vertical, thus

Fcabley = 0;    % N

F13x = 0;       % N

We can now solve for the forces on part 3 directly,

Fcablex = -F13x - Fc3x  % N

F13y = -Fcabley - Fc3y  % N

Fcablex =

-1.0465e+003

F13y =

-73.1800

4	The assembly of elements labeled part 1 in Figure 3-2 has both force and moments on it (i.e., it is not a concurrent system), so the three equations 3.3b are needed.

Fx=F21x+Fb1x+F31x+Px+Fsheathx=0Fy=F21y+Fb1y+F31y+Py=0Mz=Mh+R21×F21+Rb1×Fb1+R31×F31+Rp×Fp+Rd×Fsheath=0	(f)

Expanding cross products in the moment equation gives the moment magnitude as

Mz=Mh+R21x⋅F21y−R21y⋅F21x ...+Rb1x⋅Fb1y−Rb1y⋅Fb1x ...+R31x⋅F31y−R31y⋅F31x ...+RPx⋅FPy−RPy⋅FPx ...+0−Rdy⋅Fsheathx=0	(g)

Using Newton's third law, we have

F31x = -F13x    % N

F21x = -F12x    % N

Fb1x = -Fb2x    % N

F31y = -F13y    % N

F21y = -F12y    % N

Fb1y = -Fb2y    % N

F31x =

0

F21x =

-1.0465e+003

Fb1x =

4.9047e-014

F31y =

73.1800

F21y =

-340.1800

Fb1y =

267

Fsheathx = -Fcablex % N

Fsheathx =

1.0465e+003

Given

RPx = -27/1000  % m

RPy = 0         % m

Rdx = -41/1000  % m

Rdy = 27/1000   % m

RPx =

-0.0270

RPy =

0

Rdx =

-0.0410

Rdy =

0.0270

Again, we have a system of linear equations that can be solved using matrix methods. First, we establish the coefficient matrix that describes the equations to be solved.

Coeff = [1 0 0; 0 1 0; -RPy RPx 1];

Set the right-hand-side vector for these equations.

rhs = [(-F21x-Fb1x-F31x-Fsheathx);...

(-F21y-Fb1y-F31y);...

(-(R21x*F21y - R21y*F21x) - (Rb1x*Fb1y - Rb1y*Fb1x) -...

(R31x*F31y - R31y*F31x) + Rdy*Fsheathx)];

Solve the three equations simultaneously for Px, Py, and Mh.

Soln = Coeff \ rhs;

Px = Soln(1)

Py = Soln(2)

Mh = Soln(3)

Px =

0

Py =

-1.4211e-014

Mh =

0.0468

5	Summarizing, the results obtained for a grip force Fb2 = 267 N are:

Handlebar (1)

Fb1x

Fb1y

Fb1x =

4.9047e-014

Fb1y =

267

F21x

F21y

F21x =

-1.0465e+003

F21y =

-340.1800

F31x

F31y

F31x =

0

F31y =

73.1800

Px

Py

Mh

Px =

0

Py =

-1.4211e-014

Mh =

0.0468

Lever (2)

Fc2x

Fc2y

Fc2x =

-1.0465e+003

Fc2y =

-73.1800

F12x

F12y

F12x =

1.0465e+003

F12y =

340.1800

Cable (3)

Fc3x

Fc3y

Fc3x =

1.0465e+003

Fc3y =

73.1800

F13x

F13y

F13x =

0

F13y =

-73.1800

Fcablex

Fcabley

Fcablex =

-1.0465e+003

Fcabley =

0

6	A summary of the results for the given dimensions is given above in step 5, assuming a 267-N (60-lb) force applied by the person's hand normal to the brake lever.  The force generated in the cable (Fcable) is then 1047 N (235 lb), the reaction force against the handlebar (F21) is 1100 N (247 lb) at -198 deg, and a 34-N-m (303-lb-in) moment (Mh) is applied to the handlebar.

7	As an exercise, move the point of application of the hand force along the lever by changing the values Rb2x and Rb2y in the position vector data at the begining of the case study and then observe the changes to the forces and moments in step 5.