CASE STUDY 1B

Bicycle Brake Lever Stress and Deflection Analysis

Problem	Determine the stresses and deflections at critical points in the brake lever assembly shown in Figures 3-1 and 4-47.

Given	The geometry and loading are known from the previous Case Study 1A (p. 107).  The average human's hand can develop a grip force of about 267 N (60 lb) in the lever position shown.

Magnitude of handle force  Fb2

Fb2 = 267;      % N

Magnitude of cable force  Fc2

Fc2 = 1047;     % N

Magnitude of pivot force  F12

F12 = 1100;     % N

Horizontal distance, B to Q

dBQx = 76.2;    % mm

Handle diameter

dhandle = 14.3; % mm

Pivot pin diameter

dpivot = 8;     % mm

Flange thickness

t = 6.4;        % mm

Modulus of elasticity

E = 71.7;       % GPa

Assumptions	The most likely failure points are the two holes where the pins insert and at the root of the cantilever-beam lever handle.  The cross-section of the lever handle is essentially circular.

Solution	See Figures 4-47 and 4-48, M-Book CASE1B.doc and M-File Case1b.m.

1	The 14.3-mm-diameter portion of the handle can be modeled as a cantilever beam with an intermediate concentrated load as shown in Figure 4-48 if we assume that the more massive block at its left end serves as a "ground plane."  The location of likely failure is the root of the round-handle portion where the shear and moment are both maximum as shown in Figure 4-24 for this model that was analyzed for reactions, moments, and deflections in Examples 3-3 and 4-5.  The tensile bending stress at the root of the cantilever is maximum at the outer fiber (at point P as shown in Figure 4-47) and is found from equation 4.11b.  Summing forces and moments yields

R1 = Fb2                % N

M1 = dBQx/1000 * Fb2    % N m

R1 =

267

M1 =

20.3454

Distance to extreme fiber and moment of inertia:

c = 0.5 * dhandle           % mm

I = (pi/64) * dhandle^4     % mm^4

c =

7.1500

I =

2.0526e+003

The tensile bending stress is

SIGx = (M1 * c) / I * 1000  % MPa

SIGx =

70.8693

This is a relatively low stress for this material.  There is some stress concentration due to the small radius at the root of the beam but since this is made of a marginally ductile cast material (5% elongation to fracture) we can ignore the stress concentrations on the basis that local yielding will relieve them.

2	The effective length-to-depth ratio of this beam is small at 76.2 / 14.3 = 5.3.  Since this ratio is less than 10, the shear stress due to transverse loading will be calculated.  For this solid-circular section, it is found from equation 4.15c.  First, the maximum shear and cross-section area are:

V = R1                  % N

A = (pi/4) * dhandle^2  % mm2

V =

267

A =

160.6061

The maximum transverse shear stress  is:

TAUxy = (4*V) / (3*A)   % MPa

TAUxy =

2.2166

The shear stress is maximum at the neutral axis (point Q) and the normal bending stress is maximum at the outer fiber (point P).  The largest principal stress at the top outer fiber is then σ1 =  σx = 71 MPa, σ2 = σ3 = 0, and,

TAUmax = 0.5 * SIGx     % MPa

TAUmax =

35.4347

The Mohr circle for this stress element looks like the one in Figure 4-8.

3	The deflection calculation for the handle is complicated by its curved geometry and its slight taper from root to end.  A first approximation of the deflection can be obtained by simplifying the model to a straight beam of constant cross-section as shown in Figure 4-48b.  The deflection due to transverse shear will also be neglected.  This will be in slight error in a nonconservative direction but will never-the-less give an order-of-magnitude indication of the deflection.  If this result shows a problem with excessive deflection it will be necessary to improve the model.  Equation (i) from Example 4-5 provides the deflection equation for our simple model.  In this case,

l = 127;     % mm

a = 76.2;    % mm

and

x = l        % mm

x =

127

for maximum deflection at the end of the beam.

ymax = (Fb2 / (6*E*I)) *...

(x^3 - 3*a*x^2 - (x-a)^3) / 1000   % mm

ymaxInch = ymax / 25.4    % in

ymax =

-0.5351

ymaxInch =

-0.0211

This is a little over 1/64-in deflection at the handle-end, which is not considered excessive in this application.  See Figure 4-24 for a graph of the general shape of this beam's deflection curve, though the values are different in that example.

4	Other locations of likely failure must also be checked.  The material around the two holes may experience any of several modes of failure due to bearing stress, direct shear stress, or tear out.  The hole at point A in Figure 4-47 contains a pivot pin that bears against the handle with the 1100-N force shown.  We will check this for the three modes listed above.

5	Bearing stress is compressive and is considered to act upon the projected area of the hole, which in this case, is the 8-mm pivot hole diameter times the total length of the bearing (two 6.4-mm-thick flanges).

Abear = 2 * dpivot * t  % mm2

SIGbear = F12 / Abear   % MPa

Abear =

102.4000

SIGbear =

10.7422

6	Tearout in this case requires that four 6.4-mm-thick sections fail in shear through the 5-mm length of material between hole and edge.  (See also Figure 4-13 for a definition of tearout area).  Let edge distance be

dedge = 7.1;                % mm

Atearout = 4 * dedge * t    % mm2

TAUtearout = F12 / Atearout % MPa

Atearout =

181.7600

TAUtearout =

6.0519

7	These are very low stresses for the specified material but remember that the applied force used is based on typical human-hand force capability and does not anticipate abuse due to impact or other means.

8	The cable-end inserts in a blind hole, which is half-slotted to allow the cable to pass through at assembly as shown in Figure 4-47.  This slot weakens the part and makes the section at C the most likely failure location at this joint.  We will assume the failure of the open (slotted) half of the material around the hole is sufficient to disable the part since the cable-end could then slip out.  The small section that retains the cable pin can be modeled to a first approximation as a cantilever beam with a cross-sectional width of

w = 0.5 * ( 25 - 5)         % mm

w =

10

and a depth of

h = 5;                      % mm

This is a conservative assumption as it ignores the increase in depth due to the radius of the hole.  The moment arm of the force will be assumed to be equal to the radius of the pin, 4 mm.  The force on the slotted half of the width is taken as half of the total force of Fc2 = 1047 N on the cable.  The stress on the outer fiber at C is then

Moment arm

a = 4;                      % mm

Bending moment

M = 0.5 * (a/1000) * Fc2    % N m

M =

2.0940

Distance to outer fiber

c = 0.5 * h                 % mm

c =

2.5000

Moment of inertia

I = (w * h^3) / 12          % mm4

I =

104.1667

Bending stress

SIGx = (M*c) / I * 1000     % MPa

SIGx =

50.2560

and the shear stress at the neutral axis due to transverse loading is (from equation 4.14b)

Transverse shear

V = Fc2 / 2             % N

V =

523.5000

Area

A = w * h               % mm2

A =

50

Transverse shear stress

TAUxy = (3*V) / (2*A)   % MPa

TAUxy =

15.7050

9	The normal stress is principal here as shown in Figure 4-8 and the maximum shear stress is then half of the principal normal stress.  These are the highest stresses found for the three sections checked.  A failure analysis of this part will be done in the continuation of this case study in the next chapter.

10	A more complete analysis can also be done using finite element methods to determine the stresses and deformations at many other locations on the part.  This preliminary analysis shows some of the areas that might benefit from further investigation.