EXAMPLE 4-6

Finding Beam Slope and Deflection of an Overhung Beam Using Singularity Functions

Problem	Determine and plot the slope and deflection functions for an overhung beam with a uniformly distributed load over part of its length and a concentrated force at its end as shown in Figure 4-22c.

Given	Beam length

l = 10; % in

Load locations

a = 4;  % in

b = 7;  % in

Distributed load

w = 100;    % lbf / in

Concentrated load

F = 200; % lbf

Moment of inertia

I = 0.2;  % in4

Modulus of elasticity

E = 30e6;    % psi

Assumptions	The weight of the beam is negligible compared to the applied load and so can be ignored.

Solution	See Figures 4-22c, 4-25 and 4-26, M-Book Ex04-06.doc and M-File Ex04_06.m

1	The distributed load does not extend over the entire length of this beam.  All singularity functions extend from their initial point to the end of the beam.  So, to terminate the uniform load's step function at some point short of the end of the beam it is necessary to apply another step function of equal amplitude and opposite sign in order to cancel it for all points beyond length a as shown in Figure 4-25.  The sum of the two step functions of opposite sign is then zero to the right of distance a.

q(x) = R1<x>-1 - w<x>0 + w<x - a>0 + R2<x - b>-1 - F<x - l>-1	(a)

V(x) = R1<x>0 - w<x>1 + w<x - a>1 + R2<x - b>0 - F<x - l>0 + C1	(b)

M(x) = R1<x>1 - w<x>2/2 + w<x - a>2/2 + R2<x - b>1 - F<x - l>1 + C1x + C2	(c)

θ(x) = (R1<x>2/2 - w<x>3/6 + w<x - a>3/6 + R2<x - b>2/2 - F<x - l>2/2 +

C1x2/2 + C2x + C3) / EI	(d)

y(x) = (R1<x>3/6 - w<x>4/24 + w<x - a>4/24 + R2<x - b>3/6 - F<x - l>3/6 +

C1x3/6 + C2x2/2 + C3x + C4) / EI	(e)

2	Because the reactions have been included in the loading function, the shear and moment diagrams both close to zero at each end of the beam, making C1 = C2 = 0.

3	Since both the shear and moment are zero at  x = l+, the reactions R1 and R2 can be calculated simultaneously from equations (b) and (c) with x = l+ =  l:

V = R1 + w L + w (L – a) + R2 – F = 0	(f)

M = R1 L – (w/2) L2 + (w/2) (L – a)2 + R2 (L – b) = 0	(g)

R = inv([1 1; 1 (l-b)/l]) * [w*a + F;...

((w*l)/2 - (w*(l-a)^2)/(2*l))];

R1 = R(1)

R2 = R(2)

R1 =

200

R2 =

400

Note that equation (f) is just ΣFy = 0, and equation (g) is ΣMz = 0.

4	Substitute x = 0, y = 0 and x = b, y = 0 in equation (e)  and solve for C3 and C4:

At x = 0

C4 = 0	(h)

At x = b

(R1/6) b3 – (w/24)b4 + (w/24) (b-a)4 + C3b = 0	(i)

C3 = (1/b)*((-R1/6)*b^3 + (w/24)*b^4 -...

(w/24)*(b-a)^4)  % lbf in2

C3 =

-252.3810

5	Substitution of the expressions for C3 ,C4, R1 and R2 from equations f, g, h, and i into equation e gives the resulting deflection equation

y(x) = [R1x3/6 - wx4/24 + w<x - a>4/24 + R2<x - b>3/6 - F<x - l>3/6 +

x(-R1b3/6 + wb4/24 - w<b-a>4/24)/b ] / EI	(j)

6	Since an overhung beam is a form of cantilever beam, the maximum deflection is most likely at the free end.  Substitute x = l in equation j to find ymax.

ymax = [R1l3/6 - wl4/24 + w<l - a>4/24 + R2<l - b>3/6 - F<l - l>3/6 +

l(-R1b3/6 + wb4/24 - w<b-a>4/24)/b ] / EI	(k)

ymax = (1/(E*I))*((R1/6)*l^3 - (w/24)*l^4 + ...

(w/24)*(l-a)^4 + (R2/6)*(l-b)^3 + ...

(l/b)*((-R1/6)*b^3 + (w/24)*b^4 -...

(w/24)*(b-a)^4))  % in

ymax =

-6.0952e-004

7	The shear, moment, slope, and deflection diagrams can now be plotted with the same technique used in Examples 3-2B, 3-3B, and 3-4.

Range of x

x = 0:0.005*l:l;    % in

V = R1.*(x>=0) - w.*(x>=0).*x + w.*(x>=a).*(x-a) + ...

R2.*(x>=b);    % lbf

M = R1.*(x>=0).*x - (w/2).*(x>=0).*x.^2 + ...

(w/2).*(x>=a).*(x-a).^2 + R2.*(x>=b).*(x-b);    % lbf in

THETA = (1/(E*I))*((R1/2).*(x>=0).*x.^2 -...

(w/6).*(x>=0).*x.^3 + (w/6).*(x>=a).*(x-a).^3 +...

(R2/2).*(x>=b).*(x-b).^2 -...

(F/2).*(x>=l).*(x-l).^2 + C3);   % rad

y = (1/(E*I))*((R1/6).*(x>=0).*x.^3 -...

(w/24).*(x>=0).*x.^4 + (w/24).*(x>=a).*(x-a).^4 +...

(R2/6).*(x>=b).*(x-b).^3 -...

(F/6).*(x>=l).*(x-l).^3 + C3.*x);   % in

subplot(2,2,1)

plot(x,V)

title('Shear Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Shear, V (lbf)')

subplot(2,2,2)

plot(x,M)

title('Moment Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Moment, M (lbf in)')

subplot(2,2,3)

plot(x,THETA)

title('Slope Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Slope (Rad)')

subplot(2,2,4)

plot(x,y*1000)

title('Deflection Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Deflection (1000th in)')

FIGURE 4-26. Example 4-6 Graphs

8	Graphs of the shear, moment, slope, and deflection functions are shown above.