EXAMPLE 4-4

Finding Beam Slope and Deflection of a Simply Supported Beam Using Singularity Functions

Problem	Determine and plot the slope and deflection functions for the simply supported beam shown in Figure 4-22a.

Given	The load is uniform over part of the beam length.

Beam length

l = 10; % in

Load location

a = 4;  % in

Distributed load

w = 100;    % lbf/in

Moment of inertia

I = 0.163;  % in4

Modulus of elasticity

E = 30e6;    % psi

Assumptions	The weight of the beam is negligible compared to the applied load and so can be ignored.

Solution	See Figures 4-22a and 4-23, M-Book Ex04-04.doc and M-File Ex04_04.m

1	Solve for the reaction forces using equation 3.3.  Summing moments about the right-hand end and summing forces in the y direction:

Mz=0=R1⋅l−w⋅l−a22

R1 = (w*(l-a)^2) / (2*l)    % lbf

R1 =

180

Fy=0=R1−l−w⋅l−a+R2

R2 = w*(l-a) - R1   % lbf

R2 =

420

2	Write the equations for the load function in terms of equations 3.17 and integrate the resulting function four times using equations 3.18 to obtain the shear, moment, slope, and deflection functions.  For the simply supported beam with a distributed load over part of its length:

q(x) = R1<x - 0>-1 - w<x - a>0 + R2<x - l>-1	(c)

V(x) = R1<x - 0>0 - w<x - a>1 + R2<x - l>0  + C1	(d)

M(x) = R1<x - 0>1 - w<x - a>2/2 + R2<x - l>1  + C1x + C2	(e)

θ(x) = (R1<x - 0>2/2 - w<x - a>3/6 + R2<x - l>2/2  + C1x2/2 + C2x + C3 ) / EI	(f)

y(x) = (R1<x - 0>3/6 - w<x - a>4/24 + R2<x - l>3/6  + C1x3/6 + C2x2/2 + C3x + C4 ) / EI	(g)

3	There are four constants of integration to be evaluated.  The constants C1 and C2 are zero because the reaction forces and moments acting on the beam are included in the loading function.  The deflection y is zero at the supports.  The constants C3 and C4 are found by substituting the boundary conditions x = 0, y = 0 and x = l, y = 0 into equation g.

y(0) = 0 = R1<0 - 0>3/6 - w<0 - a>4/24 + R2<0 - l>3/6  + C3(0) + C4	(h)

C4 = 0; % lbf in3

y(l) = 0 = R1<l - 0>3/6 - w<l - a>4/24 + R2<l - l>3/6  + C3l	(i)

C3 = (1/l)*((w/24)*(l-a)^4 - (R1/6)*l^3)    % lbf in2

C3 =

-2460

4	The shear, moment, slope, and deflection diagrams can now be plotted with the same technique used in Examples 3-2B, 3-3B, and 3-4.

Range of x

x = 0:0.005*l:l;    % in

V = R1.*(x>=0).*(x-0).^0 - w.*(x>=a).*(x-a).^1 +...

R2.*(x>=l).*(x-l).^0;    % lbf

M = R1.*(x>=0).*(x-0).^1 - (w/2).*(x>=a).*(x-a).^2 +...

R2.*(x>=l).*(x-l).^1;    % lbf in

THETA = (1/(E*I))*((R1/2).*(x>=0).*(x-0).^2 -...

(w/6).*(x>=a).*(x-a).^3 + (R2/2).*(x>=l).*(x-l).^2 + C3);

y = (1/(E*I))*((R1/6).*(x>=0).*(x-0).^3 -...

(w/24).*(x>=a).*(x-a).^4 +...

(R2/6).*(x>=l).*(x-l).^3 + C3.*x);

subplot(2,2,1)

plot(x,V)

xlabel('Distance along beam, x (in)')

ylabel('Shear, V (lbf)')

subplot(2,2,2)

plot(x,M)

xlabel('Distance along beam, x (in)')

ylabel('Moment, M (lbf in)')

subplot(2,2,3)

plot(x,THETA)

xlabel('Distance along beam, x (in)')

ylabel('Slope (Rad)')

subplot(2,2,4)

plot(x,y)

xlabel('Distance along beam, x (in)')

ylabel('Deflection (1000th in)')

FIGURE 4-23. Example 4-4 Graphs

5	The maximum deflection will occur at the point in x where the slope of the deflection curve is zero.  From Figure 4-23, we see that this point will be near the middle of the beam where a < x < l.  Let the point where θ = 0 be x = c, then, solving for c

θ(c)=R12⋅c−02−w6⋅c−a3+C3

Expanding and collecting terms in powers of c,

c3−3⋅R1w+a⋅c2+3⋅a2⋅c−a3−6⋅C3w=0

The numerical coefficients of this equation put in nondimensional form are

A0 = -(a^3 + (6*C3)/w)  % 1/in3

A1 = 3*a^2  % 1/in2

A2 = -3*(R1/w + a)  % 1/in

A3 = 1

A0 =

83.6000

A1 =

48

A2 =

-17.4000

A3 =

1

Using the roots function, the roots are

A = [A3 A2 A1 A0];

r = roots(A)

r =

13.3279

5.2638

-1.1917

There are three real roots, but only one is on the beam and it is near the center.  The slope is zero at x = c, where

c = r(2)    % in

c =

5.2638

6	Use the value of c in equation g to find the largest deflection.

y(c) = ymax = (R1<c - 0>3/6 - w<c - a>4/24 + R2<c - l>3/6  + C3c ) / EI	(q)

ymax = (1/(E*I)) * ((R1/6)*c^3 - (w/24)*(c-a)^4 + C3*c)   % in

ymax =

-0.0018