EXAMPLE 12-2

Stress Analysis of a Bevel-Gear Train

Problem	Determine the bending and surface stresses and safety factors in a straight bevel gearset made of the same steel materials and operating under the same conditions for the same 5-year life as in Example 11-7.

Given	From Example 11-7:

Transmitted power

P = 10;          % hp

P = 10 * 6600.4 % lbf in / sec

P =

66004

Pinion shaft speed

OMEGAp = 2500;  % rpm

OMEGAp = 2500 * 2*pi    % rad / min

OMEGAp =

1.5708e+004

Tooth numbers

Np = 20;

Ng = 35;

Pressure angle

PHIdeg = 25;    % deg

PHI = PHIdeg * (2*pi/360)   % rad

PHI =

0.4363

Diametral pitch

pd = 8;         % in-1

Face width

F = 2.667;      % in

Fatigue strength

Bending

Sfb = 38.937 * 1e3; % psi

Surface

Sprmfc = 118 * 1e3; % psi

Sfc = 105063;       % psi

Assumptions	From Example 11-7:

Application factor

Ka = 1;

Ca = Ka;

Load dist. factor

Km = 1.6;

Cm = Km;

Size factor

Ks = 1;

Cs = Ks;

Surface factor

Cf = 1;

Velocity factor

Kv = 0.652;

Cv = Kv;

Hardness factor

CH = 1;

Temperature factor

CT = 1;

Reliability factor

CR = 1;

Life factor

CL = 0.890;

Elastic coefficient

Cp = 2276;  % psi^(1/2)

From this section:

Crowning factor

Cxc = 1;

Kx = Cxc;

Stress adj. factor

Cb = 0.634;

Mounting factor

Cmd = 1.5;

Solution	See M-Book Ex12-02.doc and M-File Ex12_02.m.

1	Determine the pinion torque from the given power and speed.

Tp = P / OMEGAp * 60    % lbf in

Tp =

252.1167

2	Find the pitch diameters of pinion and gear.

dp = Np / pd

dg = Ng / pd

dp =

2.5000

dg =

4.3750

3	Find the pitch cone angles from equation 12.7b:

ALPHAg = atan(Ng/Np)                      % rad

ALPHAgDeg = atan(Ng/Np) * (360/(2*pi))    % deg

ALPHAg =

1.0517

ALPHAgDeg =

60.2551

ALPHApDeg = 90 - ALPHAgDeg                % deg

ALPHAp = ALPHApDeg * (2*pi/360)           % rad

ALPHApDeg =

29.7449

ALPHAp =

0.5191

4	Find the pitch cone length L from equation 12.7a:

L = dp / (2 * sin(ALPHAp))  % in

L =

2.5195

5	Use the pitch cone length L to find a suitable face width, set to the maximum recommended value.

F = L / 3   % in

F =

0.8398

6	Look up the bending geometry factors for pinion and gear in Figure 12-5 to find

Jp = 0.237;

and

Jg = 0.201;

7	Find the bending stress in the pinion from equation 12.9 using Jp.

SIGbp = (2*Tp)/dp * pd/(F*Jp) * (Ka*Km*Ks)/(Kv*Kx)    % psi

SIGbp =

1.9894e+004

8	Find the bending stress in the gear from equation 12.9 using Jg.

SIGbg = (2*Tp)/dp * pd/(F*Jg) * (Ka*Km*Ks)/(Kv*Kx)    % psi

SIGbg =

2.3457e+004

Note that the gear tooth is more highly stressed than the pinion tooth because the long addendum on the pinion makes it stronger at the expense of the short-addendum gear tooth.

9	Look up the surface geometry factor for this combination of pinion and gear in Figure 12-6 to find

I = 0.076;

Use this in equation 12.11 to find TD.

TD = (F/2)*(I*Cv)/(Cs*Cmd*Cf*Ca*Cxc)*(((Sprmfc*dp)/...

(Cp*Cb))*((0.774*CH)/(CT*CR)))^2 % lbf in

TD =

347.3188

10	Since TD > Tp, z = 0.667.  Use these data to find the surface stress with equation 12.10.

SIGc = Cp * Cb * sqrt(((2*TD)/(F*I*dp^2)) *...

(Tp/TD)^0.667 * ((Ca*Cm)/Cv) * Cs * Cf * Cxc)   % psi

SIGc =

8.4769e+004

11	The safety factors can now be found as

Pinion

Nbp = Sfb / SIGbp

Nbp =

1.9572

Gear

Nbg = Sfb / SIGbg

Nbg =

1.6599

Surface

Nc = (Sfc/SIGc)^2

Nc =

1.5361

12	These are acceptable safety factors.