EXAMPLE 11-5

Bending Stress Analysis of a Spur-Gear Train

Problem	Determine a suitable face width and the bending stresses in the gear teeth of the 3-gear train from Example 11-4.

Given	Transmitted load

Wt = 432;        % lbf

Pinion speed

OMEGAp = 2500;   % rpm

OMEGAp = 2500 * 2*pi;   % rad/min

Train ratio

mG = 3.5;

Tooth numbers

Np = 14;

Ni = 17;

Ng = 49;

Diametral pitch

pd = 6;      % in-1

Pitch diameters

dp = 2.333;  % in

di = 2.833;  % in

dg = 8.167;  % in

Pressure angle

PHIdeg = 25; % deg

PHI = PHIdeg * (2*pi/360)   % rad

PHI =

0.4363

Assumptions	The teeth are standard AGMA full-depth profiles.  The load and source are both uniform in nature.  A gear quality index of 6 will be used.

Quality index

Qv = 6;

Solution	See M-Book Ex11-05.doc and M-File Ex11_05.m.

1	Even though the transmitted load is the same, the bending stress in the teeth of each size gear will be different because of its slightly different tooth geometry.  The general formula for tooth-bending stress is equation 11.15:

σb=Wt⋅pdF⋅J⋅Ka⋅KmKv⋅Ks⋅KB⋅KI

Wt, pd, F, Ka, Km, Kv, and Ks are common to all gears in the set and J, KB and KI are potentially different for each gear.

2	A first approximation of the face width can be estimated as a function of the diametral pitch.  Take the middle of the recommended face width factor range 8/pd < F < 16/pd for a first calculation:

F = 12/pd   % in

F =

2

3	Based on the assumption of uniform load and source, the application factor Ka can be set to

Ka = 1;

4	The load distribution factor Km can be estimated from Table 11-16 based on the assumed face width:

Km = 1.6;

5	The velocity factor Kv can be calculated from equations 11.16 and 11.17 based on the assumed gear-quality index Qv and the pitch-line velocity Vt.

Pitch-line velocity

Vt = (dp/2) * OMEGAp / 12   % ft / min

Vt =

1.5269e+003

Constants used to calculate Kv

B = 0.25 * (12 - Qv)^(2/3)

A = 50 + 56*(1-B)

B =

0.8255

A =

59.7730

Velocity factor

Kv = (A/(A + sqrt(Vt)))^B

Kv =

0.6602

6	The size factor Ks = 1 for all three gears.

Ks = 1;

7	These gears are all too small to have a rim and spokes, so

KB = 1;

8	The idler factor KIpg = 1 for the pinion and gear and KIi = 1.42 for the idler gear.

KIpg = 1;

KIi = 1.42;

9	The bending geometry factor J for the 25 deg, 14-tooth pinion in mesh with the 17- tooth idler is found from Table 11-13 to be Jp = 0.33.

Jp = 0.33;

The pinion-tooth bending stress is then

SIGbp = (Wt*pd)/(F*Jp) * (Ka*Km)/Kv * Ks * KB * KIpg    % psi

SIGbp =

9.5181e+003

10	The bending geometry factor J for the 25 deg, 17-tooth idler in mesh with the 14- tooth pinion is found from Table 11-13 to be Ji = 0.36.

Ji = 0.36;

The idler-tooth bending stress is then

SIGbi = (Wt*pd)/(F*Ji) * (Ka*Km)/Kv * Ks * KB * KIi     % psi

SIGbi =

1.2389e+004

Note in Table 11-13 that the idler has a different J factor when considered to be the "gear" in mesh with the smaller pinion (0.36) than when considered to be the "pinion" in mesh with the larger gear (0.37).  The smaller value of the two is used because it gives the higher stress.

11	The bending geometry factor J for the 25 deg, 49-tooth gear in mesh with the 17- tooth idler is found (by interpolation) from Table 11-13 to be Jg = 0.46.

Jg = 0.46;

The gear- tooth bending stress is then

SIGbg = (Wt*pd)/(F*Jg) * (Ka*Km)/Kv * Ks * KB * KIpg    % psi

SIGbg =

6.8282e+003

12	If this is an acceptable stress level then the assumed face width can be used.  This issue will be revisited for this design in a later example.