EXAMPLE 9-9

Determining Shaft Deflection by Numerical Integration

Problem	Find the deflection of the shaft in Example 9-2 using numerical integration.

Given	Refering to Figure 9-5, the shaft is steel with E = 30 x 106 psi.

E = 30e6;    % psi

Starting at A, the shaft diameters are

d0 = 0.875;  % in

d1 = 0.750;  % in

d2 = 0.669;  % in

d3 = 0.531;  % in

Other dimensions from Figure 9-5

Distance to gear shoulder

a = 1.50;    % in

Distance to gear plane

p = 2.00;    % in

Distance to right support

b = 5.00;    % in

Distance to sheave shoulder

c = 6.50;    % in

Distance to sheave plane

q = 6.75;    % in

Assumptions	The change in area moment of inertia at the keyseats is negligible and does not affect the results.

Solution	See Figures 9-5 and E9-6, M-Book Ex09-09.doc and M-File Ex09_09.m.

FIGURE E9-6. Shaft of Example 9-2 Showing Points Where Diameter and Load Change.

1	Starting with the letter "A" at the left-hand load, designate each change in diameter and each change in loading with the next letter in the alphabet until you reach the last load on the right.  In this case there are 3 changes in diameter (at B, D, and E) and 3 changes in load (at C, D and F), for a total of 6 letters ("A" through "F").  Set N equal to the number of designated stations:

Number of stations

N = 6;

2	Define the n and i indices that will be used in the numerical integration:

n = 4 * (N - 1) + 1     % indexing will run from 1 to n = 21

i = 1:n;

index(i) = i;

index=index';

n =

21

3	Defining the centerline of the shaft as the z axis with origin at A, determine the z-coordinates, zs, of the stations A, B, C, etc.  Also, define the diameters in order, starting at A.  For this example,

Station coordinates:

% zs = [0; a; p; b; c; q]		% (not used in this Matlab solution)

Diameters:

% d = [d0; d1; d2; d3]		% (not used in this Matlab solution)

4	Each station and the midpoint between stations will have two identical z-coordinates associated with it (except the last station, which will have only one z-coordinate). We define the location of each of the 21 points that will be used in the numerical integration. The result is shown in step 7.

z = [0;0;(a/2);(a/2);a;a;(a+p)/2;(a+p)/2;p;p;(p+b)/2;(p+b)/2;...

b;b;(b+c)/2;(b+c)/2;c;c;(c+q)/2;(c+q)/2;q];

5	Determine the area moment of inertia as a function of z.  Between each station there will be four identical values.  One just to the right of the station line, two at the midpoint, and one just to the left of the next station line.

ImA = 0                % in4

ImAB = (pi*d0^4)/64

ImBC = (pi*d1^4)/64

ImCD = (pi*d1^4)/64

ImDE = (pi*d2^4)/64

ImEF = (pi*d3^4)/64

ImA =

0

ImAB =

0.0288

ImBC =

0.0155

ImCD =

0.0155

ImDE =

0.0098

ImEF =

0.0039

I = [ImA;ImAB;ImAB;ImAB;ImAB;ImBC;ImBC;ImBC;ImBC;ImCD;ImCD;...

ImCD;ImCD;ImDE;ImDE;ImDE;ImDE;ImEF;ImEF;ImEF;ImEF]    % in4

I =

0

0.0288

0.0288

0.0288

0.0288

0.0155

0.0155

0.0155

0.0155

0.0155

0.0155

0.0155

0.0155

0.0098

0.0098

0.0098

0.0098

0.0039

0.0039

0.0039

0.0039

6	Determine the bending moment as a function of z.  Between each station there will be four values.  One just to the right of the station line, two at the midpoint, and one just to the left of the next station line.  From Example 9-1 and Figure 9-7, the magnitude of the bending moment at each station and the midpoints between stations is

MA = 0            % lbf in

MmidAB = 12.31

MB = 24.62

MmidBC = 28.77

MC = 32.83

MmidCD = 42.06

MD = 63.94

MmidDE = 36.54

ME = 9.13

MmidEF = 4.57

MF = 0

MA =

0

MmidAB =

12.3100

MB =

24.6200

MmidBC =

28.7700

MC =

32.8300

MmidCD =

42.0600

MD =

63.9400

MmidDE =

36.5400

ME =

9.1300

MmidEF =

4.5700

MF =

0

There are five station intervals, each with 4 evaluation points (one just to the right of a station line, two at the midpoint, and one just to the left of the next station line).  Thus, in this example, there are 21 evaluations of M needed (including one just to the left of station A).  Thus,

M = [MA;MA;MmidAB;MmidAB;MB;MB;MmidBC;MmidBC;MC;MC;MmidCD;MmidCD;...

MD;MD;MmidDE;MmidDE;ME;ME;MmidEF;MmidEF;MF]     % lbf in

M =

0

0

12.3100

12.3100

24.6200

24.6200

28.7700

28.7700

32.8300

32.8300

42.0600

42.0600

63.9400

63.9400

36.5400

36.5400

9.1300

9.1300

4.5700

4.5700

0

7	We can now review the data and calculated values in the table below.  The first column is the index i.  The second column gives the z-coordinates associated with each station and station midpoint.  The third column is the area moment of inertia.  The fourth column is the bending moment, and the fifth column is M/EI, which is calculated here, and called Mprm (M'). Mprm is scaled in the ALL matrix to display additional significant digits.

Mprm = M ./ (E .*I);

Mprm(1) = 0;     % handles NaN returned by 0/0 division

Mprm;

Warning: Divide by zero.

ALL = [index z I M (Mprm.*1e5)]

ALL =

1.0000         0         0         0         0

2.0000         0    0.0288         0         0

3.0000    0.7500    0.0288   12.3100    1.4260

4.0000    0.7500    0.0288   12.3100    1.4260

5.0000    1.5000    0.0288   24.6200    2.8521

6.0000    1.5000    0.0155   24.6200    5.2839

7.0000    1.7500    0.0155   28.7700    6.1745

8.0000    1.7500    0.0155   28.7700    6.1745

9.0000    2.0000    0.0155   32.8300    7.0459

10.0000    2.0000    0.0155   32.8300    7.0459

11.0000    3.5000    0.0155   42.0600    9.0268

12.0000    3.5000    0.0155   42.0600    9.0268

13.0000    5.0000    0.0155   63.9400   13.7226

14.0000    5.0000    0.0098   63.9400   21.6759

15.0000    5.7500    0.0098   36.5400   12.3872

16.0000    5.7500    0.0098   36.5400   12.3872

17.0000    6.5000    0.0098    9.1300    3.0951

18.0000    6.5000    0.0039    9.1300    7.7983

19.0000    6.6250    0.0039    4.5700    3.9034

20.0000    6.6250    0.0039    4.5700    3.9034

21.0000    6.7500    0.0039         0         0

8		We are now ready to perform the numerical integration.  We shall do this by writing function M-files that progressively sum first the area under the M/EI curve using trapezoidal integration (modTrapInt.m), then the area under the slope curve using Simpson's rule integration (modSimpInt.m). The function M-files are listed at the end of this example.

PHI = modTrapInt(z,Mprm);   % function m-files are used for recursion

PSI = modSimpInt(z,PHI);

We transpose the PHI and PSI vectors to display the results as column vectors:

PHI = PHI'  % transpose to create column vectors

PSI = PSI'

PHI =

1.0e-003 *

0

0

0.0053

0

0.0214

0

0.0357

0

0.0522

0

0.1728

0

0.3434

0

0.4711

0

0.5292

0

0.5365

0

0.5390

PSI =

0

0

0

0

0.0000

0

0

0

0.0000

0

0

0

0.0006

0

0

0

0.0013

0

0

0

0.0014

9	The constants of integration may now be evaluated based on the knowledge that at the supports the deflection must be zero.  For supports located at z = s1 and s2,

C3=ψs2−ψs1s1−s2		and	C4=s2⋅ψs2−s1⋅ψs1s1−s2

where ψs1 and ψs2 are the values of ψ at z = s1 and z = s2, respectively.

Left support

s1 = z(1)

s1 =

0

Right support

s2 = z(13)

s2 =

5

C3 = (PSI(13) - PSI(1)) / (s1 - s2)

C4 = (s2*PSI(1) - s1*PSI(13)) / (s1 - s2)

C3 =

-1.1443e-004

C4 =

0

10	The equations for the slope and deflection are determined now from equations d and e in Example 9-3.

Index to take values at the stations

i = 1:4:n;

Slope

THETA(i) = PHI(i) + C3;     % rad

THETAdeg = THETA .* (360 / (2 * pi));

Deflection

DEL(i) = PSI(i) + C3 * z(i);

Again, we transpose to create column vectors.

THETA = THETA'  % transpose to create column vectors

DEL = DEL'

THETA =

1.0e-003 *

-0.1144

0

0

0

-0.0930

0

0

0

-0.0622

0

0

0

0.2290

0

0

0

0.4148

0

0

0

0.4245

DEL =

1.0e-003 *

0

0

0

0

-0.1609

0

0

0

-0.2001

0

0

0

0

0

0

0

0.5177

0

0

0

0.6230

11	We can now generate a matrix [ALL] of values of θ and δ at each station and plot both as functions of z.

ii = 1:length(i);

ii = ii';

zPlot(ii,1) = z(i);

DELplot(ii,1) = DEL(i);

THETAplot(ii,1) = THETAdeg(i);

ALL = [ii zPlot DELplot*1000 THETAplot*1000]

ALL =

1.0000         0         0   -6.5561

2.0000    1.5000   -0.1609   -5.3305

3.0000    2.0000   -0.2001   -3.5630

4.0000    5.0000         0   13.1195

5.0000    6.5000    0.5177   23.7648

6.0000    6.7500    0.6230   24.3237

subplot(2,1,1)

plot(zPlot,THETAplot)

xlabel('Distance along shaft (in)')

ylabel('Slope (deg)')

subplot(2,1,2)

plot(zPlot,DELplot)

xlabel('Distance along shaft (in)')

ylabel('Deflection (in)')

FIGURE E9-7. Slope and Deflection Graphs

12	We see that the slope is maximum at the sheave (z = 6.75 in) with a value of 0.425*10-3 rad (0.0243°).  The deflection is also a maximum at the sheave with a value of 0.00062 in.  The values are approximations that could be refined if we divided the shaft into smaller intervals for the integration.  Although we used 21 points in the integration, we ultimately had only the original six stations in the table and graphs above.  Notice that the bending moment changes rapidly along the shaft but in this model it is kept constant between stations and their midpoints.  We could also get a better estimate by using a cubic spline fit to the data points that were calculated.

Function M-File: modTrapInt.m

function PHI = modTrapInt(x,y)

% This is a modified trapezoidal integration designed

% to be compatible with Example 9-9.

PHI(1) = 0;    % initialize summing variable

for i = 1:2:(length(x)-2)

PHI(i+2) = PHI(i) + (1/2)*(y(i+2) + y(i+1))*(x(i+2) - x(i+1));

end

Function M-File: modSimpInt.m

function PSI = modSimpInt(x,y)

% This is a modified Simpson's integration designed

% to be compatible with Example 9-9.

PSI(1) = 0;    % initialize summing variable

for j = 1:4:(length(x)-4)

PSI(j+4) = PSI(j) + (1/6)*(y(j+4) + 4*y(j+2) +...

y(j))*(x(j+4) - x(j));

end