EXAMPLE 3-1

Impact Loading on an Axial Rod

Problem	The axial rod shown in Figure 3-18a is hit by a mass moving at 1 m/sec.

a.  Determine the sensitivity of the impact force to the length /diameter ratio of the rod for a constant 1 kg moving mass.

b.  Determine the sensitivity of the impact force to the ratio of moving mass to rod mass for a constant length/diameter ratio of 10.

Given	Rod length

ell = 100;   % mm

Rod modulus of elasticity

E = 207;   % GPa

Rod mass density

RHO = 7.86;   % gm / cm3

Impact velocity

vi = 1;   % m / sec

Assumptions	An approximate energy method will be acceptable.  The correction factor for energy dissipation will be applied.

Solution	See Figures 3-18a, 3-20, and 3-21; M-Book Ex03-01.doc and M-File Ex03_01.m

1	Figure 3-18a shows the system.  The moving mass strikes the flange on the end of the rod with stated velocity of 1 m/sec.

2	For part (a), we will keep the moving mass constant at 1 kg, and the rod length constant at 100 mm and vary the rod diameter to obtain l/d ratios in the range of 1 to 20.  The static deflection that would result from application of the weight force of the mass is calculated from the expression for the deflection of a bar in tension.  (See equation 4.8 in the next chapter for derivation.)

Part (a)			Mass of striker

m = 1;    % kg

Rod diameters

d = 5:2.5:100;    % mm

Rod stiffness

k = (pi .*d .^2)/4 .* (E/ell) * 1e6;    % N / m

Rod mass

mb = (pi .* d.^2)/4 .* ell*RHO /1e6;    % kg

Gravitational Acceleration

g = 9.8;    % m / sec2

Static deflection of rod

delta_st = (m * g) ./ k;   % m

Correction factor

eta = 1 ./ (1 + (mb ./ (3*m)));

Force ratio

FprimeI = vi .* sqrt(eta ./ (g .* delta_st));

Length/diameter ratio

Lprime = ell ./ d;

Plot the result

subplot(2,2,1)

plot(Lprime,FprimeI)

xlabel('l/d ratio')

ylabel('force ratio')

Impact force

Fi = vi .* sqrt(eta .* m .* k);    % N m / sec2

Plot the result

subplot(2,2,2)

plot(Lprime,Fi./1000)

xlabel('l/d ratio')

ylabel('force (kN)')

The variation in force ratio with changes in l/d ratio for a constant amount of moving mass and a constant impact velocity (i.e. constant input energy) is shown in Figure 3-20.  As the l/d ratio is reduced, the rod becomes much stiffer and generates much larger dynamic forces from the same impact energy.  This clearly shows that impact forces can be reduced by increasing compliance (reducing stiffness) of the impacted system.

3	For part (b), we will keep the l/d ratio constant at 10 and vary the ratio of moving mass to rod mass over the range of 1 to 20.  Figure 3-21a shows that the dynamic force ratio Fi/W varies inversely with the mass ratio.  However, the value of the dynamic force is increasing with mass ratio as shown in Figure 3-21b, because the static force W is also increasing with mass ratio.

Part (b)	l/d ratio

loverd = 10;

Rod diameter

dprime = ell / loverd / 1000     % m

dprime =

0.0100

Mass of rod

mb = (pi * dprime^2)/4 * ell*RHO     % kg

mb =

0.0617

Mass ratio

msratio = 1:1:20;

Striker mass

m = msratio .* mb;     % kg

Rod stiffness

k = (pi * dprime^2)/4 * E/ell * 1e12;    % N / m

Static deflection of rod

delta_st = m .* g ./ k;     % m

Correction factor

eta = 1 ./ (1 + (mb ./ (3*m)));

Force ratio

FprimeI = vi .* sqrt(eta ./ (g .* delta_st));

Plot the result

subplot(2,2,3)

plot(msratio,FprimeI)

xlabel('mass ratio')

ylabel('force ratio')

Impact force

Fi = vi .* sqrt(eta .* m .* k);    % N m / sec2

Plot the result

subplot(2,2,4)

plot(msratio,Fi./1000)

xlabel('mass ratio')

ylabel('force (kN)')