EXAMPLE 15-2

Design of a Short-Shoe Drum Brake

Problem	For the drum brake arrangement shown in Figure 15-8, determine the ratio c/r that will give a self-energizing ratio Fn/Fa of 2.  Also find the c/r ratio that will cause self-locking.

Given	The dimensions are

a = 6;   % in

b = 6;   % in

r = 5;   % in

Self-energizing ratio

SEratio = 2;

Assumptions	Coefficient of friction

MU = 0.35;

Solution	See Figure 15-8, M-Book Ex15-02.doc and M-File Ex15_02.m.

1	Rearrange equation 15.11 to form the desired ratio.

FnFa=SEratio=ab−μ⋅c

2	Substitute the desired self-energizing ratio, the given dimensions, and solve for c.

c = (1/MU) * (b - a/SEratio)    % in

c =

8.5714

3	Form the c/r ratio for a self-energizing ratio of 2 with the given brake geometry.

crRatio = c / r

crRatio =

1.7143

4	For self-locking to begin, Fa becomes zero, making Fn/Fa infinite and Fa/Fn = 0.  The second of these ratios will need to be used to avoid division by zero.  Rearrange equation 15.11 to form the desired ratio and solve for c.

FaFn=b−μ⋅ca=0

c = b / MU  % in

c =

17.1429

5	Form the c/r ratio for self-locking with the given brake geometry.

crRatio = c / r

crRatio =

3.4286

6	Note that these ratios are specific to the dimensions of the brake.  The length a was set equal to b in this example in order to eliminate the effect of the lever arm ratio a/b, which further reduces the application force Fa required for any normal force Fn.