EXAMPLE 11-4

Load Analysis of a Spur-Gear Train

Problem	Determine the torques and transmitted loads on the gear teeth in a 3-gear train containing a pinion, an idler gear, and a gear.  Find the gear diameters and the mean and alternating components of trans- mitted load on each gear.

Given	The pinion shaft passes 20 hp at 2500 rpm.  The train ratio is 3.5:1.  The pinion has 14 teeth, a 25 deg  pressure angle, and pd = 6.  The idler has 17 teeth.

Transmitted power

P = 20;                      % hp

P = 20 * 6600.4              % lbf in / sec

P =

132008

Pinion speed

OMEGAp = 2500;               % rpm

OMEGAp = 2500 * 2 * pi / 60  % rad / sec

OMEGAp =

261.7994

Train ratio

mG = 3.5

mG =

3.5000

Pinion teeth

Np = 14;

Idler teeth

Ni = 17;

Pressure angle

PHI = 25;    % deg

PHI = 25 * (2*pi/360)   % rad

PHI =

0.4363

Diametral pitch

pd = 6;      % in-1

Assumptions	The pinion meshes with the idler and the idler meshes with the gear.

Solution	See M-Book Ex11-04.doc and M-File Ex11_04.m.

1	Find the number of gear teeth from the given information:

Ng = mG * Np

Ng =

49

2	The torque on the pinion shaft is found from equation 9.1:

Tp = P / OMEGAp   % lbf in

Tp =

504.2334

3	The output torque is

Tg = mG * Tp      % lbf in

Tg =

1.7648e+003

4	The pitch diameters are

Pinion

dp = Np / pd      % in

dp =

2.3333

Idler

di = Ni / pd      % in

di =

2.8333

Gear

dg = Ng / pd      % in

dg =

8.1667

5	The transmitted load is the same on all three gears and can be found from the torque and radius of any one of the gears:

Wt = Tp / (0.5 * dp)    % lbf

Wt =

432.2001

6	The radial component of load is

Wr = Wt * tan(PHI)      % lbf

Wr =

201.5382

7	The total load is

W = Wt / cos(PHI)       % lbf

W =

476.8800

8	The repeated loads on any pinion or gear tooth are

Alternating

Wta = 0.5*Wt    % lbf

Wta =

216.1000

Mean

Wtm = 0.5*Wt    % lbf

Wtm =

216.1000

9	The fully reversed loads on the idler are

Alternating

Wta = Wt        % lbf

Wta =

432.2001

Mean

Wtm = 0;        % lbf