EXAMPLE 9-5

Designing an Interference Fit

Problem	Redesign the attachment of the gear to the shaft in Figure 9-3 to make it an interference fit rather than a keyed connection.  Define the shaft and gear hole dimensions and their tolerances for a press fit.

Given	Gear hub diameter

dhub = 3.00; % in

Gear hub length

L = 1.50;    % in

Peak shaft torque

Tp = 146;    % lbf in

Shaft diameters

d0 = 0.875;  % in

d1 = 0.750;  % in

Assumptions	The shaft material is the same as in Example 9-2.  Class 30 gray cast iron is used for the gear with properties below.  See Figure 9-20 for stress concentration factors.  The 0.750-in shaft will be increased slightly to 0.781 (25/32) nominal, where it press fits into the gear hub to allow the gear to slip over the rest of the shaft at assembly.

Shaft (subscript i), Hub (subscript o)

Tensile strength

Suti = 53;   % ksi

Suto = 32;   % ksi

Yield strength

Syi = 44;    % ksi

Modulus

Ei = 30e6;   % psi

Eo = 14e6;   % psi

Poisson's ratio

vi = 0.28;

vo = 0.28;

New shaft diameter

d1 = 0.781;  % in

Solution	See M-Book Ex09-05.doc and M-File Ex09_05.m.

1	The nominal diameter of the shaft at the gear hub is 0.781 in.  Based on the rule of thousandths, a reasonable diametral interference would be 0.0015 in.  From this assumption, the bearing pressure after pressing can be found from equation 9.14.

Diametral interference

DEL = 0.0015;   % in

Hub radius

ro = 0.5 * dhub % in

ro =

1.5000

Nominal interface radius

r = 0.5 * d1    % in

r =

0.3905

Shaft inside diameter

ri = 0;         % in

Interface pressure

p= (DEL/(2*r))*(1/((1/Eo)*((ro^2 + r^2)/(ro^2 - r^2) + vo)+...

(1/Ei)*((r^2 + ri^2)/(r^2 - ri^2) - vi)))   % psi

p =

1.5265e+004

2	The stresses in the shaft after pressing are found from equations 9.15.

Tangential

SIGti = -p*(r^2 + ri^2) / (r^2 - ri^2)   % psi

SIGti =

-1.5265e+004

Radial

SIGri = -p  % psi

SIGri =

-1.5265e+004

3	The stresses in the hub after pressing are found from equations 9.16.

Tangential

SIGto = p*(ro^2 + r^2) / (ro^2 - r^2)   % psi

SIGto =

1.7485e+004

Radial

SIGro = -p  % psi

SIGro =

-1.5265e+004

4	To find the stress-concentration factor we need the hub length to shaft diameter ratio, l/d:

Length to diameter ratio

Ld1Ratio = L / d1

Ld1Ratio =

1.9206

and the ratio of press fit pressure to nominal bending stress

Bending stress	σ=M⋅cI=32⋅Mπ⋅d3

M = 65.6;    % lbf in

SIG = (32 * M) / (pi * d1^3)    % psi

SIG =

1.4027e+003

Ratio of pressure to stress

pSIGratio = p / SIG

pSIGratio =

10.8833

5	Taking these values to Figure 9-20, we find that the l/d ratio and the p/? ratio are both off the chart.  Using an approximate value, let

Stress-concentration factor

Kt = 2.2;

6	The safety factors against failure (yielding in the shaft and fracture of the hub) during press fit can now be found:

Shaft

Nsi = -Syi / (Kt * SIGti) * 1000

Nsi =

1.3101

Hub

Nso = Suto / (Kt * SIGto) * 1000

Nso =

0.8319

7	The shaft is safe, but the hub is at failure with this much interference.  The computations were repeated for a range of interference values from 0.0008 to 0.0015 in and those results are shown in Table 9-4.  The hub safety factors range from 2.3 to 1.0.

8	We would like a tolerance of at least 0.0002 in on each part or a total variation in interference of at least 0.0004 in for a set of mass-produced parts.  So we choose to set the range of interference from 0.0008 to 0.0013 in, a total range of 0.0005 in.

9	The part dimensions are then established using the basic hole system as

Hub diameter = 0.7812 +0.0002/-0.0000 = 0.78140.7812in

Dmax = 0.7814;   % in

Dmin = 0.7812;   % in

Shaft diameter = 0.7825 +0.0000/-0.0003 = 0.78250.7822 in

dmax = 0.7825;   % in

dmin = 0.7822;   % in

giving a range of interference of

DELmin = dmin - Dmax    % in

DELmax = dmax - Dmin    % in

DELmin =

8.0000e-004

DELmax =

0.0013

10	What torque will this pressed joint transmit at its minimum interference assuming

Coefficient of friction

MU = 0.15;

T= (pi*L*r*MU*DELmin)*...

(1/((1/Eo)*((ro^2 + r^2)/(ro^2 - r^2) + vo)+...

(1/Ei)*((r^2 + ri^2)/(r^2 - ri^2) - vi)))   % lbf in

T =

1.7551e+003

This is well in excess of the peak operating torque of 146 in-lbf, so will work.