EXAMPLE 4-8

Design of a Torsion Bar

Problem	Determine the best cross-sectional shape for a hollow torsion bar to be made from a sheet of steel of known dimensions in order to withstand a pure torsional load with minimum angular deflection.  Also find the maximum shear stress.

Given	Applied torque

T = 10;  % N m

Length of sheet

l = 1;   % m

Width of sheet

w = 100; % mm

Thickness of sheet

t = 1;   % mm

Shear modulus

G = 80.8;    % GPa  (GPa = 10e9 x Pa)

Assumptions	Try four different cross-sectional shapes:  unformed flat plate, open-circular section, closed-square circular section, and closed-square section.  The closed shapes' seams are welded to create a continuous cross section.  Assume a mean diameter or mean perimeter consistent with the sheet width.

Solution	See Figure 4-29 and Table 4-6, M-Book Ex04-08.doc and M-File Ex04_08.m

1	Equations 4.26 will apply for all sections provided that we substitute J for K and J/r for Q in the case of the closed-circular section.

2	The unformed flat plate behaves as a solid rectangular section as shown in Figure 4-28a and Table 4-6.  It has dimensions

a = w/2 % mm

b = t/2 % mm

a =

50

b =

0.5000

Calculating K and Q

K = a*b^3 * ((16/3) - 3.36*(b/a)*(1-(b^4/(12*a^4))))    % mm4

K =

33.1233

THETA = (T*l)/(G*K) * 1000 % rad

THETAdeg = THETA * (360 / (2*pi))   % deg

THETA =

3.7364

THETAdeg =

214.0806

This is obviously a rather large angular deflection indicating that the flat plate has been wound into a corkscrew by this torsional load.

Q = (8 * a^2 * b^2) / (3 * a + 1.8 * b) % mm3

Q =

33.1345

TAU = (T / Q) * 1000 % MPa

TAUpsi = TAU * 14.696/101326 * 1e6   % psi

TAU =

301.8000

TAUpsi =

4.3772e+004

The maximum shear stress is

TAU

TAU =

301.8000

(MPa) which would require a material with a tensile yield strength of over 520 MPa (75000 psi) in order not to yield and take a set.  This requires a high-strength steel.  (See Section 5.1 for discussion of this relationship between tensile and shear strength).

3	The open-circular shape is now formed into a 3.18-cm diameter tube but its longitudinal seam is left unwelded and open as shown in Figure 4-29b.  The expressions for K and Q from Table 4-6 are

Mean radius

r = 1/2 * ((w/pi) - t)  % mm

2*r

r =

15.4155

ans =

30.8310

K = (2/3) * pi * r * t^3    % mm4

K =

32.2861

THETA = (T*l)/(G*K) * 1000 % rad

THETAdeg = THETA * (360 / (2*pi))   % deg

THETA =

3.8333

THETAdeg =

219.6318

This is essentially as large an angular deflection as that of the flat plate.

Q = (4 * pi^2 * r^2 * t^2) / (6 * pi * r + 1.8 * t) % mm3

Q =

32.0874

TAU = (T / Q) * 1000 % MPa

TAUpsi = TAU * 14.696/101326 * 1e6   % psi

TAU =

311.6491

TAUpsi =

4.5201e+004

The stress and deflection are unacceptable.  It is just as bad a design as the flat plate.

4	The closed-square tube is formed by folding the sheet into a square section with side dimension

a = w/4

a =

25

The seam is welded as shown in Figure 4-29c.  From Table 4-6, K and Q and the stress and deflection are now

K = (2 * t^2 * (a-t)^4) / (2 * a * t - 2 * t^2) % mm4

K =

13824

THETA = (T*l)/(G*K) * 1000 % rad

THETAdeg = THETA * (360 / (2*pi))   % deg

THETA =

0.0090

THETAdeg =

0.5130

Q = 2 * t * (a-t)^2 % mm3

Q =

1152

TAU = (T / Q) * 1000 % MPa

TAUpsi = TAU * 14.696/101326 * 1e6   % psi

TAU =

8.6806

TAUpsi =

1.2590e+003

This angular deflection of the sqaure tube is much less than that of either open section and the maximum shear stress is now much more reasonable.

5	The closed-circular shape is formed into a 3.18-cm diameter tube and its longitudinal seam is welded shut as shown in Figure 4-29d.  We can either use equations 4.24 and 4.25 or use the general equations 4.26 involving K and Q, which are now a function of J for this circular shape.  For deflection,

Outside diameter

do = w/pi   % mm

do =

31.8310

Inside diameter

di = do - 2 * t % mm

di =

29.8310

K = (pi/32) * (do^4 - di^4) % mm4

K =

2.3041e+004

THETA = (T*l)/(G*K) * 1000 % rad

THETAdeg = THETA * (360 / (2*pi))   % deg

THETA =

0.0054

THETAdeg =

0.3078

and for the maximum shear stress at the outer surface,

Outside radius

r = w / (2 * pi)    % mm

r =

15.9155

Q = K / r   % mm3

Q =

1.4477e+003

TAU = (T / Q) * 1000 % MPa

TAUpsi = TAU * 14.696/101326 * 1e6   % psi

TAU =

6.9073

TAUpsi =

1.0018e+003

Note the much smaller stress and deflection for this design.  It is clearly the best choice of the four designs presented and is viable for any steel, though the wall thickness could be increased to further reduce stress and deflection if desired.  It needs to be checked for possible torsional buckling as well.