EXAMPLE 14-2

Preloaded Fasteners in Static Loading

Problem	Determine a suitable bolt size and preload for the joint shown in Figure 14-23.  Find its safety factor against yielding and separation.  Determine the optimum preload as a percentage of proof strength to maximize the safety factors.

Given	The joint dimensions are:

Cylinder diameter

D = 1.00;    % in

Clamp length

lm = 2.00;   % in

Applied load

P = 2000;    % lbf

Assumptions	Both of the clamped parts are steel.  The effects of the flanges on the joint stiffness will be ignored.  A preload of 90% of the bolt's proof strength will be applied as a first trial.

Young's modulus

E = 30e6;    % psi

Preload fraction of proof strength

fp = 0.9;

Solution	See Figures 14-23 and 14-25, M-Book Ex14-02.doc and M-File Ex14_02.m.

1	As with most design problems, there are too many unknown variables to solve the necessary equations in one pass.  Trial values must be chosen for various parameters and iteration used to find a good solution.  We actually made several iterations to solve this problem, but will only present two in the interest of brevity.  Thus the trial values used here have already been massaged to reasonable values.

2	The bolt diameter is the principal trial value to be chosen along with a thread series and a bolt class to define the proof strength.  We choose a 5/16-18 UNC-2A steel bolt of SAE class 5.2.  (This was actually our third trial choice.)  For a clamp length of 2 in, assume a bolt length of 2.5 in to allow sufficient protrusion for the nut.  The preload is taken at 90% of proof strength as assumed above but we will carry the fraction of proof strength along as a paraemeter so that we can adjust it later.

Bolt diameter

d = 0.3125;  % in

Bolt length

l = 2.500;   % in

Shank area

Ab = (pi * d^2) / 4   % in2

Ab =

0.0767

3	Table 14-6 shows the proof strength of this bolt to be Sp = 85 ksi.

Sp = 85 * 1000  % psi

Sp =

85000

The tensile stress area from Table 14-1 is

At = 0.05243;   % in2

The preload is then

Fi = @(fp) fp * Sp * At;   % lbf

Fi(0.9)   % lbf

ans =

4.0109e+003

4	Find the lengths of thread lthd and shank ls of the bolt as shown in Figure 14-21:

lthd = 2*d + 0.25   % in

ls = l - lthd       % in

lthd =

0.8750

ls =

1.6250

from which we can find the length of thread lt that is in the clamp zone:

lt = lm - ls        % in

lt =

0.3750

5	Find the stiffness of the bolt from equation 14.11a.

kb = 1/((lt / (At*E)) + (ls / (Ab*E)))  % lbf / in

kb =

1.0586e+006

6	The calculation for the stiffness of the clamped material is simplified in this example by its relatively small diameter.  We can assume in this case that the entire cylinder of material is compressed by the bolt force.  (We will soon address the problem of finding the clamped area in a continuum of material.)  The material stiffness from equation 14.11d is

km = (pi/4) * (D^2 - d^2) * (E/lm)  % lbf / in

km =

1.0630e+007

7	The joint stiffness factor from equation 14.13c is

C = kb / (km + kb)

C =

0.0906

8	The portions of the applied load P felt by the bolt and the material can now be found from equations 14.13.

Pb = C * P          % lbf

Pm = (1 - C) * P    % lbf

Pb =

181.1275

Pm =

1.8189e+003

9	Find the resulting loads in bolt and material after the load P is applied.

Fb = @(fp) Fi(fp) + Pb;  % lbf

Fb(0.9)   % lbf

ans =

4.1920e+003

Fm = @(fp) Fi(fp) - Pm;  % lbf

Fm(0.9)   % lbf

ans =

2.1920e+003

Note how little the applied load adds to the preload in the bolt.

10	The maximum tensile stress in the bolt is

SIGb = @(fp) Fb(fp) / At;    % psi

SIGb(0.9)

ans =

7.9955e+004

Note that no stress-concentration factor is used because it is static loading.

11	This is a uniaxial stress situation, so the principal stress and von Mises stress are identical to the applied tensile stress.  The safety factor against yielding for Grade 5.2 with Sy = 92 ksi is then

Sy = 92*1000    % psi

Ny = @(fp) Sy ./ SIGb(fp);

Ny(0.9)

Sy =

92000

ans =

1.1507

The yield strength is found from Tables 14-6 and 14-7.

12	The load required to separate the joint and the safety factor against joint separation are found from equations 14.14c and 14.14d.

P0 = @(fp) Fi(fp) / (1 - C);   % lbf

P0(0.9)

ans =

4.4103e+003

Nsep = @(fp) P0(fp) ./ P;

Nsep(0.90)

ans =

2.2052

13	The safety factor against separation is acceptable, but the yielding safety factor is too low.  We will now solve for these two safety factors as functions of the preload factor fp, which ranges from 0 to 1and plot them versus preload percentage.  The results are shown in Figure 14-25.

The separation safety factor (dashed line) rises linearly with increased preload, but is <1 until the preload exceeds about 40% of the proof strength.  At least that much preload is needed to keep the joint together under the applied load.

The yielding safety factor (solid line) is high at low preloads and decreases nonlinearly with increasing preload.  The two lines cross at a preload of about 65% of proof strength at point A.  That preload would balance the safety factors against both modes of failure at a value of 1.6.  However, if the goal is to protect the joint against possible overloads, then a larger preload is better.  At point B, the safety factor against overloading is 2.2 and there is still a 15% reserve against yielding during preloading as shown in the equations above.

Range of preload fraction:

fp = 0:0.05:1;

NyPlot = Ny(fp);

NsepPlot = Nsep(fp);

plot(fp,NyPlot,'-',fp,NsepPlot,'--')

xlabel('Preload as fraction of proof strength')

ylabel('Safety factors')

ylim([0 5])

legend('Ny','Nsep')

FIGURE 14-25. Safety Factors Versus Preload Fraction for the Statically Loaded Bolt in Example 14-2

14	The recommended design is then a 5/16-18 UNC-2A, grade 5.2 bolt, 2.5 in long, preloaded to 90% of proof strength with a preload force of

Fi = 0.90 * Sp * At   % lbf

Fi =

4.0109e+003