EXAMPLE 11-3

Analyzing an Epicyclic Gear Train

Problem	Determine the train ratio for the epicyclic train shown in Figure 11-16.

Given	The sun has 40 teeth, the planet 20 teeth, and the ring gear 80 teeth.  The arm is the input and the sun is the output.  The ring gear is held stationary.

Tooth numbers

N2 = 40;

N3 = 20;

N4 = 80;

Input speed (arm)

OMEGAarm = 1;

Ring gear speed

OMEGAL = 0;

Assumptions	The sun gear is the first gear in the train and the ring gear is the last.  Let the arm have an ω of 1 rpm.  The train ratio desired is sun/arm.

Solution	See M-Book Ex11-03.doc and M-File Ex11_03.m.

1	Equation 11.12 defines the kinematics of an epicyclic train:

−N2N3⋅N3N4=ωL−ωarmωF−ωarm

Solving for ωF

OMEGAF = (N2*OMEGAarm - N4*OMEGAL + N4*OMEGAarm) / N2

OMEGAF =

3

which defines the train ratio as +3.  The sun gear rotates three times as fast and in the same direction as the arm.  Note the signs on the gearset ratios.  One is an external set (-) and one an internal set (+).