EXAMPLE 13-7

Design of a Belleville Spring for Static Loading

Problem	Design a Belleville spring to give a constant load over a given deflection.

Given	An end seal on a shaft requires a nearly constant load over small motions associated with temperature change.  The spring must apply a nominal force of 10 lb ±5% over a range of ±0.006 in at any convenient nominal deflection.  The spring must fit in a 1.25-in- diameter hole.

Nominal force

Fflat = 10; % lbf

Assumptions	Assume a diameter ratio Rd = 2.  Use unset carbon spring steel 50HRC.

Rd = 2;

Properties

E = 30e6;   % psi

v = 0.3;

Solution	See M-Book Ex13-07.doc and M-File Ex13_07.m.

1	Assume an outside diameter to allow some clearance in the 1.25-in-diameter hole.

Outside diameter

Do = 1.2;   % in

2	Since a constant force spring is needed, the h/t ratio (see Figure 13-30) is

h/t ratio

hovert = 1.414;

3	The required force variation of not more than ±5% can be met by choosing an appropriate deflection range to operate in from Figure 13-31.  If the deflection is kept between about 65% and 135% of the flat deflection, this tolerance will be achieved.  The nominal force will then occur at the flat position and the spring must operate on both sides, so must be mounted in similar fashion to that shown in Figure 13-33.

4	Use the above assumptions and the specified nominal force in equation 13.37a to find an appropriate spring thickness t:

t = ((Fflat/19.2e7)*(Do^2/hovert))^0.25    % in

t =

0.0152

5	The height h can now be found:

h = hovert * t   % in

h =

0.0215

6	Based on the choices in step 3, find the minimum and maximum deflections:

ymin = 0.65 * h  % in

ymax = 1.35 * h  % in

ymin =

0.0139

ymax =

0.0290

The difference between these distances is greater than the required deflection range, so the force tolerance can be met over that range.

7	Figure 13-34 shows that the worst stress state will occur at the largest deflection ymax, so solve equations 13.36 for stresses at that deflection:

K1 = (6/(pi * log(Rd)))*((Rd - 1)^2 / Rd^2)

K2 = (6/(pi * log(Rd)))*(((Rd - 1) / log(Rd)) - 1)

K3 = (6/(pi * log(Rd)))*((Rd - 1) / 2)

K4 = ((Rd*log(Rd) - (Rd - 1)) / log(Rd))*(Rd / (Rd - 1)^2)

K5 = Rd / (2 * (Rd - 1))

K1 =

0.6888

K2 =

1.2198

K3 =

1.3777

K4 =

1.1146

K5 =

1

SIGc = -((4*E*ymax) / (K1 * Do^2 * (1-v^2)))*...

(K2 * (h - ymax/2) + K3*t)   % psi

SIGti = ((4*E*ymax) / (K1 * Do^2 * (1-v^2)))*...

(-K2 * (h - ymax/2) + K3*t)  % psi

SIGto = ((4*E*ymax) / (K1 * Do^2 * (1-v^2)))*...

(K4 * (h - ymax/2) + K5*t)   % psi

SIGc =

-1.1328e+005

SIGti =

4.7757e+004

SIGto =

8.8382e+004

8	Table 13-5 gives Sut = 246 ksi for this material.  Table 13-15 indicates that 120% of this value can be used for an unset spring.  The safety factor for static loading is then

Sut = 246*1000  % psi

Ns = 1.20 * Sut / abs(SIGc)

Sut =

246000

Ns =

2.6059

which is acceptable.

9	A summary of the spring design is

Outside diameter

Do              % in

Do =

1.2000

Inside diameter

Di = Do / Rd    % in

Di =

0.6000

Thickness

t               % in

t =

0.0152

Height

h               % in

h =

0.0215