EXAMPLE 4-3

Determining 3-D Stresses Using Analytical Methods

Problem	A triaxial stress element as shown in Figure 4-1 has the stress components given below.  Find the principal stresses using a numerical method and draw the resulting Mohr's circles.

All stresses are in units of ksi (103 psi).

Given	Normal stresses

SIGx = 40;   % ksi (ksi = 1000 x psi)

SIGy = -20;   % ksi

SIGz = -10;   % ksi

Shear stress

TAUxy = 5;      % ksi

TAUyz = -1.5;   % ksi

TAUzx = 2.5;   % ksi

Solution	See Figures 4-2 and 4-7, M-Book Ex04-03.doc and M-File Ex04_03.m

1	Calculate the tensor invariants C0, C1, and C2 from equation 4.4c.

C2 = SIGx + SIGy + SIGz

C2 =

10

C1 = det([SIGx TAUxy; TAUxy SIGy]) +...

det([SIGx TAUzx; TAUzx SIGz]) +...

det([SIGy TAUyz; TAUyz SIGz])

C1 =

-1.0335e+003

C0 = det([SIGx TAUxy TAUzx; TAUxy SIGy TAUyz; TAUzx TAUyz SIGz])

C0 =

8.2475e+003

2	Substitute the invariants into equation 4.4c and solve for its three roots using the Mathcad polyroots function.  Define a vector C that consists of the coefficients of the polynomial, taking C1 as positive, starting with the coefficient on the term with the highest power.  Thus,

C = [1 -C2 C1 -C0];

r = roots(C)

r =

40.5249

-20.6869

-9.8379

The principal stresses are contained in the vector r that is returned by the polyroots function.  Setting the largest to σ1 and the smallest to σ3, we have

SIG1 = r(1) % ksi

SIG2 = r(3) % ksi

SIG3 = r(2) % ksi

SIG1 =

40.5249

SIG2 =

-9.8379

SIG3 =

-20.6869

3	The principal shear stresses can now be found from equation 4.5.

TAU13 = (SIG1 - SIG3) / 2  % ksi

TAU12 = (SIG1 - SIG2) / 2  % ksi

TAU23 = (SIG2 - SIG3) / 2  % ksi

TAU13 =

30.6059

TAU12 =

25.1814

TAU23 =

5.4245

Note that units were not used in this example because the polyroot function requires that the elements of the coefficient vector (C is this case) all have the same units.  In equation 4.4c the units of C2 are [stress], C1 are [stress]2, C0 are [stress]3, and the coefficient of σ3 is dimensionless.