EXAMPLE 9-4

Designing Shaft Keys

Problem	Design the keys for the shaft in Examples 9-2 and 9-3 and refine the estimate of the shaft's safety factors based on the preliminary design dimensions from that earlier example in conjunction with refined stress concentration factors.

Given	The loading is the same as in Example 9-2.  See Figure 9-3 for labels.

Mean torque

Tm = 73.07;  % lbf in

Alternating torque

Ta = 73.07;  % lbf in

Mean and alternating moments:

MBm = 32.82; % lbf in

MCm = 63.94; % lbf in

MDm = 9.13;  % lbf in

MBa = 32.82; % lbf in

MCa = 63.94; % lbf in

MDa = 9.13;  % lbf in

Shaft diameters:

d0 = 0.875;  % in

d1 = 0.750;  % in

d2 = 0.669;  % in

d3 = 0.531;  % in

Assumptions	Use square, parallel keys with end-milled keyseats.  The shaft material is the same as in Example 9-3.  A lower carbon steel, SAE 1010, will be used for the keys.  See Figure 9-16 for stress-concentration factors.  Key material properties are:

Tensile strength

Sut = 53;    % ksi

Yield strength

Sy = 44;     % ksi

Endurance strength

Se = 22.9;   % ksi

Solution	See M-Book Ex09-04.doc and M-File Ex09_04.m.

1	There are two locations with keys on this shaft, at points B and D.  The design diameters chosen in Example 9-3 at these points were d1 and d3, respectively.  Table 9-2 shows the standard key widths for these diamters as:

For

d1

w1 = 0.1875; % in

d1 =

0.7500

d3

w3 = 0.125;  % in

d3 =

0.5310

The key length can be adjusted for each location.

2	At point B, the mean and alternating components of force on the key are found from the torque component divided by the shaft radius at that point.

Fa = Ta / (0.5 * d1)    % lbf

Fm = Tm / (0.5 * d1)    % lbf

Fa =

194.8533

Fm =

194.8533

3	Assume a key length of 0.5 in and calculate the alternating and mean shear stress components from

Key length

L1 = 0.50;   % in

Shear area

Ashear = w1 * L1    % in2

Ashear =

0.0938

Shear stress

TAUa = Fa / Ashear  % psi

TAUm = Fm / Ashear  % psi

TAUa =

2.0784e+003

TAUm =

2.0784e+003

4	To find a safety factor for shear fatigue of the key, compute the von Mises equivalent stresses for each of these components from equation 5.7d,

von Mises stress components

SIGprma = sqrt(3) * TAUa    % psi

SIGprmm = sqrt(3) * TAUm    % psi

SIGprma =

3.6000e+003

SIGprmm =

3.6000e+003

and use them in equation 6.18e to determine the fatigue safety factor:

Nf = (Se*Sut) / (Sut * SIGprma + Se * SIGprmm) * 1000

Nf =

4.4419

5	The bearing stress on the key is compression and thus can be considered a static load.  It is calculated using the maximum force on the key.

Bearing area

Abearing = (1/2)*w1*L1  % in2

Abearing =

0.0469

Bearing stress

SIGmax = (Fm + Fa) / Abearing  % psi

SIGmax =

8.3137e+003

6	Calculate the safety factor for bearing failure from:

Ns = Sy / SIGmax * 1000

Ns =

5.2924

7	At point D, the force on the key is

Fa = Ta / (0.5*d3)  % lbf

Fm = Tm / (0.5*d3)  % lbf

Fa =

275.2166

Fm =

275.2166

8	Assume a key length of 0.5 in and calculate the alternating and mean shear stress components from

Key length

L3 = 0.50;   % in

Shear area

Ashear = w3 * L3    % in2

Ashear =

0.0625

Shear stress

TAUa = Fa / Ashear  % psi

TAUm = Fm / Ashear  % psi

TAUa =

4.4035e+003

TAUm =

4.4035e+003

9	Compute the von Mises equivalent stresses for each of these components from equation 5.7d,

von Mises stress components

SIGprma = sqrt(3) * TAUa    % psi

SIGprmm = sqrt(3) * TAUm    % psi

SIGprma =

7.6270e+003

SIGprmm =

7.6270e+003

and use them in equation 6.18e:

Nf = (Se*Sut) / (Sut * SIGprma + Se * SIGprmm) * 1000

Nf =

2.0966

10	The bearing stress on the key is calculated using the maximum force on the key.

Bearing area

Abearing = (1/2)*w3*L3  % in2

Abearing =

0.0313

Bearing stress

SIGmax = (Fm + Fa) / Abearing   % psi

SIGmax =

1.7614e+004

11	Calculate the safety factor for bearing failure from:

Ns = Sy / SIGmax * 1000

Ns =

2.4980

12	The safety factors for the shaft at these locations can now be recalculated using a stress-concentration factor at the keyseats that takes into account the actual shaft diameter.  Our previous design calculation in Example 9-2 used a worst-case assumption for these values.  Figure 9-16 shows the stress concentration functions for end-milled keyseats in both bending and torsion.  To use this graph we must calculate the r/d ratio of the end-mill radius versus the shaft diameter.  Assume a radius on the end-mill of 0.010 in.  Then

End-mill radius

r = 0.010;  % in

for point B

rd1Ratio = r / d1

rd1Ratio =

0.0133

for point D

rd3Ratio = r / d3

rd3Ratio =

0.0188

The corresponding stress-concentration factors are read from Figure 9-16 as

for point B

KtB = 2.5;

KtsB = 2.9;

for point D

KtD = 2.2;

KtsD = 2.7;

13	These are used in equations m, n, and o of Example 9-1 to obtain the fatigue stress-concentration factors, which are for material notch sensitivities of

Notch sensitivity

q = 0.5;

qs = 0.57;

for point B

KfB = 1 + q*(KtB - 1)

KfsB = 1 + qs*(KtsB - 1)

KfB =

1.7500

KfsB =

2.0830

for point D

KfD = 1 + q*(KtD - 1)

KfsD = 1 + qs*(KtsD - 1)

KfD =

1.6000

KfsD =

1.9690

for both points

KfmB = KfB

KfmD = KfD

KfsmB = KfsB

KfsmD = KfsD

KfmB =

1.7500

KfmD =

1.6000

KfsmB =

2.0830

KfsmD =

1.9690

14	The new safety factors are then calculated using equation 9.8 with the data from equations b and c from Example 9-2 with the design values for shaft diameter and the above stress-concentration values inserted:

Shaft material properties

Tensile strength

Sut = 65;    % ksi

Yield strength

Sy = 38;     % ksi

Endurance strength

Se = 26.4;   % ksi

for point B

Nf = (pi*d1^3*Se*Sut)/(32*(Sut*sqrt((KfB*MBa)^2 +...

(3/4)*(KfsB*Ta)^2)+Se*sqrt((KfmB*MBm)^2 +...

(3/4)*(KfsmB*Tm)^2))) * 1000

Nf =

5.4081

for point D

Endurance strength

Se = 26.9;   % ksi

Nf = (pi*d3^3*Se*Sut)/(32*(Sut*sqrt((KfD*MDa)^2 +...

(3/4)*(KfsD*Ta)^2)+Se*sqrt((KfmD*MDm)^2 +...

(3/4)*(KfsmD*Tm)^2))) * 1000

Nf =

2.2292

At point B the safety factor is greater than the specified value of 2.5.  At point D it is lower.  Increasing the diameter at D to 0.562 in gives a safety factor of 2.7.  Then, the safety factors for key failure (4.4 at B and 2.1 at D) are lower than those for shaft failure, which is desireable since the keys will then fail before the shaft in an overload situation.  This is now a viable and acceptable design.