EXAMPLE 9-2

Shaft Design for Repeated Torsion with Repeated Bending

Problem	Design a shaft to support the attachments shown in Figure 9-5 with a minimum design safety factor of 2.5.

Given	Mean torque

Tm = 73.07;  % lbf in

Alternating torque

Ta = 73.07;  % lbf in

Mean and alternating moments:

MBm = 32.82; % lbf in

MCm = 63.94; % lbf in

MDm = 9.13;  % lbf in

MBa = 32.82; % lbf in

MCa = 63.94; % lbf in

MDa = 9.13;  % lbf in

Design factor of safety

Nf = 2.5;

Assumptions	There are no applied axial loads.  Steel will be used for infinite life.  Assume a stress concentration factor of 3.5 for the step radii in bending, 2 for step radii in torsion, and 4 at the keyways.  Since the torque is steady and the bending moment fully reversed, the ASME method of equation 9.6 should not be used.

Solution	See Figures 9-9, 9-10, Table 9-1, M-Book Ex09-02.doc and M-File Ex09_02.m.

1	For comparison purposes, we will keep all factors except the loading configuration the same as in the previous example.  The same, low-carbon, cold-rolled steel SAE 1020, which has the properties given below is used.

Tensile strength

Sut = 65;    % ksi

Sut = 65*1000    % psi

Sut =

65000

Yield strength

Sy = 38;     % ksi

Sy = 38*1000     % psi

Sy =

38000

Unmodified endurance limit

Sprme = 0.5 * Sut   % psi

Sprme =

32500

Endurance limit correction factors:

Load

Cload = 1;

Size

Csize = @(d) 0.869*d^-0.097;     % assumes d [=] inches

Surface (machined)

Csurf = 2.7*(Sut/1000)^-0.265;     % assumes Sut [=] ksi

Temperature

Ctemp = 1;

Reliability (50%)

Creliab = 1;

Modified endurance limit

Se = @(d) Cload * Csize(d) * Csurf * Ctemp * Creliab * Sprme;  % psi

Notch sensitivity

Bending

r = 0.01;   % in

a = 0.10^2  % in

q = 1 / (1 + sqrt(a/r))

a =

0.0100

q =

0.5000

Torsion

r = 0.01;   % in

a = 0.075^2  % in

qs = 1 / (1 + sqrt(a/r))

a =

0.0056

qs =

0.5714

Geometric stress-concentration factors

Bending at a step

Ktb = 3.5;

Torsion at a step

Kts = 2.0;

Keyway

Ktk = 4.0;

2	There are three points of interest, labeled B, C, and D in Figure 9-5.  The fatigue stress-concentration factors are assumed to be the same at C and D and are larger at B.

At C, with reversed bending and steady torsion, the fatigue stress-concentration factors are:

Kfb = 1 + q * (Ktb - 1)

Kfs = 1 + qs * (Kts - 1)

Kfb =

2.2500

Kfs =

1.5714

From equation 6.17 we find in this case, the same factor should be used on the mean torsional stress component, thus:

Kfsm = Kfs

Kfbm = Kfb

Kfsm =

1.5714

Kfbm =

2.2500

3	The required shaft diameter at point C can now be found from equation 9.8 using Matlab's fzero function.  To start the iteration used by fzero, we provide an intial guess a value for d of 0.5 in as the final argument in the fzero function call.

FUNC = @(d) d-(((32*Nf)/pi)*...

(((sqrt((Kfb*MCa)^2 + (3/4)*(Kfs*Ta)^2))/Se(d))+...

((sqrt((Kfbm*MCm)^2 + (3/4)*(Kfsm*Tm)^2))/Sut)))^(1/3);

The result is

d = fzero(FUNC,0.5)

d =

0.6189

For this value of d

Csize(d)    % no units

Se(d)       % psi

ans =

0.9104

ans =

2.6428e+004

Compare this to the value of 0.534 in from the same equation in the prior example where the loads were steady.

4	At point B the stress-concentration factors are

Kfb = 1 + q * (Ktk - 1)

Kfs = 1 + qs * (Ktk - 1)

Kfsm = Kfs

Kfbm = Kfb

Kfb =

2.5000

Kfs =

2.7143

Kfsm =

2.7143

Kfbm =

2.5000

and the required diameter from equation 9.8 is (using the value of d above as the guess value):

FUNC = @(d) d-(((32*Nf)/pi)*...

(((sqrt((Kfb*MBa)^2 + (3/4)*(Kfs*Ta)^2))/Se(d))+...

((sqrt((Kfbm*MBm)^2 + (3/4)*(Kfsm*Tm)^2))/Sut)))^(1/3);

The result is

d = fzero(FUNC,0.5)

d =

0.6370

For this value of d

Csize(d)    % no units

Se(d)       % psi

ans =

0.9079

ans =

2.6354e+004

Compare this to the value of 0.518 in from the same equation in the prior example where the loads were steady.

5	At point D the stress-concentration factors are

Kfb = 1 + q * (Ktb - 1)

Kfs = 1 + qs * (Kts - 1)

Kfsm = Kfs

Kfbm = Kfb

Kfb =

2.2500

Kfs =

1.5714

Kfsm =

1.5714

Kfbm =

2.2500

and the required diameter from equation 9.8 is (using the value of d above as the guess value):

FUNC = @(d) d-(((32*Nf)/pi)*...

(((sqrt((Kfb*MDa)^2 + (3/4)*(Kfs*Ta)^2))/Se(d))+...

((sqrt((Kfbm*MDm)^2 + (3/4)*(Kfsm*Tm)^2))/Sut)))^(1/3);

The result is

d = fzero(FUNC,0.5)

d =

0.5141

For this value of d

Csize(d)    % no units

Se(d)       % psi

ans =

0.9269

ans =

2.6907e+004

Compare this to the value of 0.411 in from the same equation in the prior example where the loads were steady.

6	The presence of repeated stresses requires a larger shaft to maintain the same safety factor.  We need the next larger bearing at C, which is 17 mm (0.669 in) inside diameter.  Selecting this value for d2, we try values of d3 = 0.531 in, which is a smaller standard inch size, and d1 = 0.750 in, which is a larger standard inch size.  The stock size is now 0.875 in, and left as-rolled for the outside diameter at the gear flange.  These dimensions will give safety factors that meet or exceed the specification as shown in the modified-Goodman diagrams of Figure 9-10.  The stresses and safety factors should now be recalculated using more accurate strength-reduction and stress-concentration factors based on final dimensions.

7	Table 9-1 compares the results from Examples 9-1 and 9-2 to show the differences in shaft diameter needed for steady loading or fluctuating loading.  Note that the peak load in Example 9-1 is half that of Example 9-2.  The final safety factors are larger than the design minimum due to the need to size-up the shaft to fit available stock bearing sizes.