EXAMPLE 4-10

Column Design for Concentric Loading

Problem	A beachfront house is to be jacked up 10 ft above grade and placed on a set of steel columns.  The weight to be supported by each column is estimted to be 200000 lb.  Two designs are to be considered, one using square steel tubes and the other using round steel tubes.

Given	Column length

l = 10;      % ft

l = 120;     % in

Column load

Load = 200000;  % lbf

Safety factor

FS = 4;

Wall thickness

t = 0.5;     % in

Compressive yield strength

Syc = 60000; % psi

Modulus of elasticity

E = 30e6;    % psi

Assumptions	The loading is concentric and the columns are vertical.  Their bases are set in concrete and their tops are free, creating a fixed-free end-constraint condition.  Use AISC recommended end-condition factors.

Solution	See M-Book Ex04-10.doc and M-File Ex04_10.m

1	Start by calculating the slenderness ratio that divides the unit load vs slenderness ratio graph into Johnson and Euler regions using equation 4.42..

SrD = pi * sqrt((2*E) / Syc)    % no units

SrD =

99.3459

2	Using Table 4-7, calculate the effective column length.

leff = 2.1 * l      % in

leff =

252

3	To start the iterative process, assume that the final design will be an Euler column with the critical load equal to FS*Load.  From equation 4.38b,

Pcr=π2⋅E⋅A⋅k2L2		and	k2=IA

Substituting for k2

Pcr=π2⋅E⋅IL2=FS⋅Load

Solving for I

I = (leff^2 * FS * Load) / (pi^2 * E)   % in4

I =

171.5813

This is the required moment of inertia (in4) for square or round tubing, assuming an Euler column.

4	Using the relationships given in Appendix A, solve for the outside diameter of the tube.

CIRCULAR CROSS-SECTION

D0 = 8  % in (guess value for iterative solver)

D = fzero(@(D)(pi/64)*(D^4 - (D - 2*t)^4) - I,D0)   % in

D0 =

8

D =

10.0518

5	Using this diameter, calculate the slenderness ratio and compare to SrD. If it is greater than SrD the assumption of an Euler column is correct, if not, recalculate using the Johnson equation.

Inside diameter

d = D - 2*t     % in

d =

9.0518

Area

Ar = (pi/4) * (D^2 - d^2)   % in2

Ar =

15.0039

Radius of gyration

kr = sqrt(I / Ar)       % in

kr =

3.3817

Slenderness ratio

Sr = leff / kr      % no units

Sr =

74.5191

This value is less than

SrD

SrD =

99.3459

so we need to go back to step 3 and assume a Johnson column.

6	Assuming a Johnson column and solving for the outside diameter.

Pcr=A⋅Syc−1E⋅Syc⋅Sr2⋅π2=FS⋅Load

D0 = 8  % in (guess value for iterative solver)

D = fzero(@(D)(Syc-(1/E)*((leff*Syc)/(2*pi))^2*...

((pi/4)*(D^2-(D-2*t)^2))/((pi/64)*(D^4-(D-2*t)^4)) -...

(FS*Load) / ((pi/4)*(D^2-(D-2*t)^2))),D0)   % in

D0 =

8

D =

11.3544

7	Using this diameter, calculate the slenderness ratio and compare to SrD. If it is less than SrD the assumption of a Johnson column is correct.

Inside diameter

d = D - 2*t     % in

d =

10.3544

Area

Ar = (pi/4) * (D^2 - d^2)       % in2

Ar =

17.0500

Radius of gyration

kr = sqrt(((pi/64)*(D^4-(D-2*t)^4)) / Ar)       % in

kr =

3.8417

Slenderness ratio

Sr = leff / kr      % no units

Sr =

65.5964

8	Repeat steps 6 and 7 for the square cross-section using the Johnson equation.

SQUARE CROSS-SECTION

s0 = 8  % in (guess value for iterative solver)

so = fzero(@(so)(Syc-(1/E)*((leff*Syc)/(2*pi))^2*...

(so^2-(so-2*t)^2)/((1/12)*(so^4-(so-2*t)^4)) -...

(FS*Load) / (so^2-(so-2*t)^2)),s0)   % in

s0 =

8

so =

9.3426

9	Using this dimension, calculate the slenderness ratio and compare to SrD. If it is less than SrD the assumption of a Johnson column is correct.

Inside dimension

si = so - 2*t       % in

si =

8.3426

Area

As = D^2 - d^2      % in2

As =

21.7088

Radius of gyration

ks = sqrt(((1/12)*(so^4-(so-2*t)^4))/As)    % in

ks =

3.2635

Slenderness ratio

Sr = leff / ks      % no units

Sr =

77.2169

The value of s calculated in step 8 is the minimum value required to assure that, with a safety factor of 4, the column will not buckle under the applied load of 200 kip.

10	Check the results.

CIRCULAR CROSS-SECTION

Outside diameter

D   % in

D =

11.3544

Inside diameter

d   % in

d =

10.3544

Moment of inertia

I = (pi/64)*(D^4 - d^4)     % in4

I =

251.6317

Cross-section area

A = (pi/4)*(D^2 - d^2)     % in2

A =

17.0500

Radius of gyration

k = sqrt(I/A)   % in

k =

3.8417

Slenderness ratio

Sr = leff / k   % no units

Sr =

65.5964

Critical load

Pcr = A * (Syc - (1/E)*((Syc*Sr)/(2*pi))^2) / 1000     % kip

Pcr =

800

FS * Load / 1000    % kip

ans =

800

SQUARE CROSS-SECTION

Outside dimension

so  % in

so =

9.3426

Inside dimension

si  % in

si =

8.3426

Moment of inertia

I = (1/12)*(so^4 - si^4)    % in4

I =

231.2125

Cross-section area

A = so^2 - si^2             % in2

A =

17.6853

Radius of gyration

k = sqrt(I/A)               % in

k =

3.6158

Slenderness ratio

Sr = leff/k                 % no units

Sr =

69.6949

Critical load

Pcr = A * (Syc - (1/E)*((Syc*Sr)/(2*pi))^2) / 1000     % kip

Pcr =

800.0000

FS * Load / 1000    % kip

ans =

800

Both the square and the circular designs check.