EXAMPLE 5-2

Failure of Brittle Materials Under Static Loading

Problem	Determine the safety factors for the bracket rod shown in Figure 5-7 based on the Modified Mohr theory.

Given	Material is class 50 gray cast iron

Ultimate strength

Tensile

Sut =  52500;           % psi

Compressive

Suc = 164000;           % psi

Rod length

L = 6;                  % in

Arm length

a = 8;                  % in

Rod diameter

d = 1.5;                % in

Load

F = 1000;               % lbf

Assumptions	The load is static and the assembly is at room temperature.  Consider shear due to transverse loading as well as other stresses.

Solution	See Figures 5-7 and 4-33, M-Book Ex05-2.doc and M-File Ex05_02.m.  Also see Examples 4-9 and 5-1.

1	The rod of Figure 5-7 is loaded in both bending (as a cantilever beam) and in torsion. The largest tensile bending stress will be in the top outer fiber at point A.  The largest torsional shear stress will be all around the outer circumference of the rod.  First take a differential element at point A where both of these stresses combine.  Find the normal bending stress and torsional shear stress on point A using equations 4.11b and 4.24b, respectively.

Bending moment at point A

M = F*L                 % lbf in

M =

6000

Distance to outer fiber

c = 0.5*d               % in

c =

0.7500

Moment of inertia

I = (pi*d^4)/64         % in4

I =

0.2485

Torque carried by rod

T = F*a                 % lbf in

T =

8000

Polar moment of inertia

J = 2*I                 % in4

J =

0.4970

Normal bending stress

SIGx = (M*c)/I          % psi

SIGx =

1.8108e+004

Torsional shear stress

TAUxz = (T*c)/J         % psi

TAUxz =

1.2072e+004

2	Find the maximum shear stress and principal stresses that result from this combination of applied stresses at point A using equations 4.6.

Normal stress in z-direction

SIGz = 0    % psi

SIGz =

0

Maximum shear stress

TAUmax = sqrt(((SIGx - SIGz)/2)^2 + TAUxz^2)    % psi

TAUmax =

1.5090e+004

Principal stresses

SIG1 = (SIGx + SIGz) / 2 + TAUmax     % psi

SIG2 = 0                              % psi

SIG3 = (SIGx + SIGz) / 2 - TAUmax     % psi

SIG1 =

2.4144e+004

SIG2 =

0

SIG3 =

-6.0361e+003

Note that these stresses are identical to those of Example 5-1.

3	The principal stresses for point A can now be plotted on a modified-Mohr diagram as shown in Figure 5-12a.  This shows that the load line crosses the failure envelope above the Sut, -Sut point, making equation 5.12a appropriate for the safety factor calculation.

N = Sut / SIG1      % no units

N =

2.1744

4	An alternative approach that does not require drawing the modified-Mohr diagram is to find the Dowling factors C1, C2, C3 using equations 5.12b:

C1 = (1/2) * (abs(SIG1-SIG2) +...

(-Suc + 2*Sut)/(-Suc) * (SIG1 + SIG2))   % psi

C2 = (1/2) * (abs(SIG2-SIG3) +...

(-Suc + 2*Sut)/(-Suc) * (SIG2 + SIG3))   % psi

C3 = (1/2) * (abs(SIG3-SIG1) +...

(-Suc + 2*Sut)/(-Suc) * (SIG3 + SIG1))   % psi

C1 =

1.6415e+004

C2 =

1.9323e+003

C3 =

1.8348e+004

5	Then find the largest of the six stresses C1, C2, C3, σ1, σ2, σ3:

SIGeff = max([C1 C2 C3 SIG1 SIG2 SIG3])   % psi

SIGeff =

2.4144e+004

which is the modified-Mohr effective stress.

6	The safety factor for point A can now be found using equation 5.12d.

N = Sut / SIGeff    % no units

N =

2.1744

7	Since the rod is a short beam, we need to check the shear due to transverse loading at point B on the neutral axis.  The maximum transverse shear stress at the neutral axis of a round rod was given in equation 4.15c.

Shear load at B

V = F    % lbf

V =

1000

Cross-section area

A = (pi/4) * d^2    % in2

A =

1.7671

Shear stress at the neutral axis

TAUbending = (4*V) / (3*A)      % psi

TAUbending =

754.5123

Point B is in pure shear.  The total shear stress at point B is the algebraic sum of the transverse shear stress and the torsional shear stress, which both act on the same planes of the differential element in this case, in the same direction as shown in Figure 4-33b.

TAUmax = TAUxz + TAUbending     % psi

TAUmax =

1.2827e+004

8	Find the principal stresses for this pure shear loading:

SIG1 = TAUmax         % psi

SIG2 = 0              % psi

SIG3 = -TAUmax        % psi

SIG1 =

1.2827e+004

SIG2 =

0

SIG3 =

-1.2827e+004

9	These principal stresses for point B can now be plotted on a modified-Mohr diagram as shown in Figure 5-12b.  Because this is pure shear loading, the load line crosses the failure envelope at the Sut, -Sut point, making equation 5.12a appropriate for the safety factor calculation.

N = Sut / SIG1    % no units

N =

4.0930

10	To avoid drawing the modified-Mohr diagram, find the Dowling factors C1, C2, C3 using equations 5.12b:

C1 = (1/2) * (abs(SIG1-SIG2) +...

(-Suc + 2*Sut)/(-Suc) * (SIG1 + SIG2))   % psi

C2 = (1/2) * (abs(SIG2-SIG3) +...

(-Suc + 2*Sut)/(-Suc) * (SIG2 + SIG3))   % psi

C3 = (1/2) * (abs(SIG3-SIG1) +...

(-Suc + 2*Sut)/(-Suc) * (SIG3 + SIG1))   % psi

C1 =

8.7206e+003

C2 =

4.1061e+003

C3 =

1.2827e+004

11	And find the largest of the six stresses C1, C2, C3, σ1, σ2, σ3:

SIGeff = max([C1 C2 C3 SIG1 SIG2 SIG3])   % psi

SIGeff =

1.2827e+004

which is the modified-Mohr effective stress.

12	The safety factor for point B can now be found using equation 5.12d.

N = Sut / SIGeff   % no units

N =

4.0930

which is the same as was found in step 9.