EXAMPLE 3-2B

Shear and Moment Diagrams of a Simply Supported Beam Using Singularity Functions

Problem	Determine and plot the shear and moment functions for the simply supported beam with uniformly distributed load shown in Figure 3-22a.

Given	Beam length

l = 10;  % in

Distance to load w

a = 4;   %in

Distributed load

w = 10;  % lbf / in

Assumptions	The weight of the beam is negligible compared to the applied load and so can be ignored.

Solution	See Figures 3-22a and 3-24, M-Book Ex03-02.doc and M-file Ex03_02.m

1	Write an equation for the load function in terms of equations 3.17 and integrate the resulting function twice using equations 3.18 to obtain the shear and moment functions.  For the beam in Figure 3-22a,

q(x) = R1<x - 0>-1 - w<x - a>0 + R2<x - l>-1	(a)

V(x) = R1<x - 0>0 - w<x - a>1 + R2<x - l>0  + C1	(b)

M(x) = R1<x - 0>1 – (w/2)<x - a>2 + R2<x - L>1 + C1 x + C2	(c)

There are two reaction forces and two constants of integration to be found.  We are integrating along a hypothetical infinite beam from -∞ to x.  The variable x can take on values both before and beyond the ends of the beam.  If we consider the conditions at a point infinitesimally to the left of x = 0 (denoted as x = 0-), the shear and moment will both be zero there.  The same conditions apply at a point infinitesimally to the right of x = l (denoted as x = l+).  These observations provide the four boundary conditions needed to evaluate the four constants C1, C2, R1, R2: when x = 0-, V = 0, M = 0; when x = l+, V = 0, M = 0.

2	The constants C1 and C2 are found by substituting the boundary conditions x = 0-, V = 0, and x = 0-, M = 0 in equations (b) and (c), respectively.

V(x) = 0 = R1<0- - 0>0 - w<0- - a>1 + R2<0- - l>0  + C1

C1 = 0	(d)

M(x) = 0 = R1<0- - 0>1 – (w/2)<0- - a>2 + R2<0- - L>1 + C2

C2 = 0

Note that in general, the constants C1 and C2 will always be zero if the reaction forces and moments acting on the beam are included in the loading function, because the shear and moment diagrams must close to zero at each end of the beam.

3	The reaction forces R1 and R2 can be calculated from equations (c) and (b) respectively by substituting the boundary conditions x = l+, V = 0, M = 0.  Note that we can substitute l for  l+ since their difference is vanishingly small.

M(l) = R1<l - 0>1 – (w/2)<l - a>2 + R2<l - l>1 = 0	(e)

M = R1 l – (w/2) (l-a)2 = 0

R1 = (w * (l - a)^2) / (2 * l)  % lbf

R1 =

18

V(l) = R1<l - 0>0 – w<l - a>1 + R2<l - l>0  = 0	(f)

V = R1 – w(l – a) + R2 = 0

R2 = w * (l - a) - R1   % lbf

R2 =

42

Since w, l, and a are known from the given data, equation (e) can be solved for R1, and this result substituted into equation (f) to find R2.  Note that equation (f) is just ΣF = 0, and equation (e) is the sum of moments taken about point l and set to 0.

4	To generate the shear and moment functions over the length of the beam, equations (b) and (c) must be evaluated for a range of values of x from 0 to l, after substituting the above values of C1, C2, R1, and R2 in them.  For a Matlab solution, a step function can be created simply by incorporating a logical evaluation such as (x >= z) in the equation. This evaluates to zero when x is less than the dummy variable z, and a value of one when it is greater than or equal to z.  It will have the same effect as the singularity function.

Range of x

x = 0:0.01*l:l;  % in

Write the shear and moment equations in Matlab form, including a logical evaluation of the form (x >= z) as a multiplying factor to get the effect of the singularity functions.

V = R1*(x>=0).*(x-0).^0 - w*(x>=a).*(x-a).^1 + R2*(x>=l).*(x-l).^0;  % lbf

M = R1*(x>=0).*(x-0).^1 - (w/2)*(x>=a).*(x-a).^2 + R2*(x>=l).*(x-l).^1;  % lbf in

Plot the shear and moment diagrams.

subplot(1,2,1)

plot(x,V)

title('Shear Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Shear, V (lbf)')

subplot(1,2,2)

plot(x,M)

title('Moment Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Moment, M (lbf in)')

FIGURE 3-24. Example 3-2 Graphs

5	The largest absolute values of the shear and moment functions are of interest for the calculation of stresses in the beam.  The graphs show that the shear force is largest at x = l and the moment has a maximum Mmax near the center of the beam.  The value of x at Mmax can be found by setting V to 0 in equation (b) and solving for x.  (The shear function is the derivative of the moment function and so must be zero at each of the moment function's minima and maxima.)  Let the distance along the beam from the origin to the point where V = 0 be c,

V(c) = R1<c - 0>0 - w<c - a>1 + R2<c - l>0 = 0	(g)

Solving for c

R1 – w (c – a) = 0

c = (R1 + w * a) / w    % in

c =

5.8000

For the maximum absolute value of the shear force at x = l-,

V(l-) = R1<l- - 0>0 - w<l- - a>1 + R2<l- - l>0  = Vmax	(h)

Vmax = R1 – w (l – a) + 0

Vmax = R1 - w * (l - a)     % lbf

Vmax =

-42

Note that the first singularity term evaluates to 1 since l- > 0 (see Eq. 3.17c), the second singularity term evaluates to (l - a) because l- > a in this problem (see Eq. 3.17b), and the third singularity term evaluates to 0 as defined in equation 3.17c.  The maximum moment is found in similar fashion:

M(c) = R1<c - 0>1 – (w/2)<c - a>2 + R2<c - l>1 = Mmax	(i)

Mmax = R1 * c - (w/2)*(c - a)^2 % lbf in

Mmax =

88.2000