EXAMPLE 6-4A

Design of a Cantilever Bracket for Fully Reversed Bending:  An Alternate Approach

Problem	A feed-roll assembly is to be mounted at each end on support brackets cantilevered from the machine frame as shown in Figure 6-41.  The feed rolls experience a fully reversed load of 1000 lb amplitude, split equally between the two support brackets.  Design a cantilever bracket to support a fully reversed bending load of 500 lb amplitude for 109 cycles with no failure.  Its dynamic deflection cannot exceed 0.01 in.

Given	Load amplitude

Fa = 500;    % lbf

The operating environment is room air at a maximum temperature of 120 F.  The space available allows a maximum cantilever length of 6 in.  Only ten of these parts is required.

Assumptions:	The bracket can be clamped between essentially rigid plates or bolted at its root.  The normal load will be applied at the effective tip of the cantilever beam from a rod attached through a small hole in the beam.  Since the bending moment is effectively zero at the beam tip, the stress concentration from this hole can be ignored.  Given the small quantity required, machining of stock mill shapes is the preferred manufacturing method.  Use a reliability of 99.9%.

Solution	See Figure 6-41, M-Book Ex06-04A.doc and M-File Ex06_04.m.

1	This is a typical design problem.  Very little data are given except for the required performance of the device, some limitations on size, and the required cycle life.  We will have to make some basic assumptions about part geometry, materials, and other factors as we go.  Some iteration should be expected.

2	The first two steps of the process suggested above, finding the load amplitude and the number of cycles, are defined in the problem statement.  We will begin at the third step, creating a tentative part-geometry design.

3	Figure 6-41a shows a tentative design configuration.  A rectangular cross-section is chosen to provide ease of mounting and clamping.  A piece of cold-rolled bar stock from the mill could simply be cut to length and drilled to provide the needed holes, then clamped into the frame structure.  This approach appears attractive in its simplicity because very little machining is required.  The mill-finish on the sides could be adequate for this application.  This design has some disadvantages however.  The mill tolerances on the thickness are not tight enough to give the required accuracy on thickness, so the top and bottom would have to be machined or ground flat to dimension.  Also, the sharp corners at the frame where it is clamped provide stress concentrations of about Kt = 2 and also create a condition called fretting fatigue due to slight motions that will occur between the two parts as the bracket deflects.  This motion continually breaks down the protective oxide coating, exposing new metal to oxidation and speeding up the fatigue-failure process.  The fretting could be a problem even if the edges of the frame pieces were radiused.

4	Figure 6-41b shows a better design in which the mill stock is purchased thicker than the desired final dimension and machined top and bottom to dimension D, then machined to thickness d over length l.  A fillet radius r is provided at the clamp point to reduce fretting fatigue and achieve a lower Kt.  (See Figure 4-33).  Figure 4-31 shows that with suitable control of the r/d and D/d ratios for a stepped flat bar in bending, the geometric stress-concentration factor Kt can be kept under about 1.5.

5	Some trial dimensions must be assumed for b, d, D, r, a, and l.  We will assume (guess) values of

Distance to load

a = 5.00;   % in

Beam length

l = 6;      % in

This length will leave some material around the hole and still fit within the 6-in-length constraint.  This leaves us to choose values for b, d, D, and r.  Instead of guessing these values, solving for the resulting factor of safety and then guessing new values if the resulting factor of safety is too small, we will, instead, specify the ratios b/d, D/d, and r/d and carry the analysis through with d unknown until we set the factor of safety to the required value and then solve for d.  We will assume values for the following ratios

Beam width to thickness, b/d

boverd = 2;

Base thickness to beam thickness, D/d

Doverd = 1.125;

Fillet radius to beam thickness, r/d

roverd = 0.5;

6	A material must be chosen.  For infinite life, low cost, and ease of fabrication, it is desirable to use a low-carbon alloy steel if possible and if environmental conditions permit.  Since this is used in a controlled, indoor environment, carbon steel is acceptable on the latter point.  The fact that the deflection is of concern is also a good reason to choose a material with a large E.  Low-carbon, ductile steels have the requisite endurance-limit knee for the infinite life required in this case and also have low notch sensitivities.  A low-carbon steel with Sut = 80 ksi is selected for the first trial.

Sut = 80;   % ksi

7	First the reaction force and reaction moment at the support are found using equations h from Example 4-5.  Next the area moment of inertia of the cross-section, the distance to the outer fiber, and the nominal alternating bending stress at the root are found using the alternating load's 500-lb amplitude.

Reaction force

R = Fa  % lbf

R =

500

Reaction moment

M = R*l - Fa*(l-a)  % lbf in

M =

2500

Moment of inertia

I = @(d) (boverd * d^4)/12;  %in4

Distance to neutral axis

c = @(d) 0.5 * d;   % in

Nominal alternating stress

SIGanom = @(d) (M * c(d)) / I(d) / 1000;   % ksi

8	Calculate the stress concentration factor for this geometry using Figure 4-36 or Figure E-10 in Appendix E.  The r/d and D/d ratios are

roverd

roverd =

0.5000

and

Dprime = Doverd    % no units

Dprime =

1.1250

Using cubic interpolation to find the constants A and b (b' in this case since we are already using b as the width of the bar).  From the table and equation in Figure 4-36,

Coeff = [1 1.07 1.07^2 1.07^3; ...

1 1.10 1.10^2 1.10^3; ...

1 1.20 1.20^2 1.20^3; ...

1 1.30 1.30^2 1.30^3];

CArhs = [1.01990; 1.01650; 0.99590; 0.95880];

CA = Coeff\CArhs

CA =

0.9911

-0.4106

0.9309

-0.4877

A = CA(1) + CA(2)*Dprime + CA(3)*Dprime^2 + CA(4)*Dprime^3

A =

1.0127

CBrhs = [-0.20333; -0.21548; -0.23829; -0.27269];

Cb = Coeff\CBrhs

Cb =

13.7466

-35.2542

29.7899

-8.4360

bprime = Cb(1) + Cb(2)*Dprime + Cb(3)*Dprime^2 + Cb(4)*Dprime^3

bprime =

-0.2229

Kt = A * (roverd)^bprime

Kt =

1.1819

9	Calculate the notch sensitivity q of the chosen material based on its ultimate strength and notch radius using equation 6.13 and Table 6-6.

Neuber constant

a = 0.080^2  % in (redefines 'a' temporarily)

a =

0.0064

q = @(d) (1 / (1 + sqrt(a)/sqrt(roverd * d)));

10	The values of q and Kt are used to find the fatigue stress-concentration factor Kf, which in turn is ued to find the local alternating stress σa in the notch.  Because we have the simplest case of a uniaxial tensile stress, the largest alternating principal stress σ1a for this case is equal to the alternating tensile stress, as is the von Mises alternating stress σ'a.  See equations 4.6 and 5.7c.

Kf = @(d) 1 + q(d) * (Kt - 1);

SIGa = @(d) Kf(d) * SIGanom(d);     % ksi

Since this is the only stress component present, it is the principal stress and

SIG1a = @(d) SIGa(d);

SIG2a = 0       % ksi

SIG3a = 0       % ksi

SIG2a =

0

SIG3a =

0

SIGprimeA = @(d) SIG1a(d);

11	Using the assumed value for the ultimate tensile stress above, calculate the corrected endurance limit.  The load factor is found from equation 6.7a.  The size factor for this rectangular part is determined by calculating the cross-sectional area stressed above 95% of its maximum stress (see Figure 6-25) and using that value in equation 6.7d find an equivalent diameter test specimen for use in equation 6.7b to find Csize.  Csurf for a machined finish is calculated using the values from Table 6-3.  Ctemp is found from equation 6.7e and Creliab is chosen from Table 6-4 for a 99.99% reliability level.

Uncorrected endurance limit

Sprime_e = 0.5 * Sut    % ksi

Sprime_e =

40

Load factor

Cload = 1;

95% stress area

A95 = @(d) 0.05 * boverd * d^2;

Equivalent diameter

deq = @(d) sqrt(A95(d) / 0.0766);

Size factor

Csize = @(d) 0.869 * deq(d)^-0.097;  % assumes deq has units of inches

Surface factor

A = 2.70;

bprime = -0.265;

Csurf = A * Sut^bprime

Csurf =

0.8454

Temperature factor

Ctemp = 1;

Reliability factor

Creliab = 0.753;

12	The corrected endurance limit is calculated using equation 6.6.

Se = @(d) Cload * Csize(d) * Csurf * Ctemp * Creliab * Sprime_e;

13	We can now use equation 6.14 as a design equation to solve for the beam thickness, d.  Setting the desired factor of safety to

Nf = 2.5;

Design equation: Nf=Seσ'

d0 = 0.75;   % in

d = fzero(@(d) Nf * SIGprimeA(d) - Se(d),d0)

d =

0.9996

Using d = 1.000 in (rounded) , the values of b, D, and r are

d = 1;   % in (rounded)

b = boverd * d

D = Doverd * d

r = roverd * d

b =

2

D =

1.1250

r =

0.5000

The beam deflection y is computed using equation i from Example 4-5.

Modulus of elasticity

E = 30e6;   % psi

a = 5.0;    % in (restores original 'a' definition)

Deflection at x = l

ymax = (Fa/(6*E*I(d)))*(l^3 - 3*a*l^2 - (l-a)^3)   % in

ymax =

-0.0054

14	Using this approach assures a safety factor of at least 2.5.  Since the deflection is 0.0054 in, the design is satisfactory.  This choice of ratios results in a low stress-concentration factor of

Kf(d)

ans =

1.1634

The dimension D is slightly less than a stock mill size so that material would be available for the cleanup and truing of the mounting surfaces.  Also, with this design, hot-rolled steel (HRS) could be used, rather than cold-rolled steel (CRS) initially assumed (Figure 6-41a).  Hot-rolled steel is less expensive than CRS and, if normalized, has less residual stress, but its rough, decarburized surface needs to be removed by machining all over, or be treated with shot peening to strengthen it.

15	If further iteration is desired, go back to step 5 and change the value of one or more of the ratios.