EXAMPLE 9-4A

Designing Shaft Keys:  An Alternate Approach

Problem	Design the key for the gear at B on the shaft in Examples 9-2 and 9-3.

Given	The loading is the same as in Example 9-2.  See Figure 9-3 for labels.

Mean torque

Tm = 73.07;  % lbf in

Alternating torque

Ta = 73.07;  % lbf in

Shaft diameter

d = 0.750;  % in

Assumptions	Use square, parallel keys with end-milled keyseats.  The shaft material is the same as in Example 9-3.  A lower carbon steel, SAE 1010, will be used for the keys.  See Figure 9-16 for stress- concentration factors.  Key material properties are:

Tensile strength

Sut = 53;    % ksi

Yield strength

Sy = 44;     % ksi

Solution	See M-Book Ex09-04A.doc and M-File Ex09_04.m.

1	There are two locations with keys on this shaft, at points B and D.  The design diameters chosen in Example 9-3 at these points were d1 and d3, respectively.  Table 9-2 shows the standard key width for the diameter at B as:

For

d

w = 0.1875; % in

d =

0.7500

The key length can be adjusted for each location.

2	At point B, the mean and alternating components of force on the key are found from the torque component divided by the shaft radius at that point.

Fa = Ta / (0.5 * d)    % lbf

Fm = Tm / (0.5 * d)    % lbf

Fa =

194.8533

Fm =

194.8533

3	The shear area and stress components can be written in terms of the unknown key length, L.

Shear area

Ashear=w⋅L

Shear stress

τa=FaAshear=Faw⋅L

4	To find a safety factor for shear fatigue of the key, compute the von Mises equivalent stresses for each of these components from equation 5.7d,

von Mises stress components

σ'a=3⋅τa=3Faw⋅Lσ'm=3⋅τm=3Fmw⋅L

and use them in equation 6.18e to determine the fatigue safety factor:

Nf=Se⋅SutSut⋅σ'a+Se⋅σ'm=Se⋅Sut3w⋅L+Sut⋅Fa+Se⋅Fm

5	Solve the fatigue safety factor equation for L and use the resulting equation as a design equation for any desired factor of safety.

L=3⋅Nf⋅Sut⋅Fa+Se⋅Fmw⋅Sut⋅Se

Before we can solve this equation we must first determine the endurance limit for the key, which is also a function of the unknown length, L.

6	Compute the unmodified endurance limit and the modification factors.

Uncorrected endurance strength

Sprme = 0.5*Sut % ksi

Sprme =

26.5000

Correction factors:

Load

Cload = 1;

Size

Csize=0.869⋅deq−0.097

95% stress area

A95=w⋅L

Equivalent diameter

deq=A950.0766=w⋅L0.0766

Substituting

Csize=0.869⋅w⋅L0.0766−0.5⋅0.097

Evaluating all but L

0.869*(w/0.0766)^(-0.5*0.097)   % assumes w [=] inches

ans =

0.8321

Csize as a function of L

Csize=0.832⋅L−0.0485

Surface (machined)

Csurf = 2.7*Sut^-0.265  % assumes Sut [=] ksi

Csurf =

0.9428

All other factors unity (assumes 50% reliability).  The endurance limit is then a function of L,

Se=Cload⋅Csize⋅Csurf⋅S'e=1⋅0.832⋅L−0.0485⋅0.943⋅26.5=20.791⋅L−0.0485

7	Substituting the expression for Se in the previous equation into the design equation in step 5 will give us a single equation in L that we can solve by iteration.

L=3⋅Nf⋅Sut⋅Fa+20.791⋅L−0.0485⋅Fmw⋅Sut⋅20.791⋅L−0.0485

Solving by iteration, let Nf = 2 and then the right-hand-side of the equation is

Nf = 2

Nf =

2

RHS = @(L) (sqrt(3)*Nf*(Sut*Fa + 20.791*L^-0.0485 * Fm)) / ...

(w*Sut*20.791*L^-0.0485) / 1000;   % in

L = 0.5 % in

L =

0.5000

L = RHS(L)

L =

0.2353

L = RHS(L)

L =

0.2293

L = RHS(L)

L =

0.2291

Round this up to 1/4 in

L = 0.250   % in, rounded

L =

0.2500

8	The bearing stress on the key is compression and thus can be considered a static load.  It is calculated using the maximum force on the key.

Bearing area

Abearing = (1/2)*w*L  % in2

Abearing =

0.0234

Bearing stress

SIGmax = (Fm + Fa) / Abearing /1000 % ksi

SIGmax =

16.6275

9	Calculate the safety factor for bearing failure from:

Ns = Sy / SIGmax

Ns =

2.6462

10	This is a satisfactory design.  The same method can also be used to size the key at D.  The endurance limit size-modification factor and the endurance limit can now be evaluated.

Csize = 0.832 * L^-0.0485   % assumes L [=] inches

Se = 20.791 * L^-0.0485     % assumes L [=] inches

Csize =

0.8899

Se =

22.2370