EXAMPLE 13-5

Design of a Helical Extension Spring for Cyclic Loading

Problem	Design an extension spring for a dynamic load over a given deflection.

Given	Minimum force

Fmin = 50;      % lbf

Maximum force

Fmax = 85;      % lbf

Working deflection

DELTAy = 0.50;  % in

Forcing frequency

ff = 500;       % cpm

Life

L = inf;

Assumptions	Standard hooks will be used at each end.  Music wire (ASTM A228) will be used since the loads are dynamic.  Setting and peening cannot be used to obtain a higher endurance strength in an extension spring.

Shear modulus

G = 11.5e6;     % psi

Solution	See Figure 13-24, M-Book Ex13-05.doc and M-File Ex13_05.m.

1	Assume a trial wire diameter from the available sizes in Table 13-2.  Assume a spring index of 9 and calculate the mean coil diameter D from equation 13.5.

Wire diameter

d = 0.177;  % in

Spring index

C = 9;      % coils

Mean coil diameter

D = C * d   % in

D =

1.5930

2	Use the assumed value of C to find an appropriate value of initial coil stress τi from equations 13.21:

TAUi1 = -4.231*C^3 + 181.5*C^2 - 3387*C + 28640  % psi

TAUi2 = -2.987*C^3 + 139.7*C^2 - 3427*C + 38404  % psi (eqn c)

TAUi = (TAUi1 + TAUi2) / 2   % psi (eqn d)

TAUi1 =

9.7741e+003

TAUi2 =

1.6699e+004

TAUi =

1.3237e+004

3	Find the direct shear factor Ks:

Direct shear factor

Ks = 1 + 0.5 / C

Ks =

1.0556

4	Use the value of τi from (c) in equation 13.8 to find the corresponding initial coil- tension force Fi:

Fi = (pi * d^3 * TAUi) / (8 * Ks * D)   % lbf

Fi =

17.1420

Check that this force is less than the required minimum applied force Fmin, which in this case, it is.  Any applied force smaller than Fi will not deflect the spring.

5	Find the the mean and alternating forces from equation 13.15a:

Alternating force

Fa = (Fmax - Fmin) / 2  % lbf

Fa =

17.5000

Mean force

Fm = (Fmax + Fmin) / 2  % lbf

Fm =

67.5000

6	Use the direct shear factor Ks and previously assumed values to find the mean stress τm:

Stress at Fm

TAUm = Ks * (8 * Fm * D) / (pi * d^3)   % psi

TAUm =

5.2122e+004

7	Find the Wahl factor Kw and use it to calculate the alternating shear stress τa in the coil.

Wahl factor

Kw = (4*C - 1)/(4*C - 4) + 0.615 / C

Kw =

1.1621

Alternating stress

TAUa = Kw * (8 * Fa * D) / (pi * d^3)   % psi

TAUa =

1.4877e+004

8	Find the ultimate tensile strength of this wire material from equation 13.3 and Table 13-4 and use it to find the ultimate shear strength from equation 13.4 and the torsional yield strength for the coil body from Table 13-10, assuming no set removal.

From Table 13-4, for A228 music wire

A = 184.649 * 1000; % psi

b = -0.1625;   % A and b from Table 13-4 for A228 music wire

Ultimate tensile strength

Sut = A * d^b       % psi (assumes d [=] in)

Sut =

2.4465e+005

Shear yield strength

Sys = 0.45 * Sut    % psi (assumes d [=] in)

Sys =

1.1009e+005

Ultimate shear strength

Sus = 0.667 * Sut   % psi (assumes d [=] in)

Sus =

1.6318e+005

9	Find the wire endurance limit for unpeened springs from equation 13.13 and convert it to fully reversed endurance strength with equation 13.16c.

Wire endurance limit

Sew = 45.0 * 1000   % psi

Sew =

45000

Fully reversed endurance limit

Ses = 0.5 * (Sew*Sus) / (Sus - 0.5*Sew)   % psi

Ses =

2.6098e+004

10	The fatigue safety factor for the coils in torsion is calculated from equation 13.16b.

Minimum stress

TAUmin = TAUm - TAUa    % psi

TAUmin =

3.7245e+004

Fatigue factor of safety

Nfs = (Ses*(Sus-TAUmin))/(Ses*(TAUm-TAUmin)+Sus*TAUa)

Nfs =

1.1672

Note that the minimum stress due to force Fmin is used in this calculation, not the coil-winding stress from (d).

11	The stresses in the end hooks also need to be determined.  The bending stresses in the hook are found from equation 13.23:

C1 = C

Kb = (4 * C1^2 - C1 - 1) / (4 * C1 * (C1 - 1))

C1 =

9

Kb =

1.0903

SIGa = Kb * (16*D*Fa)/(pi*d^3) + (4*Fa)/(pi*d^2)        % psi

SIGm = Kb * (16*D*Fm)/(pi*d^3) + (4*Fm)/(pi*d^2)        % psi

SIGmin = Kb * (16*D*Fmin)/(pi*d^3) + (4*Fmin)/(pi*d^2)  % psi

SIGa =

2.8626e+004

SIGm =

1.1042e+005

SIGmin =

8.1790e+004

12	Convert the torsional endurance strength to a tensile endurance strength with the von Mises relationship and use it and the ultimate tensile strength in equation 13.16 to find a fatigue safety factor for the hook in bending:

Se = Ses / 0.67     % psi

Nfb = (Se*(Sut-SIGmin))/(Se*(SIGm-SIGmin)+Sut*SIGa)

Se =

3.8953e+004

Nfb =

0.7814

13	The torsional stresses in the hook are found from equation 13.24 using an assumed value of

C2 = 5;

R2 = (C2*d) / 2   % in

R2 =

0.4425

Kw2 = (4*C2 - 1)/(4*C2 - 4)

Kw2 =

1.1875

TAUBa = Kw2 * (8 * Fa * D) / (pi * d^3)         % psi

TAUBm = Kw2 * (8 * Fm * D) / (pi * d^3)         % psi

TAUBmin = Kw2 * (8 * Fmin * D) / (pi * d^3)     % psi

TAUBa =

1.5202e+004

TAUBm =

5.8637e+004

TAUBmin =

4.3435e+004

14	The fatigue safety factor for the hook in torsion is calculated from equation 13.16b.

Nfs = (Ses*(Sus-TAUBmin))/(Ses*(TAUBm-TAUBmin)+Sus*TAUBa)

Nfs =

1.0861

One of these safety factors is less than 1, making this an unacceptable design.  To get some idea of what to change to improve it, the model was solved for a list of values of the spring index from 4 to 14, keeping all other parameters as defined above.  The resulting values of coil diameter, free length, spring weight, and fatigue safety factor are plotted in Figure 13-24.

The safety factor decreases with increasing spring index, so a reduction in our assumed value for C will improve it even with no change in wire diameter.  Note however that the spring free length shows a minimum value at a spring index of about 7.5.  The coil diameter increases linearly with the spring index for a constant wire diameter.  Spring weight decreases with increasing spring index.

If we decrease the spring index from 9 to 7.5, and increase the wire diameter one size to 0.192 in, keeping all other parameters the same, we will obtain an acceptable design in this case with the smallest Nf = 1.2 for the hook in bending.  Using these new values of d and C, the new safety factors are calculated below as are the calculations necessary to complete the design.  A summary of the changed values is

Wire diameter

d = 0.192;  % in

Spring index

C = 7.5;    % coils

Mean coil diameter

D = C * d   % in

D =

1.4400

Direct shear factor

Ks = 1 + 0.5 / C

Ks =

1.0667

Torsion in coil body:

Stress at Fmin

TAUmin = Ks * (8 * Fmin * D) / (pi * d^3)   % psi

TAUmin =

2.7631e+004

Stress at Fm

TAUm = Ks * (8 * Fm * D) / (pi * d^3)       % psi

TAUm =

3.7302e+004

Wahl factor

Kw = (4*C - 1)/(4*C - 4) + 0.615 / C

Kw =

1.1974

Alternating stress

TAUa = Kw * (8 * Fa * D) / (pi * d^3)   % psi

TAUa =

1.0856e+004

Fatigue factor of safety for coil body

Nfs = (Ses*(Sus-TAUmin))/(Ses*(TAUm-TAUmin)+Sus*TAUa)

Nfs =

1.7480

Bending in end hooks:

C1 = C

C1 =

7.5000

Kb = (4 * C1^2 - C1 - 1) / (4 * C1 * (C1 - 1))

Kb =

1.1103

SIGa = Kb * (16*D*Fa)/(pi*d^3) + (4*Fa)/(pi*d^2)        % psi

SIGm = Kb * (16*D*Fm)/(pi*d^3) + (4*Fm)/(pi*d^2)        % psi

SIGmin = Kb * (16*D*Fmin)/(pi*d^3) + (4*Fmin)/(pi*d^2)  % psi

SIGa =

2.0737e+004

SIGm =

7.9984e+004

SIGmin =

5.9247e+004

Nfb = (Se*(Sut-SIGmin))/(Se*(SIGm-SIGmin)+Sut*SIGa)

Nfb =

1.2280

Torsion in end hooks:

R2 = (C2*d) / 2   % in

Kw2 = (4*C2 - 1)/(4*C2 - 4)

R2 =

0.4800

Kw2 =

1.1875

TAUBa = Kw2 * (8 * Fa * D) / (pi * d^3)         % psi

TAUBm = Kw2 * (8 * Fm * D) / (pi * d^3)         % psi

TAUBmin = Kw2 * (8 * Fmin * D) / (pi * d^3)     % psi

TAUBa =

1.0766e+004

TAUBm =

4.1528e+004

TAUBmin =

3.0761e+004

Nfs = (Ses*(Sus-TAUBmin))/(Ses*(TAUBm-TAUBmin)+Sus*TAUBa)

Nfs =

1.6959

15	The spring rate is defined from the two specified forces at their relative deflection.

Spring rate

k = (Fmax - Fmin) / DELTAy   % lbf / in

k =

70

16	To get the defined spring rate, the number of active coils must satisfy equation 13.7, solving for Na yields:

Number of active coils

Na = (d^4 * G) / (8 * D^3 * k)

Na =

9.3460

Na = 9.25  % rounding to nearest 1/4 coil

Na =

9.2500

Note that we round it to the nearest 1/4 coil as the manufacturing tolerance cannot achieve better than that accuracy.  Having rounded the number of active coils, we must now calculate the spring rate using equation 13.7:

Corrected spring rate

k = (d^4 * G) / (8 * D^3 * Na)  % lbf / in

k =

70.7267

17	The total number of coils in the body and the body length are

Total coils

Nt = Na + 1

Nt =

10.2500

Body length

Lb = Nt*d       % in

Lb =

1.9680

18	The free length can now be determined.  The length of a standard hook is equal to the coil inside diameter:

Hook length

Lhook = D - d       % in

Lhook =

1.2480

Free length

Lf = Lb + 2*Lhook   % in

Lf =

4.4640

19	The initial coil tension force must be found again in order to obtain the deflection to reach the larger of the two loads.

TAUi1 = -4.231*C^3 + 181.5*C^2 - 3387*C + 28640   % psi

TAUi2 = -2.987*C^3 + 139.7*C^2 - 3427*C + 38404   % psi

TAUi = (TAUi1 + TAUi2) / 2   % psi

TAUi1 =

1.1662e+004

TAUi2 =

1.9299e+004

TAUi =

1.5481e+004

Fi = (pi * d^3 * TAUi) / (8 * Ks * D)   % lbf

Fi =

28.0132

ymax = (Fmax - Fi) / k   % in

ymax =

0.8057

20	The inside and outside coil diameters are

Inside coil diameter

Di = D - d  % in

Di =

1.2480

Outside coil diameter

Do = D + d  % in

Do =

1.6320

21	The weight of the spring's active coils is found from equation 13.11b and is

Weight density

RHO = 0.28;  % lbf / in3

Weight

Wa = (pi^2 * d^2 * D * Na * RHO) / 4    % lbf

Wa =

0.3392

22	The natural frequency of this spring is found from equation 13.11a and is:

Natural frequency

g = 32.174 * 12     % in / sec2

fn = 0.5 * sqrt((k*g)/Wa)       % Hz

fn = 0.5 * sqrt((k*g)/Wa) * 60  % cpm

g =

386.0880

fn =

141.8576

fn =

8.5115e+003

The ratio between the natural frequency and the forcing frequency is

Frequency ratio

FRatio = fn / ff

FRatio =

17.0229

which is sufficiently high.

23	We now have a complete design specification for this A228-wire spring:

Wire diameter

d   % in

d =

0.1920

Outside diameter

Do  % in

Do =

1.6320

Total coils

Nt  % coils

Nt =

10.2500

Free length

Lf  % in

Lf =

4.4640