EXAMPLE 14-1

Torque and Efficiency of a Power Screw

Problem	Determine the lifting and lowering torques and the efficiency of the power screw shown in Figure 14-4 using an Acme screw and nut.  Is it self-locking?  What is the contribution of collar friction versus screw friction if the collar has a) sliding friction, b) rolling friction?

Given	The screw is a single-start Acme 1.25-5.

Axial load

P = 1000;   % lbf

Screw pitch

N = 5;      % in-1

Mean collar diameter

dc = 1.75;  % in

Assumptions	The screw and nut are lubricated with oil.

Sliding friction

MU = 0.15;

Rolling friction

MUcr = 0.02;

Solution	See M-Book Ex14-01.doc and M-File Ex14_01.m.

1	There are several aspects to this problem.  We need to compute the lifting and lowering torques for two cases, one with a sliding-friction collar and one with a ball-bearing collar.  In both cases we will calculate the screw and collar contribu- tions to torque and efficiency separately for comparison and also combine them.  First do the case of the sliding collar.

2	Since it is a single-start thread, the lead L equals the pitch p.  The pitch diameter of the thread dp is found in Table 14-3.  The torque to lift the load is found from equation 14.5a:

Screw lead

L = 1/N     % in

L =

0.2000

Screw pitch diameter

dp = 1.15;  % in

Thread angle

ALPHAdeg = 14.5;            % deg

ALPHA = 14.5 * (2*pi/360)   % rad

ALPHA =

0.2531

Screw torque

Tsu = ((P*dp)/2)*((MU*pi*dp + L*cos(ALPHA))/...

(pi*dp*cos(ALPHA) - MU*L))  % in lbf

Tsu =

121.9647

Collar torque

Tc = MU * P * dc / 2    % in lbf

Tc =

131.2500

Total lifting torque

Tu = Tsu + Tc           % in lbf

Tu =

253.2147

Note that the collar friction exceeds the screw friction.

3	The torque to lower the load is found from equation 14.5b:

Screw torque

Tsd = ((P*dp)/2)*((MU*pi*dp - L*cos(ALPHA))/...

(pi*dp*cos(ALPHA) + MU*L))  % in lbf

Tsd =

56.7698

Collar torque

Tc = MU * P * dc / 2    % in lbf

Tc =

131.2500

Total lowering torque

Td = Tsd + Tc           % in lbf

Td =

188.0198

4	The efficiency in the lifting mode will be less than for lowering and is found from equations 14.7.  We choose the version shown as equation 14.7c in order to account for both screw and collar components.

Screw only

escrew = (P * L) / (2 * pi * Tsu)

escrew =

0.2610

Screw and collar combined

ee = (P * L) / (2 * pi * Tu)

ee =

0.1257

5	Now recalculate the collar torque and total torque to lift the load with a ball-bearing thrust washer, using equation 14.4f.

Tc = MUcr * P * dc / 2      % in lbf

Tu = Tsu + Tc               % in lbf

Tc =

17.5000

Tu =

139.4647

6	The efficiency with the ball bearing thrust-washer is now:

ee = (P * L) / (2 * pi * Tu)

ee =

0.2282

The improvement in efficiency is significant and shows why it is poor practice to use anything but a rolling-element bearing as a thrust washer on a power screw.

7	The self-locking aspects of the screw are independent of the thrust-collar friction and can be found from equation 14.6a.  To be self-locking,

rhs = L/(pi*dp) * cos(ALPHA)

rhs =

0.0536

Since

MU

MU =

0.1500

it is self-locking.

Note that the positive lowering torque in step 3 also indicates that the screw is self-locking.  A negative lowering torque means that a braking torque of opposite sense to the lift torque must be applied to hold the load.