EXAMPLE 7-5

Finding the Safety Factor in Surface Fatigue

Problem	Choose a material to provide 10 years of life for the rollers in Example 7-4.

Given	Roller radii

R1 = 1.250;  % in

R2 = 2.500;  % in

Radial load

F = 5000;    % lbf

Length of contact

L = 24;      % in

Roller 1 speed

OMEGA1 = 4000;   % rpm

Principal stresses

SIG1 = -21.2;    % ksi

SIG2 = -26.0;    % ksi

SIG3 = -49.8;    % ksi

The parts are hardened steel.  Therefore

E = 30e6;        % psi

v = 0.28;

Assumptions	There is 9% sliding combined with rolling.  The machine will operate 3 shifts/day for 345 days/year.

Solution	See M-Book Ex07-05.doc and M-File Ex07_05.m.

1	Calculate the required cycle life from the given data.

year = 345;      % days

Nreq = OMEGA1 * (60 * 24) * 10 * year   % cycles

Nreq =

1.9872e+010

2	The maximum normal stress calculated in Example 7-4 is 49.8 ksi, compressive.  Its K-factor can be calculated from equation 7.25d.  The previously calculated material constants m1 and m2 are needed:

Material constants

m1 = (1 - v^2)/E    % psi^-1

m2 = m1             % psi^-1

m1 =

3.0720e-008

m2 =

3.0720e-008

K-factor

K = pi * (m1 + m2) * SIG3^2 *1e6    % psi

K =

478.6960

3	A trial material must be selected from Table 7-8.  With a K this low, virtually any of the steels can probably be used.  We will try the HB 130-170 SAE 1020, phosphate coated steel (#27 in part 2 of the table), since the same materials are running together.  The slope and intercept factors of this steel for rolling with 9% sliding are

LAMDA = 6.38;

ZETA = 28.23;

4	These are used in equation 7.26 along with the value of K from above to find the number of cycles that can be expected at this load before pitting begins.

log(K)=ζ−log(Nlife)λ

Nlife = 10^(ZETA - LAMDA*log10(K))  % cycles (assumes K [=] psi)

Nlife =

1.3528e+011

5	A safety factor against pitting can now be calculated from the ratio of the projected cycle life and the desired number of cycles.

Nf = Nlife / Nreq   % no units

Nf =

6.8073