EXAMPLE 6-5

Design of a Cantilever Bracket for Fluctuating Bending

Problem	A feed-roll assembly is to be mounted at each end on support brackets cantilevered from the machine frame as shown in Figure 6-47.  The feed rolls experience a total fluctuating load that varies from a minimum of 200 lb to a maximum of 2200 lb, split equally between the two support brackets.  Design a cantilever bracket to support a fluctuating bending load of 100- to 1100-lb amplitude for 109 cycles with no failure.  Its dynamic deflection cannot exceed 0.02 in.

Given	Load limits

Fmin = 100;    % lbf

Fmax = 1100;   % lbf

The load-time function shape is shown in Figure 6-47.  The operating environment is room air at a maximum temperature of 120 F.  The space available allows a maximum cantilever length of 6 in.  Only ten of these parts are required.

Assumptions:	The bracket can be clamped between essentially rigid plates or bolted at its root.  The normal load will be applied at the effective tip of the cantilever beam from a rod attached through a small hole in the beam.  Since the bending moment is effectively zero at the beam tip, the stress concentration from this hole can be ignored.  Given the small quantity required, machining of stock mill shapes is the preferred manufacturing method.  Use a reliability of 99.9%.

Solution	See Figure 6-47, M-Book Ex06-05.doc and M-File Ex06_05.m.

1	This is a typical design problem.  Very little data are given except for the required performance of the device, some limitations on size, and the required cycle life.  We will have to make some basic assumptions about part geometry, materials, and other factors as we go.  Some iteration should be expected.

2	Figure 6-47 shows the same tentative design configuration as in Figure 6-41a.  The mill stock is purchased thicker than the desired final dimension and machined top and bottom to dimension D, then machined to thickness d over length l.  A fillet radius r is provided at the clamp point to reduce fretting fatigue and achieve a lower Kt.  (See Figure 4-37).  Figure 4-36 shows that with suitable control of the r/d and D/d ratios for a stepped flat bar in bending, the geometric stress-concentration factor Kt can be kept under about 1.5.

3	A material must be chosen.  For infinite life, low cost, and ease of fabrication, it is desirable to use a low-carbon alloy steel if possible and if environmental conditions permit.  Since this is used in a controlled, indoor environment, carbon steel is acceptable on the latter point.  The fact that the deflection is of concern is also a good reason to choose a material with a large E.  Low-carbon, ductile steels have the requisite endurance-limit knee for the infinite life required in this case and also have low notch sensitivities.  A low-carbon steel with Sut = 80 ksi and Sy = 60 ksi is selected for the first trial.

Sut = 80;    % ksi (assumes low-carbon steel)

Sy = 60;     % ksi

4	We will assume the trial dimensions to be the same as those of the successful solution to the fully reversed case from Example 6-4.  These are

Beam width

b = 2.00;    % in

Beam thickness

d = 1.00;    % in

Base thickness

D = 1.125;   % in

Fillet radius

r = 0.5;     % in

Distance to load

a = 5.00;    % in

Beam length

l = 6.00;    % in

This valueof 'a' will leave some material around the hole and still fit within the 6-in-length constraint.

5	The mean and alternating components of the load and their reaction forces can be calculated from the given maximum and minimum load.

Mean load

Fm = 0.5 * (Fmax + Fmin)    % lbf

Fm =

600

Alternating load

Fa = 0.5 * (Fmax - Fmin)    % lbf

Fa =

500

Reaction force

Rm = Fm     % lbf

Ra = Fa     % lbf

Rmax = Fmax   % lbf

Rm =

600

Ra =

500

Rmax =

1100

From these, the mean and alternating moments, and the maximum moment acting at the root of the cantilever beam can be calculated.

Reaction moment

Ma = Ra * l - Fa * (l-a)    % lbf in

Mm = Rm * l - Fm * (l-a)    % lbf in

Mmax = Rmax * l - Fmax * (l-a)   %lbf in

Ma =

2500

Mm =

3000

Mmax =

5500

Table 6-11 contains equations for area moment of inertia and the distance to the outer fiber.

Moment of inertia

I = (b * d^3) / 12    % in4

I =

0.1667

Outer fiber distance

c = 0.5 * d     % in

c =

0.5000

The nominal bending stresses at the root are found for both the alternating load and the mean load.

Nominal alternating stress

SIGanom = (Ma * c) / I /1000    % ksi

SIGmnom = (Mm * c) / I /1000    % ksi

SIGmax = (Mmax * c) / I /1000   % ksi (used in Step 7)

SIGanom =

7.5000

SIGmnom =

9

SIGmax =

16.5000

6	Calculate the stress concentration factor for this geometry using Figure 4-36 or Figure E-10 in Appendix E.  The r/d and D/d ratios are

rdRatio = r/d    % no units

rdRatio =

0.5000

and

Dprime = D/d     % no units

Dprime =

1.1250

Using cubic interpolation to find the constants A and b (b' in this case since we are already using b as the width of the bar).  From the table and equation in Figure 4-36,

Coeff = [1 1.07 1.07^2 1.07^3; ...

1 1.10 1.10^2 1.10^3; ...

1 1.20 1.20^2 1.20^3; ...

1 1.30 1.30^2 1.30^3];

CArhs = [1.01990; 1.01650; 0.99590; 0.95880];

CA = Coeff\CArhs

CA =

0.9911

-0.4106

0.9309

-0.4877

A = CA(1) + CA(2)*Dprime + CA(3)*Dprime^2 + CA(4)*Dprime^3

A =

1.0127

CBrhs = [-0.20333; -0.21548; -0.23829; -0.27269];

Cb = Coeff\CBrhs

Cb =

13.7466

-35.2542

29.7899

-8.4360

bprime = Cb(1) + Cb(2)*Dprime + Cb(3)*Dprime^2 + Cb(4)*Dprime^3

bprime =

-0.2229

Kt = A * (rdRatio)^bprime

Kt =

1.1819

7	Calculate the notch sensitivity q of the chosen material based on its ultimate strength and notch radius using equation 6.13 and Table 6-6.  The values of q and Kt are used to find the fatigue stress-concentration factor Kf.  Kfm is calculated using equation 6.17.

Neuber constant

a = 0.080^2  % in (overwrites the previous definition of a)

a =

0.0064

q = (1 / (1 + sqrt(a)/sqrt(r)))

q =

0.8984

Kf = 1 + q * (Kt - 1)

Kf =

1.1634

if Kf⋅σmax<Sy then Kfm = Kf.

Kf * abs(SIGmax)

ans =

19.1965

Since this ans is less than Sy = 60 ksi,

Kfm = Kf

Kfm =

1.1634

8	Use these factors to find the local mean and alternating notch stresses.

SIGa = Kf * SIGanom

SIGm = Kfm * SIGmnom

SIGa =

8.7257

SIGm =

10.4708

The local stresses are used to compute the von Mises alternating and mean stresses from equations 6.22b.

SIGxa = SIGa    % ksi

SIGya = 0;      % ksi

TAUxya = 0;     % ksi

SIGprimeA = sqrt(SIGxa^2 + SIGya^2 - SIGxa*SIGya + TAUxya^2)    % ksi

SIGxa =

8.7257

SIGprimeA =

8.7257

SIGxm = SIGm    % ksi

SIGym = 0;      % ksi

TAUxym = 0;     % ksi

SIGprimeM = sqrt(SIGxm^2 + SIGym^2 - SIGxm*SIGym + TAUxym^2)    % ksi

SIGxm =

10.4708

SIGprimeM =

10.4708

9	Using the assumed value for the ultimate tensile stress above, calculate the corrected endurance limit.  The load factor is found from equation 6.7a.  The size factor for this rectangular part is determined by calculating the cross-sectional area stressed above 95% of its maximum stress (see Figure 6-25) and using that value in equation 6.7d find an equivalent diameter test specimen for use in equation 6.7b to find Csize.  Csurf for a machined finish is calculated using the values from Table 6-3.  Ctemp is found from equation 6.7e and Creliab is chosen from Table 6-4 for a 99.9% reliability level.

Uncorrected endurance limit

Sprime_e = 0.5*Sut   % ksi

Sprime_e =

40

Load factor

Cload = 1;

95% stress area

A95 = 0.05 * d * b   % in2

A95 =

0.1000

Equivalent diamter

deq = sqrt(A95/0.0766)      % in

deq =

1.1426

Size factor

Csize = 0.869*deq^-0.097    % in, deq [=] in

Csize =

0.8578

Surface factor

A = 2.70;

bprime = -0.265;

Csurf = A * Sut^bprime      % Sut [=] ksi

Csurf =

0.8454

Temperature factor

Ctemp = 1;

Reliability factor

Creliab = 0.753;

10	The corrected endurance limit is calculated using equation 6.6.  Note that the corrected Se is only about 25% of Sut.

Se = Cload * Csize * Csurf * Ctemp * Creliab * Sprime_e   % ksi

Se =

21.8426

11	The four possible safety factors are calculated using equation 6.18.  The smallest or most appropriate one can be selected from those calculated.

Modulus of elasticity

E = 30e6;    % psi

a = 5.0;     % in (restores original definition of a)

Case 1 Factor of safety

Nf1 = (Sy/SIGprimeM) * (1 - (SIGprimeA/Sy))

Nf1 =

4.8969

Case 2 Factor of safety

Nf2 = (Se/SIGprimeA) * (1 - (SIGprimeM/Sut))

Nf2 =

2.1756

Case 3 Factor of safety

Nf3 = (Se * Sut) / (SIGprimeA * Sut + SIGprimeM * Se)

Nf3 =

1.8855

Case 4 Factor of safety

SaS = Se * (1 - ((Se^2 + Sut * SIGprimeM - Se * SIGprimeA) /...

(Se^2 + Sut^2)))    % ksi

SmS = Sut * ((Se^2 + Sut * SIGprimeM - Se * SIGprimeA) /...

(Se^2 + Sut^2))    % ksi

Nf4 = (sqrt(SIGprimeA^2 + SIGprimeM^2) +...

sqrt((SaS - SIGprimeA)^2 + (SmS - SIGprimeM)^2)) /...

sqrt(SIGprimeA^2 + SIGprimeM^2)

SaS =

18.2721

SmS =

13.0773

Nf4 =

1.7260

The maximum deflection is calculated using the maximum applied force Fmax.

Deflection at x = l

ymax = (Fmax / (6 * E * I)) * (l^3 - 3 * a * l^2 - (l-a)^3)  % in

ymax =

-0.0119

13	The dimensions of the successful design from Example 6-4, which were assumed in step 4, give a Case 4, minimum safety factor of Nf4 = 1.7 in this case.  It should be expected that the addition of a mean stress to the same level of alternating stress as before would require a stronger part.

14	A few iterations will give a better result.  The final dimensions are b = 2 in, d = 1.2 in, D = 1.4 in, r = 0.5, a = 5.0 in, and l = 6.0 in. The smallest safety factor is now 2.3 and the maximum deflection is 0.007 in.  These are both acceptable.  The dimension D was deliberately chosen to be slightly less than a stock mill size so that material would be available for the cleanup and truing of the mounting surfaces.