EXAMPLE 10-3

Selection of Ball Bearings for a Designed Shaft

Problem	Select radial ball bearings for the shaft shown in Figure 9-5.  The shaft was designed in Example 9-5.

Given	Load at R1

P1 = 16;     % lbf

Load at R2

P2 = 54;     % lbf

Design life

L = 25000;   % hr

L = 25000*60   % min

L =

1500000

Shaft diameter

d = 15;      % mm

Shaft speed

n = 1725;    % rpm

Assumptions	Thrust loads are negligible.  Use the same bearings at R1 and R2

Find	The bearing fatigue lives at both shaft locations and the reliability of the bearing at R2 for the design life of 25 khr.

Solution	See M-Book Ex10-03.doc and M-File Ex10_03.m.

1	Calculate the design life in millions of revolutions.

LD = (L * n) / 1e6  % millions of revolutions

LD =

2.5875e+003

2	Solve equation 10.19 for the required basic load rating using the load at R2.  Note that the equivalent load in this case is simply the applied radial load due to the absence of any thrust load.  For the larger reaction load of 54 lb at R2, the required minimum basic load rating is:

C = P2 * LD^(1/3)   % lbf

C =

741.3453

3	From a manufacturer's catalog, select a bearing with a basic load rating that is at least this value.  In this case, consult the partial catalog reproduced in Figure 10-23.  Normally, we would find the bearing that had the smallest bore with the required load rating.  The only 15-mm bore in the figure is #6302 with

C6302 = 1930;    % lbf

4	Calculate the projected life of each bearing with equation 10.19.

The L10 life for the bearing at R2 is

L10 = (C6302/P2)^3 * 1e6 / n / 60   % hr

L10 =

4.4111e+005

The L10 life for the bearing at R1 is

L10 = (C6302/P1)^3 * 1e6 / n / 60   % hr

L10 =

1.6958e+007

This shows the nonlinear relationship between load and life.  A 3.5x reduction in load results in a 38x increase in fatigue life.  These bearings are obviously very lightly loaded, but their size was dictated by considerations of stresses in the shaft.

5	The calculations made thus far are for a reliability of 90%.  The reliability of the bearing at the design life of L = 25000 hr is found by solving the following equation for R (using Matlab's fzero function).

FUNC = @(R) C6302-P2*(LD./(0.02 + ...

4.439*log(1./R).^(1/1.483))).^(1/3);

R = fzero(FUNC,[0.90 1])

R =

0.9992

6	From Figure 10-23, this bearing's limiting speed is 18000 rpm, well above the operating speed of 1725 rpm.