EXAMPLE 13-3A

Design of a Helical Compression Spring for Static Loading:  An Alternate Approach

Problem	Design a compression spring for a static load over a known deflection with a factor of safety against yielding at shut height of at least 1.1.

Given	Minimum force

Finit = 100;     % lbf

Maximum force

Fwork = 150;     % lbf

Working deflection

DELTAy = 0.75;   % in

Assumptions	Use the least expensive, unpeened, cold-drawn spring wire (ASTM A227) since the loads are static.

Shear modulus

G = 11.5e6;      % psi

Solution	See M-Book Ex13-03A.doc and M-File Ex13_03A.m.

1	We will derive a design equation for this problem that will yield a value for the wire diameter that is a function of two parameters, spring index C and the ratio, α, of the clash allowance to the working deflection.  To start, we write the equation for the factor of safety against yielding at shut height

τshut=SysNs	(a)

From the given data we have a desired value for the spring rate

k = (Fwork - Finit) / DELTAy    % lbf / in

k =

66.6667

But, from equations 13.5 and 13.7, the spring rate is given as

k=d⋅G8⋅C3⋅Na	(c)

Eliminating k from equations b and c and solving for the number of active coils, Na, we have

Na=G⋅Δy⋅d8⋅C3⋅Fwork−Finit	(d)

Combining equations 13.5, 13.7, and 13.8b, the stress at shut height is

τshut=8⋅k⋅C+0.5π⋅d2⋅yshut	(e)

where the shut height yshut is

yshut=ywork+Δyclash=Fworkk+α⋅Δy	(f)

and

α=ΔyclashΔy

Substituting equation f into equation e,

τshut=8⋅k⋅C+0.5π⋅d2⋅Fworkk+α⋅Δy	(g)

From equation 13.3 and Table 13-6, the torsional yield strength of the wire is

S=Km⋅A⋅db	(h)

and Km is the reduction factor taken from Table 13-6, expressed as as a decimal fraction.

Substituting equations g and h into a and solving for d yields our design equation

d=8⋅Ns⋅C+0.5⋅Fwork⋅1+α−α⋅Finitπ⋅Km⋅A12+b	(i)

Once we choose a material for the wire, the only unknowns in this equation are the parameters C (spring index) and α (clash allowance to working deflection ratio).

2	Assume a spring index of 8 and a clash allowance of 15% of the working deflection, then

Spring index

C = 8;

Clash allowance ratio

ALPHA = 0.15

ALPHA =

0.1500

3	From Tables 13-4 and 13-6 for A227 wire we have

A = 141.04 * 1000   % psi

b = -0.1822

Km = 0.60

A =

141040

b =

-0.1822

Km =

0.6000

4	Using these values and equation i we can solve for the required wire diameter.  In order to compare this solution with Example 13-3, let

Ns = 1.24;

d = ((8*Ns*(C+0.5)*(Fwork*(1+ALPHA)-ALPHA*Finit))/...

(pi*Km*A))^(1/(2+b))  % in

d =

0.1923

This is a preferred diameter as given in Table 13-2, so we will accept it.

5	Calculate the mean coil diameter D from equation 13.5 for

d = 0.192   % in

d =

0.1920

Mean coil diameter

D = C*d     % in

D =

1.5360

6	Find the direct shear factor Ks and use it to calculate the shear stress in the coil at the larger force.

Direct shear factor

Ks = 1 + 0.5/C

Ks =

1.0625

Stress at Fwork

TAUwork = Ks * (8*Fwork*D) / (pi*d^3)   % psi

TAUwork =

8.8074e+004

7	Find the ultimate tensile strength of this wire material from equation 13.3 and Table 13-4 and use it to find the torsional yield strength from Table 13-6, assuming that the set has been removed and using the low end of the recommended range.

Ultimate tensile strength

Sut = A * d^b     % psi (assumes d [=] inches)

Sut =

1.9051e+005

Shear yield strength

Sys = Km * Sut    % psi

Sys =

1.1431e+005

8	Find the safety factor against yielding at this working deflection from equation 13.14.

Safety factor at working deflection

Ns = Sys / TAUwork

Ns =

1.2979

9	To achieve the desired spring rate, the number of active coils must satisfy equation 13.7, solving for Na yields:

Number of active coils

Na = (d^4 * G) / (8 * D^3 * k)

Na =

8.0859

Na = 8  % rounding to nearest 1/4 coil

Na =

8

Note that we round it to the nearest 1/4 coil as the manufacturing tolerance cannot achieve better than that accuracy.  Having rounded the number of active coils, we must now calculate the spring rate using equation 13.7:

Corrected spring rate

k = (d^4 * G) / (8 * D^3 * Na)  % lbf / in

k =

67.3828

10	Assume squared and ground ends making the total number of coils, from Figure 13-9:

Total coils

Nt = Na + 2

Nt =

10

The shut height can now be determined.

Shut height

Ls = d*Nt   % in

Ls =

1.9200

11	The initial deflection to reach the smaller of the two loads is

yinit = Finit / k

yinit =

1.4841

12	Calculate the clash allowance:

DELTAyClash = 0.15 * DELTAy  % in

DELTAyClash =

0.1125

13	The free length (see Figure 13-8) can now be found from

Lf = Ls + DELTAyClash + DELTAy + yinit   % in

Lf =

4.2666

14	To check for buckling, two ratios need to be calculated, Lf / D and ymax / Lf.

Slenderness ratio

sr = Lf / D

sr =

2.7777

Deflection ratio

yprm = (yinit + DELTAy) / Lf

yprm =

0.5236

Take these two values to Figure 13-14 and find that their coordinates are safely within the zones that are stable against buckling for either end-condition case.

15	The inside and outside coil diameters are

Inside coil dia

Di = D - d  % in

Di =

1.3440

Outside coil dia

Do = D + d  % in

Do =

1.7280

16	The smallest hole and largest pin that should be used with this spring are

Smallest hole

holeMin = Do + 0.05 * D     % in

holeMin =

1.8048

Largest pin

pinMax = Di - 0.05 * D      % in

pinMax =

1.2672

17	The total weight of the spring is

Weight density

RHO = 0.28;  % lbf / in3

Weight

Wt = (pi^2 * d^2 * D * Nt * RHO) / 4    % lbf

Wt =

0.3912

18	We now have a complete design specification for this A227-wire spring:

Wire diameter

d   % in

d =

0.1920

Outside diameter

Do  % in

Do =

1.7280

Total coils (ends squared and ground)

Nt  % coils

Nt =

10

Free length

Lf  % in

Lf =

4.2666