EXAMPLE 7-3

Stresses in a Crowned Cam Follower

Problem	A crowned cam roller-follower has a gentle radius transverse to its rolling direction to eliminate the need for critical alignment of its axis with that of the cam.  The cam's radius of curvature and dynamic load vary around its circumference.  What is the size of the contact patch between cam and follower and what are the worst-case stresses?

Given	Roller radius

R1 = 1.000;      % in

Crown radius (90 deg to roller rad)

Rprm1 = 20.000;  % in

Cam curvature (radial)

R2 = 3.460;      % in

Cam curvature (axial)

Rprm2 = inf;     % in

Radial load (normal to contact plane)

F = 250;         % lbf

Angle between planes of R1 and R2

THETA = 0;       % deg (or, rad)

The parts are hardened steel.  Therefore

E = 30e6;        % psi

v = 0.28;

Assumptions	The relative motion is rolling with < 1% sliding.

Solution	See M-Book Ex07-03.doc and M-File Ex07_03.m.

1	Find the material constants from equation 7.9b.

Material constants

m1 = (1-v^2)/E  % psi^-1

m2 = m1         % psi^-1

m1 =

3.0720e-008

m2 =

3.0720e-008

2	Two geometry constants are needed from equations 7.19a.

Geometry constants

A = (1/2)*((1/R1) + (1/Rprm1) + (1/R2) + (1/Rprm2))  % in^-1

A =

0.6695

B = (1/2)*(((1/R1)-(1/Rprm1))^2 + ((1/R2)-(1/Rprm2))^2 +...

(2*((1/R1)-(1/Rprm1))*((1/R2)-(1/Rprm2)))*...

cos(2*THETA))^(1/2)     % in-1

B =

0.6195

The angle φ is found from their ratio,

Angle φ

PHI = acos(B/A)     % rad

PHI =

0.3889

PHIdeg = acos(B/A)*(360/(2*pi)) % deg

PHIdeg =

22.2836

and used in Table 7-5 to find the factors ka and kb.  Cubic interpolation for ka and linear interpolation for kb gives

Factors from Table 7-5

Cubic interpolation

ka = 3.444;

Linear interpolation

kb = 0.427;

3	Determine the contact patch dimensions using the material and geometry constants in equations 7.19d.

Major axis half-width

a = ka*((3/4)*((m1+m2)/A)*F)^(1/3)   % in

a =

0.0889

Minor axis half-width

b = kb*((3/4)*((m1+m2)/A)*F)^(1/3)   % in

b =

0.0110

where a is the half-width of the major axis, and b is the half-width of the minor axis of the contact patch.  The contact patch area is then

Contact area

A = pi * a * b  % in2 (redefines A)

A =

0.0031

4	The average and maximum contact pressure can now be found from equations 7.18b and c.

Average pressure

Pavg = F/A/1000     % ksi

Pavg =

81.1898

Maximum pressure

Pmax = (3/2)*Pavg   % ksi

Pmax =

121.7847

5	The maximum normal stresses in the center of the contact patch at the surface are then found using equations 7.21a.

In-plane

SIGx = -(2*v+(1-2*v)*b/(a+b))*Pmax  % ksi

SIGy = -(2*v+(1-2*v)*a/(a+b))*Pmax  % ksi

SIGx =

-74.1103

SIGy =

-115.8739

Axial

SIGz = -Pmax                        % ksi

SIGz =

-121.7847

These stresses are principal:

SIG1 = SIGx     % ksi

SIG2 = SIGy     % ksi

SIG3 = SIGz     % ksi

SIG1 =

-74.1103

SIG2 =

-115.8739

SIG3 =

-121.7847

The maximum shear stress associated with them at the surface is

TAU13 = abs((SIG1 - SIG3)/2)    % ksi

TAU13 =

23.8372

6	The maximum shear stress under the surface on the z-axis is approximately

Max shear stress

TAU13max = 0.34*Pmax    % ksi

TAU13max =

41.4068

7	All of the stresses found so far exist on the centerline of the patch.  At the edge of the patch, at the surface, there will also be a shear stress.  Two constants are found from equation 7.21b for this calculation.

k3 = b/a

k4 = (1/a)*sqrt(a^2 - b^2)

k3 =

0.1240

k4 =

0.9923

These constants are used in equations 7.21c and d to find the shear stresses on the surface at the ends of the major and minor axes.

Major axis

TAUxy = (1-2*v)*(k3/k4^2)*((1/k4)*...

atanh(k4)-1)*Pmax      % ksi

TAUxy =

12.1351

Minor axis

TAUxy = (1-2*v)*(k3/k4^2)*(1-(k3/k4)*...

atan(k4/k3))*Pmax      % ksi

TAUxy =

5.5279